| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0124-v1 |
| --- | --- |

| Title: | AHG8: Weighted Prediction for VVC High Bit Depth Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Haoping Yu, Zhihuang Xie, Fan Wang, Dong Wang | Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

VVC high bit-depth extension is expected to support video formats with bit-depth higher than 10 bits. The weighted prediction in the current VVC specification is limited with less precision cannot handle video well if the bit depth is more than 10. This contribution proposes a weighted prediction method. If the bit depth of input video is not more than 10, weighted prediction will follow current VVC. If the bit depth of input video is more than 10, weighted prediction will follow the way defined at HEVC range extension.

# Introduction

In the current VVC specification, the weighted prediction offsets are derived as follows:

For luma component

luma_o0 = luma_offset_l0[ refIdxL0 ]  <<  ( bitDepth − 8 )

luma_o1 = luma_offset_l1[ refIdxL1 ]  <<  ( bitDepth − 8 )

where luma_o0 and luma_o1 are luma offsets used for weighted prediction for l0 and l1, respectively. luma_offset_l0 and luma_offset_l1 are syntax elements sent in the bitstream, and bitDepth is equal to the bit depth of the input video. Similar operations are applied for chroma components.

If the bit depth of input video is more than 8, the offset specified in the bitstream will be left shifted by (bitDepth – 8), which retains the precision of the offsets always at 8 bits. This may be fine for 10-bit video. However, the 8-bit precision for weighted prediction offset may not be sufficient for high bit depth video, e.g., 16-bit video.

In the HEVC range extension [2], the range of the offset is specified to be the same as that of input video. For example, if the input video is 16 bits, the range of offset is in the range of -2^15 to (2^15 – 1).

# Proposed Method

This contribution proposes a weighted prediction method that extends the precision of the weighted prediction offse to be the same as that of input video when the bit-depth of input video is higher than 10, whereas maintains the prediciton offset precision unchanged for 8-bit or 10-bit video.

More specifically, two flags, one in SPS and one in picture header are specified, as shown below, to control the above proposed behavior.

sps_high_precision_offsets_enabled_flag equal to 1 specifies that high precision offset is enabled for the CLVS. high_precision_offsets_enabled_flag equal to 0 specifies that high precision offset is disabled for the CLVS.

ph_high_precision_offsets_enabled_flag equal to 1 specifies that high precision offset is enabled for the current picture. When ph_high_precision_offsets_enabled_flag equal to 1, weighted prediction offset values are using a precision equivalent to eight-bit processing when the input video precision is eight or ten. Otherwise, when the input video precision is above ten bit, weighted prediction offset values are using a bit-depth-dependent precision equal to bit-depth of input video. ph_high_precision_offsets_enabled_flag equal to 0 specifies that high precision offset is disabled for the current picture and weighted prediction offset values are using a precision equivalent to eight-bit processing.  When not present, the value of ph_high_precision_offsets_enabled_flag is inferred to be equal to 0.

The corresponding changes in the VVC [1] are specified as follows in yellow color.

The variables WpOffsetBdShiftY, WpOffsetBdShiftC, WpOffsetHalfRangeY and WpOffsetHalfRangeC are derived as follows:

WpOffsetBdShiftY = high_precision_offsets_enabled_flag ? ( BitDepthY ==10 ? (BitDepthY − 8) : 0 ) : ( BitDepthY − 8 )	(1)

WpOffsetBdShiftC = high_precision_offsets_enabled_flag ? ( BitDepthC ==10 ? (BitDepthC − 8) : 0 ) : ( BitDepthC − 8 )	(2)

WpOffsetHalfRangeY = 1  <<  ( high_precision_offsets_enabled_flag ? (BitDepthY ==10 ? 7 : ( BitDepthY − 1 )) : 7 )	(3)

WpOffsetHalfRangeC = 1  <<  ( high_precision_offsets_enabled_flag ? ( BitDepthC ==10 ? 7 : (BitDepthC − 1 )) : 7 )	(4)

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList[ 0 ][ i ]. The value of luma_offset_l0[ i ] shall be in the range of -WpOffsetHalfRangeY to WpOffsetHalfRangeY, inclusive. When luma_weight_l0_flag[ i ] is equal to 0, luma_offset_l0[ i ] is inferred to be equal to 0.

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList[ 0 ][ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is derived as follows:

ChromaOffsetL0[ i ][ j ] = Clip3(-WpOffsetHalfRangeC, WpOffsetHalfRangeC - 1,

(WpOffsetHalfRangeC + delta_chroma_offset_l0[ i ][ j ] −	
	( ( WpOffsetHalfRangeYC * ChromaWeightL0[ i ][ j ] )  >>  ChromaLog2WeightDenom ) ) )	(5)

The value of delta_chroma_offset_l0[ i ][ j ] shall be in the range of −4 * WpOffsetHalfRangeC to 4 *WpOffsetHalfRangeC – 1, inclusive. When chroma_weight_l0_flag[ i ] is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0.

For luma offsets

o0 = luma_offset_l0[ refIdxL0 ] << WpOffsetBdShiftY		    (6)

o1 = luma_offset_l1[ refIdxL1 ] << WpOffsetBdShiftY    	(7)

For chroma offsets                             			    o0 = ChromaOffsetL0[ refIdxL0 ][ cIdx − 1 ]  << WpOffsetBdShiftC               (8)		      o1 = ChromaOffsetL1[ refIdxL1 ][ cIdx − 1 ]  << WpOffsetBdShiftC          (9)

# Conclusions

It is suggested to adopt the proposed weighted prediction for VVC high bit depth extension.

# References

- B. Broass, J. Chen, S. Liu and Y. K. Wang, “Versatile Video Coding Editorial Refinementas on Draft 10, ” Doc. JVET-T2001, Joint Video Experts Team of VCEG and MPEG, 20th meeting, Oct. 2020.

- “High Efficiency Video Coding” ITU-T, H.265, Dec. 2016.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).