| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0126 |
| --- | --- |

| Title: | EE-related: Modified OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes modified OBMC based on JVET-U0100 to improve coding efficiency. This includes two proposals: 1) Extending OBMC-blending line of chrome component from one pixel to two pixels. 2) Fixing the RDO of OBMC for AMVR. Simulation results reportedly show proposal 1 and proposal 1 achieves -0.04% BD-rate saving without increasing encoding/decoding running time, while proposal 2 acheves -0.03% BD-rate saving with increasing encoding/decoding running time by 1%/4%.

Introduction

In the previous meeting, JVET-U0100[1] proposed OBMC as one of the tools to improve the coding efficiency beyond VVC. In JVET-U0100, the OBMC-blending line of chroma component is one pixel against the block boundaries. Regarding RDO of OBMC for AMVR (IMV), the RD cost check of OBMC-off is almost not checked. In EE common software base [2], this part is described as “to do” even with the latest version. Furthermore, it is also skipped in HASH_INTER and INTER_MULTIHYP.

//EncCu.cpp

if( currTestMode.type == ETM_INTER_ME )

{

#if ENABLE_OBMC

bool tryObmc = true;

#endif

if( ( currTestMode.opts & ETO_IMV ) != 0 )

{

const bool skipAltHpelIF = ( int( ( currTestMode.opts & ETO_IMV ) >> ETO_IMV_SHIFT ) == 4 ) && ( bestIntPelCost > 1.25 * bestCS->cost );

if (!skipAltHpelIF)

{

tempCS->bestCS = bestCS;

#if ENABLE_OBMC

tryObmc = xCheckRDCostInterIMV(tempCS, bestCS, partitioner, currTestMode, bestIntPelCost);

#else

xCheckRDCostInterIMV(tempCS, bestCS, partitioner, currTestMode, bestIntPelCost);

#endif

tempCS->bestCS = nullptr;

}

}

else

{

tempCS->bestCS = bestCS;

xCheckRDCostInter( tempCS, bestCS, partitioner, currTestMode );

tempCS->bestCS = nullptr;

}

#if ENABLE_OBMC

if (tryObmc && tempCS->cus.size() != 0)//todo

{

xCheckRDCostInterWoOBMC(tempCS, bestCS, partitioner, currTestMode);

}

#endif

}

else if (currTestMode.type == ETM_HASH_INTER)

{

xCheckRDCostHashInter( tempCS, bestCS, partitioner, currTestMode );

}

~~

#if MULTI_HYP_PRED

else if (currTestMode.type == ETM_INTER_MULTIHYP)

{

xCheckRDCostInterMultiHyp2Nx2N(tempCS, bestCS, partitioner, currTestMode);

Proposal

This contribution includes two proposals as follows:

- Extending OBMC-blending line of chrome component from one pixel to two pixels.

- Fixing the RDO of OBMC for AMVR. In detail, the RD cost check of OBMC-off is added into AMVR, HASH_INTER, and INTER_MULTIHPY.

Experimental Results

The proposed method is implemented on the top of EE common software base [2] with the version released in Feb. 16, 2021. Simulation conditions basically follows JVET-U2024 [3] but only OBMC and/or 12TAP interpolation filter is enabled while the other tools proposed by JVET-U0100 are disabled to clarify the effect of the proposal compared to the original one. The anchor is proposal 2. Experimental results are summarized as in the following tables. Note that the current results are performed by shorter frames against JVET-U2024. Full-frame results are to be updated.

Table 1 The simulation results of Proposal 1

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE common software base only with OBMC and 12Tap where RDO is fixed |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.19% | 0.23% | 100% | 99% |
| Class A2 | -0.08% | -0.09% | 0.13% | 100% | 99% |
| Class B | -0.01% | -0.18% | 0.07% | 99% | 100% |
| Class C | -0.04% | 0.29% | 0.24% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.04% | 0.16% | 100% | 100% |
| Class D | -0.01% | 0.49% | 0.54% | 100% | 100% |
| Class F (optional) | -0.01% | 0.26% | 0.14% | 99% | 100% |

Table 1 The simulation results of OBMC where RDO is not fixed compared to Proposal 2.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE common software base only with OBMC where RDO is fixed |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.18% | -0.13% | 98% | 93% |
| Class A2 | 0.02% | 0.16% | -0.24% | 98% | 98% |
| Class B | 0.04% | 0.12% | -0.21% | 99% | 95% |
| Class C | 0.04% | 0.04% | 0.07% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.12% | -0.12% | 99% | 96% |
| Class D | 0.11% | -0.02% | 0.13% | 98% | 98% |
| Class F (optional) | 0.30% | 0.07% | 0.11% | 99% | 98% |

Conclusion

In this contribution, it was proposed that modified OBMC based on JVET-U0100 to improve coding efficiency.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

References

- Y.-J. Chang, C.-C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, and Z. Zhang, “Compression efficiency methods beyond VVC”, JVET-U0100, teleconference, 6–15 January 2021.

- https://vcgit.hhi.fraunhofer.de/jvet-u-ee2/VVCSoftware_VTM/-/tree/U0100

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, M. Winken, J. Ström, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability”, JVET-U2024, teleconference, 6–15 January 2021.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).