| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0130-v3 |
| --- | --- |

| Title: | EE2: Intra Template Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser, Tangi Poirier Fabrice Le Léannec Gaelle Martin-Cocher | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution provides details about the EE2 test on intra template matching prediction. It is reported that the proposed intra template matching coding tool reportedly provides -0.40% and -0.1% coding gain in tool-on test, with JVET SDR coding test conditions.

With respect to classes F and TGM, the following tool-on BD rate savings are reported:

- AI:

  - Class F	-2.2%	with enc/dec time 132% / 285%

  - Class TGM: -6.5% with enc/dec time 133% / 487%

- RA:

  - -1.0%	with enc/dec time 117% / 120%

  - -2.9%	with enc/time 117%	/ 138%

For tool-off tests, the obtained results are:

  - AI: +1.3% and +4.4% BD-rate loss respectively in classes F and TGM

  - RA: +0.9% and +2.4% BD-rate loss respectively in classes F and TGM

It is proposed to integrate the proposed template matching based intra prediction mode as a screen content coding tool in the exploration model of JVET for compression capability beyond VVC.

# Proposed Method

Template matching prediction was introduced in JVET-U0048. It is a coding tool that is mostly adapted for screen content coding. The prediction signal is generated at the decoder side by matching the L-shaped causal neighbor of the current block with another block in a predefined search area. This is illustrated in Figure 1. Specifically, the search range is divided into 3 regions:

R1: within the current CTU

R2: top-left outside the current CTU

R3: above the current CTU

R4: left to the current CTU

Within each region, the decoder searches for the template the has least SAD with respect to the current one and uses its corresponding block as a prediction block.

The dimensions of all regions (SearchRange_w, SearchRange_h) are set proportional to the block dimension (BlkW, BlkH) in order to have a fixed number of SAD comparisons per pixel. That is:

SearchRange_w = a * BlkW

SearchRange_h = a * BlkH

Where ‘a’ is a constant that controls the gain/complexity trade-off.

Figure 1

Current block
(PU/CU)

R3

Current CTU

Current template

Similar template

Matching block

R2

R4

R1

: intra template matching search area used.

# Experimental Results

The method is implemented on EE2 software and tested as described in EE common testing condition JVET-U2024. Two types of results are provided:

- Tool-on results:

  - Anchor:  vtm10.0 with GoP32

  - Test: anchor + intra-TMP

- Tool-off test:

  - anchor: U0100 soft + intra-TMP

  - Test: U0100 soft

## Tool on test:

The following Table 1 summarizes the results obtained with proposed template matching intra prediction on AI and RA JVET CTCs, including class TGM. It can be seen the proposed tool is particularly adapted to screen content video coding.

Table 1: tool-on test results of the proposed intra template matching prediction mode, including class TGM results.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.01% | -0.07% | 198% | 203% |
| Class A2 | -0.60% | -0.84% | -0.87% | 182% | 171% |
| Class B | -0.48% | -0.51% | -0.55% | 182% | 183% |
| Class C | -0.15% | -0.16% | -0.13% | 159% | 134% |
| Class E | -0.73% | -0.92% | -0.90% | 174% | 160% |
| Overall | -0.40% | -0.47% | -0.49% | 178% | 168% |
| Class D | 0.03% | -0.02% | 0.07% | 135% | 118% |
| Class F | -2.16% | -2.03% | -2.05% | 132% | 285% |
| Class TGM | -6.47% | -6.13% | -6.16% | 133% | 487% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.19% | 0.17% | 126% | 119% |
| Class A2 | -0.22% | -0.18% | -0.21% | 114% | 103% |
| Class B | -0.17% | -0.28% | -0.02% | 119% | 108% |
| Class C | 0.01% | 0.05% | 0.08% | 113% | 79% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.08% | 0.01% | 118% | 101% |
| Class D | 0.08% | 0.24% | 0.19% | 112% | 109% |
| Class F | -0.97% | -1.07% | -0.91% | 117% | 120% |
| Class TGM | -2.86% | -2.79% | -2.74% | 117% | 138% |

In last version of implementation delivered to the JVET repository, a different trade-off between complexity and coding gain is as follows for classes F and TGM.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | -1.78% | -1.69% | -1.70% | 124% | 231% |
| Class TGM | -5.31% | -5.01% | -5.05% | 125% | 354% |

## Tool off test:

The following Table 2 summarizes the tool-off results of intra-TMP, ported in JVET-U0100 software. Significant coding loss is obtained when deactivating the proposed intra-TMP in classes F and TGM.

Table 2: tool-off test results of proposed template matching based intra prediction.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over U0100+TMP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.07% | 64% | 92% |
| Class A2 | 0.38% | 0.47% | 0.51% | 65% | 85% |
| Class B | 0.43% | 0.43% | 0.37% | 79% | 85% |
| Class C | 0.15% | 0.03% | 0.02% | 84% | 92% |
| Class E | 0.62% | 0.59% | 0.67% | 81% | 87% |
| Overall | 0.33% | 0.31% | 0.32% | 75% | 88% |
| Class D | -0.04% | -0.16% | -0.07% | 87% | 90% |
| Class F | 1.26% | 1.10% | 1.13% | 87% | 72% |
| Class TGM | 4.43% | 4.07% | 4.08% | 88% | 47% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over U0100+TMP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | -0.13% | 90% | 97% |
| Class A2 | 0.17% | -0.04% | 0.10% | 92% | 100% |
| Class B | 0.15% | 0.22% | 0.34% | 95% | 100% |
| Class C | -0.09% | 0.05% | 0.04% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.08% | 0.12% | 94% | 99% |
| Class D | -0.10% | -0.39% | 0.02% | 97% | 100% |
| Class F | 0.88% | 0.90% | 0.87% | 94% | 95% |
| Class TGM | 2.44% | 2.36% | 2.41% | 94% | 90% |

# Various gain/complexity trade-offs

The following Table 3 and Table 4 provide various trade-offs that have been obtained in AI configuration on classes F and TGM, respectively for tool-on and tool-off tests.

For tool-on tests, the trade-offs have been obtained for different values of the multiplicative factor ‘a’ introduced in section 1, and of the maximum CU size for which Intra TMP is enabled. It can be observed the proposed tool mainly impacts the decoding time, while the encoding time increase remains reasonable.

The results corresponding to the EE2 tool-on tests are highlighted in Table 3 and correspond to a multiplicative factor a equal to 5 and a maximum CU size 64 for intra TMP. Moreover, in Table 3, the variant called ’64 variant’ was obtained with a more recent version of the delivered source code than the test 64.

Table 3: various tool-on trade-offs between coding gain and runtime obtained with intra TMP (classes F+TGM, all-intra)

| Mult Factor |  |  | Max CU Size for Intra TMP |  | Enc. Time |  | Dec. Time |  | BD-rate Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  | 4 |  | 99.80% |  | 100.58% |  | -0.12% |  |
|  |  |  | 8 |  | 108.00% |  | 122.36% |  | -0.42% |  |
|  |  |  | 16 |  | 101.10% |  | 105.57% |  | -0.12% |  |
|  |  |  | 32 |  | 114.40% |  | 128.21% |  | -1.14% |  |
| 5 |  |  | 4 |  | 103.20% |  | 106.41% |  | -0.24% |  |
|  |  |  | 8 |  | 106.12% |  | 124.56% |  | -1.16% |  |
|  |  |  | 16 |  | 118.42% |  | 158.98% |  | -2.22% |  |
|  |  |  | 32 |  | 121.78% |  | 217.46% |  | -3.23% |  |
|  |  |  | 64 default |  | 124.34% |  | 285.91% |  | -3.54% |  |
|  |  |  | 64 |  | 132.61% |  | 372.60% |  | -4.31% |  |
| 8 |  |  | 4 |  | 102.21% |  | 111.41% |  | -0.45% |  |
|  |  |  | 8 |  | 109.17% |  | 149.52% |  | -1.81% |  |
|  |  |  | 16 |  | 127.38% |  | 244.92% |  | -3.82% |  |
|  |  |  | 32 |  | 142.40% |  | 420.10% |  | -5.39% |  |

Table 4: various tool-off trade-offs between coding efficiency and runtime with intra TMP (classes F+TGM, all-intra)

| Mult. Factor | Max CU Size for Intra TMP | Enc. Time | Dec. Time | BD-rate Y |
| --- | --- | --- | --- | --- |
| 2 | 64 | 92.19% | 88.55% | 0.99% |
| 5 | 64 | 87.58% | 58.47% | 2.84% |
| 8 | 64 | 81.72% | 37.86% | 4.02% |

Finally, the diagram of Figure 2 shows the various luma-gain/decoding time trade-offs, so that one easily sees the most interesting ones.

Figure 2: tool-on trade-offs between decoding time and gain for various multiplicative factor values.

# Conclusion

The proposed template matching based intra prediction tool has been ported into U0100 platform. Significant coding gains are reported in screen content coding, namely classes F and TGM. It is suggested to integrate intra template matching prediction into the exploration model of the JVET for compression capability beyond VVC.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).