| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0131-r1 |
| --- | --- |

| Title: | AHG8: Fixing the forward transform matrices for high bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) K. Naser, Franck Galpin, Fabrice Le Leannec Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In High Bit-depth coding of VVC, the forward transform matrices are represented with higher precision than the standard coding. This modification is reported to alter the properties of DST-VII and DCT-VIII 4x4 transforms that leads to difference between matrix multiplication- and fast algorithm-based computation. This contribution proposes modifying these transform matrices such that both matrix multiplication and fast algorithm are identical and therefore restore the dual-implementation property.

The following experimental results of bdrate gain are reported:

Low QP

- 0.00 %, 0.00 % and -0.01 % in bdrateY in PQ, HLG and SVT in AI

- -0.01 %, -0.01 % and 0.00 % in bdrateY in PQ, HLG and SVT in LDB

- X.XX %, X.XX % and X.XX % in bdrateY in PQ, HLG and SVT in RA

Normal QP

- -0.07 % and 0.00 % in bdrateY in H1 and H2 in AI

- - 0.04 % and X.XX % in bdrateY in H1 and H2 in RA

# Introduction / Problem Statement / …

Similar to HEVC range extension, VVC forward transform matrices for High Bit-Depth (HBT) coding are extended 14 fractional bits instead of 6 in standard coding. The new matrices are computed as the inverse of the 6 fractional bits inverse transform. The DST-VII matrix of 4x4 is as follows:

DST7_4x4 =

a,  b,  c,  d

c,  c,  0, -c

d, -a, -c,  b

b, -d,  c, -a

In 6 fractional bit representation, the values of the constants are (a=29, b=55, c=74  d=84). Their intrinsic property is: a+b = d. It can be used to derive fast implementation of first, third and last coefficients as follows.

For an input signal (In[i]) to be transformed to Out[i], where i ranges from 0 to 3, the first coefficient of DST7 transform is computed as:

Out[0] = a*In[0] + b*In[1] + c*In[2] + d*In[3] // 4 multiplications

Out[0] = a*In[0] + b*In[1] + c*In[2] + (a+b)*In[3] // since d=a+b

Out[0] = a*(In[0]+In[3]) + b*(In[1]+In[3]) + c*In[2] // 3 multiplications

The third coefficient of DST7 transform is computed as:

Out[2] = d*In[0] - a*In[1] - c*In[2] + b*In[3] // 4 multiplications

Out[2] = (a+b)*In[0] - a*In[1] - c*In[2] + b*In[3] // since d=a+b

Out[2] = a*(In[0]-In[1])  +b*(In[0] + In[3]) - c*In[2] // 3 multiplications

The fourth coefficient of DST7 transform is computed as follows:

Out[3] = b*In[0] - d*In[1] + c*In[2] - a*In[3] // 4 multiplications

Out[3] = b*In[0] – (a+b)*In[1] + c*In[2] - a*In[3] // since d=a+b

Out[3] = b*(In[0]-In[1]) – a*(In[1] + In[3] ) + c*I[2]// 3 multiplications

For DCT8, similar algorithm is used for fast implementation.

However, for HBD coding, the constants are modified as: a = 7425, b = 14081, c = 18893, d = 21505. Clearly, the property of a+b=d does not hold any more and thus the fast algorithm cannot be used. It should be noted that the VTM software uses the fast algorithm by default, which is shown to be inconsistent with matrix multiplication and therefore breaks the desired property of dual implementation support.

# Proposed Method

It is proposed to modify the transform matrices of DST7 and DCT8 in HBD coding such that the property of a+b=d holds. Specifically, the following values of the constants are used:

a = 74254, b = 14081, c = 18893, d = 21505

# Experimental Results

The proposed modifications were implemented on top of VTM-12.0 following the common testing conditions defined in JVET-U2018. The simulation results are provided below:

## Low QP:

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | 100% | 98% |
| PQ422 | 0.00% | 0.01% | 0.00% | 0.00% | 0.01% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.01% | 0.00% | 0.00% | 0.01% | 0.00% | 100% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | 97% | 97% |
| PQ422 | -0.03% | -0.01% | -0.01% | -0.03% | -0.02% | -0.01% | 99% | 96% |
| Overall | -0.01% | 0.00% | 0.00% | -0.01% | -0.01% | 0.00% | 98% | 96% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.02% | 0.00% | 0.00% | 0.02% | 103% | 109% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall PQ | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.01% | 0.01% | 99% | 99% |
| HLG422 | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | -0.01% | -0.03% | 99% | 102% |
| HLG422 | 0.00% | -0.01% | -0.05% | 100% | 101% |
| Overall | -0.01% | -0.01% | -0.04% | 100% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall HLG | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.03% | -0.02% | -0.03% | 100% | 102% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.01% | 0.00% | 0.00% | 100% | 103% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | 0.00% | 0.00% | 0.00% | 101% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall RGB | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## normal QP:

|  | Random Access |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.20% | -0.04% | -0.03% | -0.12% | -0.14% | -0.04% | -0.07% | -0.13% | 101% | 102% |
| Class H2 |  |  |  |  |  | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | -0.20% | -0.04% | -0.03% | -0.12% | -0.14% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.07% | -0.01% | 0.00% | 0.02% | -0.10% | 0.00% | -0.01% | -0.10% | 98% | 97% |
| Class H2 |  |  |  |  |  | 0.00% | 0.05% | 0.00% | 100% | 100% |
| Overall | -0.07% | -0.01% | 0.00% | 0.02% | -0.10% | 0.00% | 0.02% | -0.05% | 99% | 98% |

# Conclusion

This contribution proposes a fix to the transform matrices of 4x4 DST7 and DCT8 in HBD coding. The modification is encoder-only and does not affect the bitrate compression efficiency. It is proposed to include the change into the next version of VTM software.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).