| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0136 |
| --- | --- |

| Title: | Crosscheck of JVET-V0047 (CE-3.1 and CE-3.2: Transform coefficients range extension for high bit-depth coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for CE-3.1 and CE-3.2, two proposals that change transform coefficient clipping for high bit depths. The simulation results matched those supplied by the proponents.

# Introduction

VVC version 1 sets MaxLog2TrDynamicRange, the number of bits of accuracy for transform processing to 15 bits for bit depths of 10 and above. VTM supports extended precision processing which extends this to 18 bits for 12-bit content and 22 bits for 16-bit content.

CE-3.1 and CE-3.2 [1] modify these as follows, and in addition change the invocation parameters of k-th order Exp-Golomb binarization.

| Configuration | Transform bits |  |
| --- | --- | --- |
|  | 12-bit content | 16-bit content |
| VVC V1 | 15 | 15 |
| VTM (extended precision) | 18 | 22 |
| CE anchor | 15 | 22 |
| CE-3.1 | 18 | 20 |
| CE-3.2 | 17 | 20 |

# Analysis

The code was found to be in agreement with the description in the CE[2] and the draft specification text changes in the proposal.

# Results

The code [3] was compiled using gcc 8.2.1 and tested according to the high bit depth / high bit rate CTC[4] on Linux clusters running openSUSE Leap 15.2. The results were compared with a similarly prepared CE anchor consisting of VTM12.0 augmented with rice coding fixes (CE-1.1 and CE-2.1).

Results for CE-3.1 and CE-3.2 were found to be in agreement with the proponent’s across the low QP and standard QP ranges. Additional results are provided for lossless coding and compared with an anchor of the CE software with extended precision enabled.

## CE-3.1

### Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.33% | -1.94% | -2.09% | -1.33% | -1.97% | -2.11% | 101% | 94% |
| PQ422 | -1.13% | -1.44% | -1.56% | -1.12% | -1.44% | -1.55% | 101% | 95% |
| Overall | -1.23% | -1.69% | -1.82% | -1.23% | -1.70% | -1.83% | 101% | 94% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.70% | -0.74% | -0.81% | -0.71% | -0.74% | -0.81% | 100% | 99% |
| PQ422 | -0.64% | -0.57% | -0.64% | -0.64% | -0.57% | -0.63% | 101% | 98% |
| Overall | -0.67% | -0.65% | -0.72% | -0.68% | -0.66% | -0.72% | 100% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.62% | -0.72% | -0.77% | -0.62% | -0.71% | -0.77% | 101% | 98% |
| PQ422 | -0.59% | -0.54% | -0.62% | -0.58% | -0.54% | -0.61% | 100% | 98% |
| Overall | -0.60% | -0.63% | -0.69% | -0.60% | -0.62% | -0.69% | 100% | 98% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.51% | -0.88% | -0.91% | 109% | 93% |
| HLG422 | -0.43% | -0.78% | -0.80% | 103% | 95% |
| Overall | -0.47% | -0.83% | -0.86% | 106% | 94% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.67% | -0.93% | -0.97% | 102% | 95% |
| HLG422 | -0.57% | -0.67% | -0.72% | 101% | 95% |
| Overall | -0.62% | -0.80% | -0.84% | 101% | 95% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.55% | -0.83% | -0.87% | 101% | 97% |
| HLG422 | -0.45% | -0.63% | -0.69% | 100% | 97% |
| Overall | -0.50% | -0.73% | -0.78% | 100% | 97% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.69% | -0.45% | -0.45% | 108% | 100% |
| SVT12 | -0.80% | -1.25% | -1.27% | 92% | 86% |
| Overall | -0.75% | -0.85% | -0.86% | 100% | 93% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.20% | -0.23% | -0.22% | 101% | 97% |
| SVT12 | -0.79% | -0.65% | -0.66% | 93% | 94% |
| Overall | -0.49% | -0.44% | -0.44% | 97% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.05% | -0.32% | -0.30% | 99% | 98% |
| SVT12 | -0.61% | -0.64% | -0.65% | 94% | 95% |
| Overall | -0.33% | -0.48% | -0.48% | 96% | 96% |

### Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.14% | -0.01% | 0.00% | -0.25% | 0.01% | 0.01% | -0.29% | 0.08% | 99% | 99% |
| Class H2 |  |  |  |  |  | -0.01% | 0.07% | 0.13% | 100% | 100% |
| Overall | 0.14% | -0.01% | 0.00% | -0.25% | 0.01% | 0.00% | -0.11% | 0.11% | 100% | 99% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.02% | 0.02% | 0.00% | 0.03% | -0.02% | 0.00% | 0.01% | -0.01% | 99% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | -0.09% | -0.08% | 100% | 100% |
| Overall | 0.02% | 0.02% | 0.00% | 0.03% | -0.02% | 0.00% | -0.04% | -0.05% | 100% | 99% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-3.1 |  | VTM_U_CE | CE-3.1 |  | VTM_U_CE | CE-3.1 |  |
| PQ444 | 2.5 | 2.5 | 0.00% | 3.0 | 3.0 | 0.00% | 3.0 | 3.0 | 0.00% |
| PQ422 | 2.3 | 2.3 | 0.00% | 2.8 | 2.8 | 0.00% | 2.8 | 2.8 | 0.00% |
| Overall | 2.4 | 2.4 | 0.00% | 2.9 | 2.9 | 0.00% | 2.9 | 2.9 | 0.00% |
| Enc Time[%] | 100% |  |  | 103% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  | 100% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-3.1 |  | VTM_U_CE | CE-3.1 |  | VTM_U_CE | CE-3.1 |  |
| HLG444 | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| HLG422 | 1.6 | 1.6 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| Overall | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| Enc Time[%] | 103% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 101% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-3.1 |  | VTM_U_CE | CE-3.1 |  | VTM_U_CE | CE-3.1 |  |
| SVT16 | 1.2 | 1.2 | -0.08% | 1.2 | 1.2 | -0.08% | 1.2 | 1.2 | -0.08% |
| SVT12 | 1.2 | 1.2 | 0.00% | 1.3 | 1.3 | 0.00% | 1.3 | 1.3 | 0.00% |
| Overall | 1.2 | 1.2 | -0.04% | 1.2 | 1.2 | -0.04% | 1.2 | 1.2 | -0.04% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  | 100% |  |  |

## CE-3.2

### Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.21% | -1.77% | -1.91% | -1.21% | -1.80% | -1.92% | 100% | 94% |
| PQ422 | -1.03% | -1.33% | -1.43% | -1.03% | -1.33% | -1.43% | 99% | 94% |
| Overall | -1.12% | -1.55% | -1.67% | -1.12% | -1.57% | -1.67% | 100% | 94% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.68% | -0.71% | -0.77% | -0.69% | -0.70% | -0.78% | 101% | 99% |
| PQ422 | -0.61% | -0.52% | -0.60% | -0.60% | -0.52% | -0.59% | 98% | 96% |
| Overall | -0.64% | -0.61% | -0.68% | -0.64% | -0.61% | -0.68% | 100% | 97% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.59% | -0.67% | -0.72% | -0.59% | -0.66% | -0.72% | 102% | 100% |
| PQ422 | -0.55% | -0.51% | -0.59% | -0.54% | -0.51% | -0.59% | 101% | 99% |
| Overall | -0.57% | -0.59% | -0.66% | -0.56% | -0.58% | -0.65% | 101% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.45% | -0.81% | -0.83% | 110% | 94% |
| HLG422 | -0.40% | -0.72% | -0.75% | 102% | 94% |
| Overall | -0.43% | -0.76% | -0.79% | 106% | 94% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.62% | -0.85% | -0.90% | 103% | 96% |
| HLG422 | -0.54% | -0.63% | -0.66% | 101% | 96% |
| Overall | -0.58% | -0.74% | -0.78% | 102% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.51% | -0.76% | -0.82% | 101% | 97% |
| HLG422 | -0.42% | -0.57% | -0.65% | 100% | 97% |
| Overall | -0.47% | -0.67% | -0.73% | 100% | 97% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.69% | -0.45% | -0.45% | 106% | 98% |
| SVT12 | -0.67% | -1.04% | -1.06% | 96% | 92% |
| Overall | -0.68% | -0.74% | -0.76% | 101% | 95% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.20% | -0.23% | -0.22% | 104% | 101% |
| SVT12 | -0.69% | -0.57% | -0.58% | 100% | 95% |
| Overall | -0.44% | -0.40% | -0.40% | 102% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.05% | -0.32% | -0.30% | 99% | 100% |
| SVT12 | -0.53% | -0.57% | -0.58% | 98% | 96% |
| Overall | -0.29% | -0.44% | -0.44% | 99% | 98% |

### Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.08% | -0.01% | 0.01% | -0.41% | -0.02% | 0.02% | -0.47% | -0.22% | 99% | 99% |
| Class H2 |  |  |  |  |  | 0.01% | -0.12% | 0.20% | 100% | 100% |
| Overall | 0.08% | -0.01% | 0.01% | -0.41% | -0.02% | 0.01% | -0.30% | -0.01% | 99% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.08% | -0.01% | -0.01% | 0.14% | -0.02% | -0.01% | 0.15% | -0.07% | 99% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | -0.03% | 0.04% | 99% | 98% |
| Overall | -0.08% | -0.01% | -0.01% | 0.14% | -0.02% | -0.01% | 0.06% | -0.02% | 99% | 98% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-3.2 |  | VTM_U_CE | CE-3.2 |  | VTM_U_CE | CE-3.2 |  |
| PQ444 | 2.5 | 2.5 | 0.00% | 3.0 | 3.0 | 0.00% | 3.0 | 3.0 | 0.00% |
| PQ422 | 2.3 | 2.3 | 0.00% | 2.8 | 2.8 | 0.00% | 2.8 | 2.8 | 0.00% |
| Overall | 2.4 | 2.4 | 0.00% | 2.9 | 2.9 | 0.00% | 2.9 | 2.9 | 0.00% |
| Enc Time[%] | 101% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  | 100% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-3.2 |  | VTM_U_CE | CE-3.2 |  | VTM_U_CE | CE-3.2 |  |
| HLG444 | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| HLG422 | 1.6 | 1.6 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| Overall | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | 0.00% | 1.9 | 1.9 | 0.00% |
| Enc Time[%] | 103% |  |  | 101% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 101% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-3.2 |  | VTM_U_CE | CE-3.2 |  | VTM_U_CE | CE-3.2 |  |
| SVT16 | 1.2 | 1.2 | -0.08% | 1.2 | 1.2 | -0.08% | 1.2 | 1.2 | -0.08% |
| SVT12 | 1.2 | 1.2 | 0.00% | 1.3 | 1.3 | 0.00% | 1.3 | 1.3 | 0.00% |
| Overall | 1.2 | 1.2 | -0.04% | 1.2 | 1.2 | -0.04% | 1.2 | 1.2 | -0.04% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 101% |  |  |

## Results versus extended precision anchor

The following tables show the BD-rate changes when CE-3.1 and CE-3.2 are compared with a CE anchor with extended precision enabled for the low QP range of the CTC. As expected for CE-3.1 the results are identical for 12-bit processing, but gains are evident for 16-bit processing. For CE-3.2, losses occur for 12-bit processing when compared to the extended precision anchor, with the same gains for 16-bit processing.

### CE-3.1

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE (EP) |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE (EP) |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE (EP) |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE (EP) |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| HLG422 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE (EP) |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| HLG422 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE (EP) |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| HLG422 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE (EP) |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.69% | -0.45% | -0.45% | #DIV/0! | #DIV/0! |
| SVT12 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | -0.35% | -0.22% | -0.23% | #DIV/0! | #DIV/0! |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE (EP) |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.20% | -0.23% | -0.22% | #DIV/0! | #DIV/0! |
| SVT12 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | -0.10% | -0.12% | -0.11% | #DIV/0! | #DIV/0! |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE (EP) |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.05% | -0.32% | -0.30% | #DIV/0! | #DIV/0! |
| SVT12 | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | -0.02% | -0.16% | -0.15% | #DIV/0! | #DIV/0! |

### CE-3.2

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.12% | 0.18% | 0.20% | 0.12% | 0.18% | 0.20% | 101% | 102% |
| PQ422 | 0.10% | 0.12% | 0.14% | 0.10% | 0.12% | 0.13% | 99% | 100% |
| Overall | 0.11% | 0.15% | 0.17% | 0.11% | 0.15% | 0.17% | 100% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 101% | 103% |
| PQ422 | 0.03% | 0.05% | 0.04% | 0.04% | 0.05% | 0.04% | 99% | 100% |
| Overall | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.04% | 100% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.05% | 0.05% | 0.03% | 0.05% | 0.04% | 100% | 101% |
| PQ422 | 0.04% | 0.03% | 0.02% | 0.04% | 0.03% | 0.02% | 101% | 101% |
| Overall | 0.03% | 0.04% | 0.04% | 0.03% | 0.04% | 0.03% | 101% | 101% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.06% | 0.08% | 0.08% | 98% | 100% |
| HLG422 | 0.03% | 0.06% | 0.06% | 99% | 101% |
| Overall | 0.05% | 0.07% | 0.07% | 99% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | 0.08% | 0.07% | 100% | 101% |
| HLG422 | 0.03% | 0.04% | 0.06% | 100% | 102% |
| Overall | 0.04% | 0.06% | 0.07% | 100% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | 0.06% | 0.06% | 100% | 101% |
| HLG422 | 0.03% | 0.05% | 0.05% | 99% | 101% |
| Overall | 0.04% | 0.06% | 0.05% | 100% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.69% | -0.45% | -0.45% | 105% | 98% |
| SVT12 | 0.14% | 0.21% | 0.22% | 101% | 104% |
| Overall | -0.28% | -0.12% | -0.12% | 103% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.20% | -0.23% | -0.22% | 103% | 102% |
| SVT12 | 0.11% | 0.08% | 0.09% | 106% | 102% |
| Overall | -0.05% | -0.07% | -0.07% | 104% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE (extended precision) |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.05% | -0.32% | -0.30% | 99% | 99% |
| SVT12 | 0.08% | 0.07% | 0.07% | 104% | 102% |
| Overall | 0.02% | -0.12% | -0.12% | 101% | 100% |

# References

[1] “CE-3.1 and CE-3.2: Transform coefficients range extension for high bit-depth coding”, JVET-V0047, T. Zhou, T. Chujoh, T. Ikai (Sharp)

[2] “CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-V0022, A.Browne, T. Hashimoto, H. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou

[3] https://vcgit.hhi.fraunhofer.de/jvet-u-ce/VVCSoftware_VTM/-/tree/CE4.5

[4] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu