| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0148 |
| --- | --- |

| Title: | Crosscheck of JVET-V0096 (EE1-2.3: Neural Network-based Super Resolution) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Chujoh | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports a cross-check result of JVET-V0096 (EE1-2.3: Neural Network-based Super Resolution). The BD-rate gain and EncT, Dec T is reported as follows:

In RA of classA1,  12.47 %, -6.24 % and 2.69 % in Y, U, V with EncT 166 % and DecT 30 %

In RA of classA2,  3.71 %, -46.82 % and -32.47 % in Y, U, V with EncT 154 % and DecT 30%

In AI of classA1,  11.96 %, -51.96 % and -0.85 % in Y, U, V with EncT 348 % and DecT 35 %

In AI of classA2,  9.64 %, -25.93 % and -18.38 % in Y, U, V with EncT 204 % and DecT 39%

Other than classA1, A2, the coding loss was observed.

The BD rate result match with those of the proponent and DecT is also similar as 0-1% difference while the tester’s EncT is larger than the one of the proponent by 33-97%.

Introduction

The super resolution techniques proposed in JVET-U0099/JVET-V0096 unitizes 2X NN-based resolution increase in decoder-side while the resolution reduction of 1/2X is made as Reference Picture resampling (RPR) of VVC.

The super resolution applys as follow

- YUV 444 operation

  - chroma up-sampling is made before SR and chroma down-sampling is made after SR

- Floating operation

  - for 10-bit video, “pixel value / 1023” is made before SR and * 1023 after SR

- QP based NN model selection

  - QP < 30:  model1

  - Q>= 30:  model2

- SR function is called via PyTorch C++ interface

Results

Experiments with QP=27, 32, 37, 42, 47 were conducted.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -12.47% | 6.24% | -2.69% | #VALUE! | #VALUE! | #VALUE! | 166% | 30% | 16% |
| Class A2 | -3.71% | 46.82% | 32.47% | #VALUE! | #VALUE! | #VALUE! | 154% | 30% | 16% |
| Class B | 34.22% | 15.84% | 15.06% | #VALUE! | #VALUE! | #VALUE! | 194% | 31% | 16% |
| Class C | 73.51% | 34.55% | 31.18% | #VALUE! | #VALUE! | #VALUE! | 211% | 33% | 27% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 27.77% | 25.11% | 19.29% | #VALUE! | #VALUE! | #VALUE! | 184% | 31% | 19% |
| Class D | 87.82% | 26.65% | 28.27% | #VALUE! | #VALUE! | #VALUE! | 391% | 36% | 30% |
| Class F | 93.67% | 95.79% | 121.81% | #VALUE! | #VALUE! | #VALUE! | 258% | 30% | 25% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -11.96% | 51.96% | 0.85% | #VALUE! | #VALUE! | #VALUE! | 348% | 35% | 12% |
| Class A2 | -9.64% | 25.93% | 18.38% | #VALUE! | #VALUE! | #VALUE! | 204% | 39% | 12% |
| Class B | 10.06% | 0.79% | 3.21% | #VALUE! | #VALUE! | #VALUE! | 196% | 37% | 22% |
| Class C | 25.47% | 5.68% | 5.21% | #VALUE! | #VALUE! | #VALUE! | 160% | 39% | 35% |
| Class E | 14.22% | -9.82% | -16.10% | #VALUE! | #VALUE! | #VALUE! | 150% | 37% | 24% |
| Overall | 7.22% | 12.83% | 2.57% | #VALUE! | #VALUE! | #VALUE! | 198% | 37% | 22% |
| Class D | 25.93% | 1.13% | 2.54% | #VALUE! | #VALUE! | #VALUE! | 145% | 39% | 37% |
| Class F | 56.22% | 44.22% | 55.89% | #VALUE! | #VALUE! | #VALUE! | 100% | 37% | 30% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Reference

[1] A. M. Kotra, K. Reuzé, J. Chen, H. Wang, M. Karczewicz, J. Li, “EE1-2.3: Neural Network-based Super Resolution, “, JVET-V0096, JVET 22nd Meeting, by teleconference, 20–28 Apr. 2021

[2] A. M. Kotra, K. Reuzé, J. Chen, H. Wang, M. Karczewicz, J. Li, “AHG11: Neural Network-based Super Resolution,” JVET-U0099

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).