| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0153 |
| --- | --- |

| Title: | AHG12: Cross-component Sample Adaptive Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C.-W. Kuo, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, W. Chen, Xianglin Wang | Email: | {cheweikuo, xiaoyuxiu, yiwenchen, jhuhong-jheng, chenwei06, xianglinwang} @kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In this contribution, one coding tool named Cross-component Sample Adaptive Offset (CCSAO) is proposed for compression efficiency improvement beyond VVC. CCSAO operates as part of SAO process and utilizes the correlation between three components as guidance to further enhance the reconstructive quality of the current sample. Compared to VTM-12.0 anchors with the encoder fix in JVET-V0095, the BD-rate impact of the proposed method is summarized as below.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| CCSAO | 0.13% | -2.43% | -2.39% | 0.04% | -3.62% | -3.61% | -0.15% | -8.45% | -8.64% |

Introduction

In the VVC standard [1], Sample Adaptive Offset (SAO) consists of edge offset (EO) and band offset (BO). EO can effectively remove ringing artifacts near sharp transitions in reconstructed video signal while BO aims at alleviating the DC distortion that is caused by the coding process. In the BO, the reconstructed samples are firstly classified into multiple bands, and one offset is derived for each band which is added to the corresponding reconstructed samples that fall into the same band. Meanwhile, in the existing SAO design, the BO classifier only uses the reconstruction of one single luma/chroma sample as input to enhance itself, without taking the strong correlation between different components into consideration.

Proposal

In this contribution, one coding tool named Cross-component Sample Adaptive Offset (CCSAO) is proposed for further compression efficiency exploration beyond VVC. Specifically, similar to SAO, the CCSAO classifies the reconstructed samples into different categories, properly derives one offset for each category and adds the offset to the reconstructed samples in that category. However, different from SAO which only uses one single luma/chroma component of current sample as input, the CCSAO utilizes all three components to classify the current sample into different categories. To facilitate the parallel processing, the output samples from the de-blocking filter are used as the input of the CCSAO. Figure 1 shows the diagram of the decoding workflow when the CCSAO is applied.

Figure 1 Modified SAO process when the proposed CCSAO is applied.

In the current CCSAO design, to achieve one better complexity/performance trade-off, only BO is used to enhance the quality of the reconstructed samples. For a given luma/chroma sample, three candidate samples are selected to classify the given sample into different categories: one collocated Y sample, one collocated U sample, and one collocated V sample. The sample values of these three selected samples are then classified into three different {bandY, bandU, bandV} bands, and a joint index i is used to indicate the category of the given sample. One offset is signaled and added to the reconstructed samples that fall into that category, which can be formulated as

bandY=Ycol∙NY≫BDbandU=Ucol∙NU≫BDbandV=Vcol∙NV≫BDi=bandY∙NU+NV+bandU∙NV+bandVC'rec=Clip1Crec+σCCSAOi                       (1)

In equation (1), {Ycol, Ucol, Vcol} are the three selected collocated samples that are used to classify the current sample; {NY, NU, NV} are the numbers of equally divided bands applied to {Ycol, Ucol, Vcol} full range respectively; BD is the internal coding bit-depth; Crec and Crec' are the reconstructed samples before and after the CCSAO is applied; σCCSAOi is the value of CCSAO offset that is applied to the i-th BO category. In the current design, the collocated luma sample can be chosen from 9 candidate positions, while the collocated chroma sample positions are fixed, as depicted in Figure 2.

Similar to SAO, different classifiers can be applied to different local region to further enhance the whole picture quality. The parameters for each classifier (i.e., the position of Ycol, NY, NU, NV, and offsets) are signaled in frame level, and which classifier to be used is explicitly signaled and switched in CTB level. For each classifier, the maximum of {NY, NU, NV} is set to {16, 4, 4}, and offsets are constrained to be within the range [-15, 15]. The maximum classifiers per frame is constrained to be 4.

Figure 2 Illustration of the candidate positions used for the CCSAO classifier.

Simulation results

The proposed technique was implemented based on VTM-12.0 reference software [2] and tested by following the common test condition (CTC) as specified in [3]. Besides, the encoder fix in [4] is incorporated and is enabled in both anchor and test. Note that SIMD optimization is not applied in the current implementation.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -1.93% | -2.63% | 113% | 101% |
| Class A2 | 0.14% | -3.60% | -0.61% | 118% | 101% |
| Class B | 0.15% | -3.12% | -3.90% | 106% | 101% |
| Class C | 0.12% | -1.59% | -1.45% | 102% | 102% |
| Class E | 0.13% | -1.77% | -2.67% | 103% | 102% |
| Overall | 0.13% | -2.43% | -2.39% | 108% | 102% |
| Class D | 0.06% | -1.35% | -0.82% | 100% | 102% |
| Class F | -0.21% | -2.67% | -2.14% | 104% | 102% |
| Class TGM | -0.94% | -1.55% | -1.57% | 110% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -2.96% | -5.04% | 106% | 103% |
| Class A2 | 0.14% | -3.45% | -1.57% | 107% | 102% |
| Class B | 0.13% | -5.18% | -5.41% | 106% | 103% |
| Class C | 0.12% | -2.30% | -1.81% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -3.62% | -3.61% | 105% | 102% |
| Class D | 0.19% | -2.38% | -1.58% | 102% | 103% |
| Class F | -0.31% | -3.20% | -2.87% | 107% | 102% |
| Class TGM | -1.14% | -3.15% | -3.51% | 116% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -13.09% | -12.86% | 107% | 103% |
| Class C | 0.06% | -5.93% | -4.78% | 102% | 101% |
| Class E | -0.16% | -4.06% | -6.75% | 103% | 102% |
| Overall | -0.15% | -8.45% | -8.64% | 104% | 102% |
| Class D | 0.21% | -5.97% | -5.12% | 101% | 102% |
| Class F | -0.87% | -8.15% | -7.15% | 110% | 103% |
| Class TGM | -2.14% | -7.42% | -7.38% | 121% | 104% |

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, Jun. 2020, Teleconference.

- VTM-12.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-12.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-U2017, Jan. 2021, Teleconference.

- N. Hu, V. Seregin, M. Karczewicz, “ AHG10: Using original samples for SAO and ALF optimization”, JVET-V0095, Apr. 2021, Teleconference.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).