| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0159 |
| --- | --- |

| Title: | Crosscheck of JVET-V0067 (AHG8: A constraint of max transform size for high bit depth and high bit rate coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Email: | tomonori.hashimoto@sharp.cojp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results for JVET-V0067 (AHG8: A constraint of max transform size for high bit depth and high bit rate). This contribution restricts max transform size for high bit depth to reduce memory size in hardware architecture. The crosscheck results match the coding performance reported by the proponent.

Results

VTM-12.0 with additional encoder option was used for the crosscheck. Additional encoder option information was provided by the proponent. In this test, additional encoder option is as followings:

Log2MaxTbSize : 5

Crosscheck results for HBD CTC are shown in following tables.

Low-QP

HDR PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.03% | 0.03% | 0.01% | 0.03% | 0.03% | 102% | 100% |
| PQ422 | 0.01% | 0.08% | 0.04% | 0.01% | 0.08% | 0.04% | 102% | 100% |
| Overall | 0.01% | 0.06% | 0.03% | 0.01% | 0.05% | 0.04% | 102% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 100% |
| PQ422 | 0.01% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.01% | -0.01% | 0.00% | 0.01% | -0.01% | 101% | 100% |
| PQ422 | 0.01% | 0.01% | 0.01% | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Overall | 0.01% | 0.01% | 0.00% | 0.00% | 0.01% | 0.00% | 101% | 100% |
| Overall PQ | 0.01% | 0.02% | 0.01% | 0.00% | 0.02% | 0.01% | 101% | 100% |

HDR HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| HLG422 | 0.01% | 0.00% | 0.00% | 102% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | 0.00% | 102% | 100% |
| HLG422 | 0.00% | 0.02% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.01% | 0.00% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.01% | 0.01% | 102% | 100% |
| HLG422 | 0.00% | 0.00% | -0.02% | 101% | 100% |
| Overall | 0.00% | 0.01% | -0.01% | 102% | 100% |
| Overall HLG | 0.00% | 0.01% | 0.00% | 102% | 100% |

SVT RGB

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 101% | 100% |

Normal-QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 8.16% | 0.63% | 0.26% | 5.87% | 5.90% | 0.22% | 5.39% | 5.40% | 96% | 101% |
| Class H2 |  |  |  |  |  | 0.47% | 3.51% | 3.89% | 97% | 102% |
| Overall | 8.16% | 0.63% | 0.26% | 5.87% | 5.90% | 0.35% | 4.45% | 4.64% | 96% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 9.68% | 0.58% | 0.13% | 9.22% | 9.44% | 0.11% | 8.47% | 8.44% | 92% | 99% |
| Class H2 |  |  |  |  |  | 0.15% | 3.69% | 4.42% | 94% | 99% |
| Overall | 9.68% | 0.58% | 0.13% | 9.22% | 9.44% | 0.13% | 6.08% | 6.43% | 93% | 99% |

Conclusion

In this contribution, the crosschecker confirmed that the results reported by proponent have matched the results obtained by crosschecker.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).