| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0173 |
| --- | --- |

| Title: | EE1-related: Report on results of remote viewing session |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Lehrstuhl für Bildverarbeitung RWTH Aachen University | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

Remote expert viewing tests were conducted during the 22nd JVET meeting for the exploration activity on DNN-based coding tools on UHD test sequences as defined in EE1. Calls for participation in the subjective viewing were issued on the JVET reflector and verbally during JVET sessions. Overall, 19 experts volunteered in participating in the tests. The testing procedure follows an online variant of the method used for visual testing during the 13th and 14th JVET meetings. The test revealed that a revision of the test configurations might be indicated in order to align the duration of the video sequences in the visual assessment. A duration of at least 5 s or more was suggested. Furthermore, the presentation of HD video sequences was recommended in order to enlarge the number of JVET experts qualified to participate in the viewing sessions.

# Introduction

Remote expert viewing tests were conducted during the 22nd JVET meeting for the exploration activity on DNN-based coding tools defined in EE1 [3]. Calls for participation in the subjective viewing were issued on the JVET reflector. Overall, 19 experts volunteered in participating in the tests. The testing procedure follows an online variant of the method used for visual testing during the 13th and 14th JVET meetings [1][2].

# Test arrangement

EE1-participants were requested to prepare and submit mp4 files of 2-3 rate points which they considered to be suitable for visually demonstrating the proposed scheme. All sequences were at UHD resolution and taken from the EE1 test set, including RA and LD configurations. According to the EE1 test configuration, the sequences were of different length, with the test sequence Campfire (30 Hz) having 10 s duration while other sequences in RA configuration were of 5 s length. All sequences coded in LD configuration were of 3 s length. The set of sequences under test is listed below:

JVET-U0053/CatRobot1_3840x2160_60fps_10bits_JVET-U0053_EE1-2.2_QP37_RA.mp4

JVET-U0053/Tango2_3840x2160_60fps_10bits_JVET-U0053_EE1-2.2_QP37_RA.mp4

JVET-U0061/Campfire_3840x2160_30fps_10bits_JVET-U0061_EE1_QP37_RA_test.mp4

JVET-U0061/Campfire_3840x2160_30fps_10bits_JVET-U0061_EE1_QP42_RA_test.mp4

JVET-U0096/Campfire_3840x2160_30fps_10bit_JVET-U0096_EE1_QP37_RA.mp4

JVET-U0096/DaylightRoad2_3840x2160_60fps_10bits_JVET-U0096_EE1_QP37_RA.mp4

JVET-U0096/Tango2_3840x2160_60fps_10bits_JVET-U0096_EE1_QP37_RA.mp4

JVET-U0099/Campfire_3840x2160_30fps_10bits_JVET-U0099_EE1_QP42_RA.mp4

JVET-U0099/FoodMarket4_3840x2160_60fps_10bits_JVET-U0099_EE1_QP42_RA.mp4

JVET-U0099/Tango2_3840x2160_60fps_10bits_JVET-U0099_EE1_QP42_RA.mp4

JVET-U0104/CampfireParty2_3840x2160_30fps_10bits_JVET-U0104_EE1-1.4_QP42_RA.mp4

JVET-U0104/DaylightRoad2_3840x2160_60fps_10bits_JVET-U0104_EE1-1.4_QP42_LB.mp4

JVET-U0104/FoodMarket4_3840x2160_60fps_10bits_JVET-U0104_EE1-1.4_QP42_LB.mp4

JVET-U0115/CampfireParty2_3840x2160_30fps_10bits_JVET-U0115_EE1-1.3_QP42_RA.mp4

JVET-U0115/DaylightRoad2_3840x2160_60fps_10bits_JVET-U0115_EE1-1.3_QP42_LB.mp4

JVET-U0115/FoodMarket4_3840x2160_60fps_10bits_JVET-U0115_EE1-1.3_QP42_LB.mp4

VTM11.0_RPR2/CampfireParty2_3840x2160_30fps_10bits_EE1_VTM11_RPR2_QP37_RA.mp4

VTM11.0_RPR2/CatRobot1_3840x2160_60fps_10bits_EE1_VTM11_RPR2_QP37_RA.mp4

VTM11.0_RPR2/FoodMarket4_3840x2160_60fps_10bits_EE1_VTM11_RPR2_QP37_RA.mp4

VTM11.0_RPR2/Tango2_3840x2160_60fps_10bits_EE1_VTM11_RPR2_QP37_RA.mp4

# Test procedure

## Test session setup

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM 11.0 anchor. It was held as an online session via Zoom with the participants having downloaded the (anonymized) mp4 files before hand. The VLC playlists for the sessions were provided to them shortly before the start of the viewing.

In the following, video A and video B denote either the anchor or the proposal for the same sequence and same QP for a test point of the core experiment under evaluation. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects. Two viewing sessions were conducted. A viewing session consisted of 12 test cells. For the 5 s sequences each test cell constructed as follows:

- “Original” (on a mid-gray background, 1 s presentation)

- Original Video

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “Vote N” (on a mid-gray background, 5 s presentation)

For the Campfire sequence, the versions “A” and “B” were only shown once such that the presentation time of the sequences was comparable. For the LD sequences, the videos were repeated once each to extend the exposure time (playing: Original Original + A A + B B + A A + B B).

The test subjects were asked to rate the video sequences presented in each test cell as follows:

- “A was much better than B”

- “A was better than B”

- “B was better than A”

- “B was much better than A”

The used voting sheet is presented in the figure below. The participants were supposed to fill the voting sheet and enter their results into an online form after completion of both sessions. The participants were also asked to enter the used screen size and any specific observations in a comment field of the online form.

The voting sheets were used in order to minimize the cognitive load for the participants during the session.

Fig. 1: Voting sheet

Before the start of the test session, the viewers were provided a demo playlist in order to familiarize themselves with the procedure of the test. The viewers were verbally instructed about the procedure and the rating scale in the introduction of the session.

## Voting scores

For evaluation, the votes of the test subjects were mapped to numbers as follows:

- +3	: “Proposal much better than anchor”

- +1	: “Proposal better than anchor”

- -1	: “Anchor better than proposal”

- -3	: “Anchor much better than proposal”

From these values, the MOS and the 95% confidence intervals were calculated. A resulting MOS value of zero indicates that anchor and proposal were rated to have the same visual quality.

# Test results

The results of all but one participant were included in the evaluation, no further processing of the data was applied. Some participants reported problems with the presentation of the sequences for JVET-U0061. Therefore, the presented results for this proposal may considered not to be conclusive.

# Conclusions

The results reveal performance improvements for the proposals compared to the VTM anchor. For the superresolution approach, a more significant improvement was observed. Participants found the very short LD sequences difficult to watch. They also found it difficult to cope with the different sequence lengths and the fact that one sequence was of full length and shown only once. It is noted that the EE configuration has been previously used for objective evaluation only and that the suitability for subjective evaluation was not taken into account.

In general, it is suggested to align the length of the sequences to be assessed visually. A sequence length of 5 – 10 s would be more suitable for subjective evaluation. It is further suggested to consider a lower resolution of the test sequences used in the subjective evaluation since the number of remote participants with access to HD displays may be higher than the number of available UHD displays. Therefore, HD sequences might be considered for this task. If UHD characteristics should be maintained, cropping an appropriate region from the full resolution could be used.

# References

- V. Baroncini, A. Norkin, A. M. Kotra, K. R. Andersson, K. Misra, H. Jang, C.M. Tsai, D. Rusanovskyy, “Subjective assessment of CE11 proposals,“ Document JVET-M0906, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 13th meeting, Jan. 2019.

- M. Wien, P. Hanhart, “Core Experiment Viewing Test Procedure and Results,” Document JVET-N0835, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, 14th meeting, Mar. 2019.

- E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. M, H. Wang, „EE1: Summary of Exploration Experiments on Neural Network-based Video Coding,“ Doc. JVET-V0023, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH,  22nd meeting, Apr. 2021.