| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0021-v2 |
| --- | --- |

| Title: | Deployment status of the VVC standard |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp 1 Microsoft Way Redmond, WA 90852 USA | Tel: Email: | +1 (425) 703-5308 garysull@microsoft.com |
| Source: | Chair |  |  |

_____________________________

# Abstract

This contribution contains a survey of deployed products and services, publicly available software source code, and related tools supporting the VVC standard (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Except for a restructuring of the document to merge two sections without changing content, revision marking is included to show changes relative to JVET-W0021-v2 of April 2021.

# Introduction to the VVC standard

The VVC standard (formalized as Rec. ITU-T H.266 and ISO/IEC 23090-3) [1] was finalized in July 2020. In ITU-T it was approved in August 2020 and formally published “in force” in November 2020. In ISO/IEC, its final approval ballot was completed in January 2021 and the standard published in February 2021. A journal article with an overview of the new standard and a historical perspective on the evolution of its features has been published by IEEE for “early access” [2]. Web sites with further information about the new standard are found at [3][4][5].

The compelling advantage of VVC is that it can achieve roughly double the compression efficiency of the previous flagship ITU-T and ISO/IEC High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 and ISO/IEC 23008-2) and about four times the compression efficiency of the previously-dominant H.264/MPEG-4 AVC standard (Rec. ITU-T H.264 and ISO/IEC 14496-10) – enabling, e.g., storage of more video content in the same storage space with the same video quality, sending more channels in a broadcasting service, reducing the bit rate needed for sending video on a network, or alternatively providing a major increase in video frame quality, frame rate, or picture resolution (when using a sufficiently powerful encoder). The need for maximal compression efficiency for video is pressing and urgent, since about 80% of the data transmitted on world-wide networks is video, and that percentage has been steadily growing and is projected to continue to grow further [6][7]. The introduction of ultra high definition (UHD), high dynamic range (HDR), wide color gamut (WCG), and high frame rate (HFR) video services have increased the challenge with their use of larger frame sizes, faster refresh rates for progressive-scan video, and greater pixel bit depth. For example, by 2023, it is projected that about two-thirds of installed flat-panel television will support UHD resolution (up from one third in 2018) [8].

The complexity requirements for implementation of VVC decoders are relatively modest (less than twice the complexity of a comparable HEVC decoder and less than four times the complexity of a comparable AVC decoder), and VVC encoders, while being somewhat more of a challenge, are also quite feasible and are vendor-customizable and generally less cost-sensitive.

In addition to providing a large improvement in compression efficiency capability, the VVC standard has been designed to address a very broad range of applications, including new and emerging use cases, and hence the inclusion of “versatile” in the name of the standard. The capabilities built into version 1 of the VVC standard include features designed to address high dynamic range (HDR), wide colour gamut (WCG), resolution-adaptive multi-rate video streaming services, 360-degree immersive video, bitstream extraction and merging, temporal scalability, gradual decoder refresh, and multi-layer coding to deliver layered video content to support application features such as multiview, alpha maps, depth maps, and spatial and quality scalability.

Along with the VVC standard itself, an accompanying standard known as Versatile Supplemental Enhancement Information (VSEI) has also been developed (formalized as Rec. ITU-T H.274 and ISO/IEC 23002-7) [9]. The VSEI specification defines supplemental metadata in a generalized way that can be used with VVC and also with systems specifications and possibly with other video coding standards as well. Like VVC, the VSEI standard was finalized in July 2020, and its final approval and publication had a very similar timeline. Both of these standards were produced by a joint collaborative team containing experts of ITU-T Study Group 16 and ISO/IEC JTC 1/SC 29 that is known as the Joint Video Experts Team (JVET).

# Publicly available software source code

- JVET has developed the VVC Test Model (VTM) as its reference software encoder and decoder codebase [4]. It is intended primarily to demonstrate coding efficiency capability and proper interpretation of the syntax and decoding process specified in the standard (but not as a speed-optimized implementation), and is intended to be usable as a starting basis for product implementations. The software is available under a BSD copyright licence.

- InterDigital developed a multi-threaded VTM decoder, and reported 6–10× speed-up relative to the single-threaded reference software in October 2020 [10]. It is intended to support all features of the VTM. The software was later placed in an accessible repository, and it is available under the same BSD copyright licence as the VTM software [11]. A further update to correspond with the newer VTM and improve speed by about 10% using various scheduling and low-level changes was reported in April 2021 [12].

- Fraunhofer HHI announced the VVenC encoder and VVdeC decoder open-source software (release 0.1) in September 2020 [13][14][15][16][17]. It includes support for multithreading operation, single-pass rate control, perceptual QP adaptation, and motion-compensated temporal filtering (MCTF). The software has four defined presets for quality/speed tradeoff (called “slow”, “medium”, “fast”, and “faster”). Subjective testing reported in October 2020 indicated that the VVenC encoder had about the same or better subjective compression performance as the VTM encoder when operating in its “medium” speed configuration (operating with MCTF and QP adaptation disabled in the VTM and enabled in VVenC and with rate control disabled in both) with encoding speed more than 100× that of the VTM, for 4K UHD SDR video content [17][18]. As of December 2020, a “slower” preset was added, along with an improved single-pass rate control and a new two-pass rate control [19]. The “slower” preset mode reportedly achieves approximately the same BD-rate coding efficiency as the VTM while providing a speedup of more than 8.6× for UHD and 5.2× for HD sequences relative to the VTM. As of December 2020, with release 0.2, the software is available under a BSD copyright licence. Release 0.3 of March 2021 includes substantial further speed and multithreading improvements [20].

- Friedrich–Alexander University Erlangen–Nürnberg released an open-source bitstream analyser as an add-on for the VTM decoder [21][22]. The analyzer counts the occurrence of coding tools and coding modes used in a decoded bitstream and can be used for evaluating the decoding energy and time demands of VVC features. The software is available under a BSD copyright licence.

# Software decoders

- Sharp announced a real-time software decoder in June 2020, and issued a corresponding press release in December 2020 [23][24]. As of June 2020, it was reportedly capable of decoding 4K common test conditions (CTC) UHD bitstreams at up to 40 Mbps at more than 60 fps.

- Tencent announced its O266dec software decoder with SIMD and multithreading support and an associated FFmpeg/VLC-based video player in October 2020 [17][25][26]. As of December 2020, it is reportedly more than 3× the speed of the VTM reference software decoder when tested under VVC CTC in single-threaded operation and about 20× the VTM decoder speed in 8-thread operation. It could reportedly decode UHD video at more than 60 fps at up to 40 Mbps and decode full HD video at more than 200 fps. In December 2020, a version with mobile platform support based on Arm Neon technology was reported. On an Apple A14 processor (iPhone 12pro) in single-threaded operation, it could reportedly decode 8-bit 1080p CTC bitstreams at more than 50 fps, and in multi-threaded operation it could decode such bitstreams at more than 100 fps and could decode 8-bit 4K UHD bitstreams at more than 30 fps in the random access (RA) example configuration [27]. Although 8-bit operation was more optimized, the decoder also supports 10-bit operation.

- Alibaba announced its Ali266 decoder for mobile devices (e.g., Android and iPhone) in December 2020 [28]. It includes optimizations for multi-threading, ARM assembly, cache efficiency, and memory usage, particularly for 8 bit video content. Real-time speed was reported for 8 bit 720p, 1080p (using 2–4 threads for up to 60 fps) and 4K (up to 7 Mbps) video content with the ALF feature disabled. In April 2021, updated information was provided with both ALF and CCALF enabled [29]. This version of Ali266 reached 15× speed-up compared to VTM-11.0 on Android P40 with 8 threads on the CTC bitstreams of class A (4K) and class B (1080p), and performed real-time decoding of tested 720p e-commerce content on most phones with 3 threads. In July 2021, updated information was provided about another new version [30]. For CTC classs A1 (4K), A2 (4K), B (1080p) and C (720p), its “slow” preset reportedly achieves BD rates for {Y, U, V, YUV} of {10.95%, −6.11%, −4.50%, 7.11%} under the RA configuration with an average speedup factor of 227 times over VTM 13.0. For internal VOD content, Ali266 “fast” preset was compared to the x265 open source HEVC software encoder “medium” preset, and reportedly achieved BD rates for {Y, U, V, YUV} of {−32.87%, −46.07%, −42.70%, −35.35%} under the RA configuration. Ali266 “fast” preset is aimed at real-time VVC encoding, and currently achieved average encoding speeds of 19.3 fps for classes 1920×1080/1080×1920 and 37.7 fps for classes 1280×720/720×1080.

- Kwai announced K266Dec in April 2020 [31]. The decoder is designed from scratch following the VVC specification and can support all coding tools specified in the VVC Main profile. For the Android platform, K266Dec can decode 2K 8-bit CTC bitstreams at 33 fps with single thread on Huawei P40, which is 4.11× of VTM-11.0 decoder. In addition, CTC bitstreams and UGC videos were tested on various Android phones based on the 50B+ daily playback data from Kwai App. For iOS platform, K266Dec can decode 2K 8-bit CTC bitstreams at 94 fps with single thread on iPhone 12 Pro Max, which is 4.78× compared to VTM-11.0 reference decoder.

- Bytedance announced its BVC decoder in April 2021 [32]. The BVC decoder supports various platforms such as Android, iOS, Linux/MacOS and Windows. It was optimized especially for mobile platforms with different decoding capabilities. For decoding of 4K 8-bit CTC bitstreams on iPhone 12 (core A14), it is reported that the BVC decoder achieves 22 fps on average with single thread, which is 10× faster than the VTM-11.0 reference decoder, and 55 fps can be achieved with 6 threads. For decoding of 2K 8-bit CTC bitstreams, it is reported that the BVC decoder achieves 86 fps on average with single thread, which is 8.8× faster than the VTM-11.0 reference decoder.

- Spin Digital announced a real-time software decoder and media player supporting VVC in June 2021 [33]. It includes support of the Scalable Main 10 profile and 8K capability at 60 and 120 fps on recent-generation CPUs.

# Services and encoding products

- Bytedance announced its BVC encoder in April 2021 [34]. The majority of the coding tools and features introduced in VVC have been integrated and optimized in the BVC encoder. In addition, it also includes real-world encoder features like scene change detection, rate control and multi-threading. When compared to the HM and VTM, the corresponding global BD-rate savings and speedup factors under the RA configuration are summarized as follows: “fast” preset (6 threads): 34.13% (vs. HM), 17× (HM), 131× (VTM); “slow” preset (6 threads): 41.63% (vs. HM), 1× (HM), 8× (VTM).

- KDDI Research announced a real-time VVC encoder with 4K @60 fps capability in September 2020 [35].

- Ateme launched support for VVC in its Titan family of products, and demonstrated the technology in an OTT channel launched in November 2020 [36].

- Bitmovin, in partnership with Fraunhofer HHI based on VVencC as described in item 3), announced support of VVC in its video encoding platform in November 2020 [37].

- MX Player and MX TakaTak, a streaming service in India, began OTT services using VVC in June 2021 [38].

# Bitstream analyser products

- Elecard announced support for VVC in its StreamEye and StreamAnalyzer products in April 2020 [39].

- ViCueSoft supports VVC in its VQ Analyzer bitstream analysis product, as of late 2020 [40].

# Conformance test sets

- A conformance test set is under development by JVET. It reached the CD stage of the ISO/IEC approval process in October 2020 [41].

- Allegro DVT began offering a conformance test set for VVC as early as January 2020 (i.e., initially in preliminary form before the finalization of the standard) [42][43][44].

# References

- ITU-T and ISO/IEC, Versatile Video Coding, July 2020 (with subsequent approval and publication). Available from ITU-T at https://www.itu.int/rec/T-REC-H.266 and from ISO/IEC at https://www.iso.org/standard/73022.html.

- Benjamin Bross, Jianle Chen, Jens-Rainer Ohm, Gary J. Sullivan, and Ye-Kui Wang, “Developments in International Video Coding Standardization After AVC, With an Overview of Versatile Video Coding (VVC)”, Proceedings of the IEEE (to appear), early access publication of January 2021 available at https://ieeexplore.ieee.org/abstract/document/9328514.

- JVET – Joint Video Experts Team, website hosted by ITU-T at https://www.itu.int/en/ITU-T/studygroups/2017-2020/16/Pages/video/jvet.aspx.

- Versatile Video Coding (VVC), software and information repository website hosted by Fraunhofer HHI at https://jvet.hhi.fraunhofer.de/.

- VVC: Versatile Video Coding, website hosted by Media Coding Industry Forum at https://www.mc-if.org/versatile-video-coding/.

- Cisco Systems, “Cisco Visual Networking Index: Forecast and Trends, 2017–2022”, Cisco Systems White Paper, November 2018. (The publication link was later redirected to a newer report with different content; the original report is available at http://web.archive.org/web/20181213105003/https://www.cisco.com/c/en/us/solutions/collateral/service-provider/visual-networking-index-vni/white-paper-c11-741490.pdf.)

- Cisco Systems, “Cisco Predicts More IP Traffic in the Next Five Years Than in the History of the Internet” (press release), 27 November 2018.

- Cisco Systems, “Cisco Annual Internet Report (2018–2023) White Paper”, Cisco Systems White Paper, 9 March 2020.

- ITU-T and ISO/IEC, Versatile Supplemental Enhancement Information, July 2020 (with subsequent approval and publication). Available from ITU-T at https://www.itu.int/rec/T-REC-H.274 and from ISO/IEC at https://www.iso.org/standard/79112.html.

- Srinivas Gudumasu, Tangi Poirier, Fabrice Urban, Franck Hiron, Remi Jullian, and Philippe de Lagrange (InterDigital), “Multi-threaded VTM decoder description and performance analysis”, doc. JVET-T0061 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Philipe de Lagrange (InterDigital), “Release of multi-threaded VTM decoder”, announcement to JVET of software availability at https://vcgit.hhi.fraunhofer.de/delagrangep/VVCSoftware_VTM/-/tree/VTM-10.2-MT, 9 December 2020.

- Fabrice Urban, Franck Hiron, Remi Jullian, and Philippe de Lagrange, “Multi-thread VTM decoder: information update”, doc. JVET-V0088 of of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- Fraunhofer HHI VVenC software repository. [Online]. Available: https://github.com/fraunhoferhhi/vvenc.

- Fraunhofer HHI, “H.266 / VVC”, website page, accessed 11 January 2021.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, and Detlev Marpe (Fraunhofer HHI), “Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations”, doc. JVET-T0099 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Christian Helmrich, Benjamin Bross, Jonathan Pfaff, Heiko Schwarz, Detlev Marpe, and Thomas Wiegand (Fraunhofer HHI), “Information on and analysis of the VVC encoders in the SDR UHD verification test”, doc. JVET-T0103 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Benjamin Bross, Ye-Kui Wang, Yan Ye, Shan Liu, Gary J. Sullivan, and Jens-Rainer Ohm, “Overview of the Versatile Video Coding (VVC) Standard and its Applications”, preprint submitted to IEEE Trans. Circuits and Systs. for Video Tech., December 2020.

- Mathias Wien and Vittorio Baroncini, VVC Verification Test Report for Ultra High Definition (UHD) Standard Dynamic Range (SDR) Video Content, doc. JVET-T2020 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, and Detlev Marpe (Fraunhofer HHI), “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-U0135 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 21st meeting: January 2021.

- Fraunhofer HHI, “VVenC Fraunhofer Versatile Video Encoder v0.3.0.0”, white paper, 3 March 2021.

- Matthias Kränzler, Christian Herglotz, and André Kaup (FAU), “Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics based on VTM-10.0”, doc. JVET-T0067 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- FAU VTM Analyzer repository. [Online]. Available: https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer.

- Frank Bossen (Sharp), Performance of a reasonably fast VVC software decoder, doc. JVET-S0224 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 19th meeting: June/July 2020.

- Sharp, “Sharp Develops 8K Real-time VVC Decoder, a World First”, 4 December 2020.

- Bin Zhu, Shan Liu, Xiaozhong Xu, Xianguo Zhang, Chenchen Gu, Liang Wang, and Weiwei Feng (Tencent), “Performance of a VVC software decoder”, doc. JVET-T0095 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Tencent O266 video player repository. [Online]. Available: https://github.com/tencentcloud/o266player.

- Yiming Li, Shan Liu, Yu Chen, Yushan Zheng, Sijia Chen, Bin Zhu, and Jian Lou (Tencent), “Performance of a VVC Software Decoder on Mobile Platform”, doc. JVET-U0071 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 21st meeting: January 2021.

- J. Chen, L. Wang, R.-L. Liao, and Y. Ye (Alibaba), “VVC software decoder implementation for mobile devices”, doc. JVET-U0088 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 21st meeting: January 2021.

- L. Yu, Z. Cui, L. Wang, J. Cheng, Z. Huang, and L. Xu (Alibaba), “VVC software decoder implementation for mobile devices”, doc. JVET-V0132 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- S. Fang, J. Guo, Z. Huang, J. Liu, S. Xu, L. Yu, J. Chen, R.-L. Liao, and Y. Ye (Alibaba), “Ali266: an optimized VVC software encoder implementation”, doc. JVET-W0127 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 23rd meeting: July 2021.

- W.-L. Feng, F.-L. Luo, Y.-S. He, Z.-H. Liu, S.-M. Meng, and X. Wen (Kwai Inc.), “VVC Software Decoder for Mobile Platforms”, doc. JVET-V0070 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- Y. He, L. Li, Y. Li, H. Yin, J. Zhang, L. Zhang, and Y. Zhang (Bytedance), “Performance of a VVC software decoder – BVC”, doc. JVET-V0128 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- Spin Digital, “Spin Digital Announces a VVC 8K Decoder and Media Player”, 21 June 2021.

- J. Cui, Y. Fan, Y. He, X. Jiang, H. Liu, H. Shi, H. Yang, H. Yang, H. Yin, J. Zhang, and L. Zhang (Bytedance), “An optimized VVC encoder implementation”, doc. JVET-V0127 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- KDDI Research, “KDDI Research Develops the world’s first 4K H.266 | VVC real-time encoder”, 1 September 2020.

- Ateme, “ATEME and The Explorers to Launch the First OTT Channel Promoting VVC”, press release, 10 November 2020.

- Bitmovin, “Bitmovin Enables Innovation with New VVC Codec Feature”, press release, 18 November 2020.

- “MX Player becomes first OTT to deploy H.266, cuts down video streaming data consumption into half”, Best Media Info, 16 June 2021.

- Elecard, “Elecard Video Analyzers Now Support VVC”, press release, 14 April 2020.

- ViCueSoft, “VQ Analyzer”, website accessed December 2020.

- Jill Boyce, Elena Alshina, Frank Bossen, Kei Kawamura, Iole Moccagatta, and Wade Wan (Editors), “Conformance testing for versatile video coding (Draft 5)”, doc. JVET-T2008 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Allegro DVT, “Allegro DVT Unveils the Industry's First VVC Compliance Test Bitstreams” press release in Design & Reuse, 21 January 2020.

- Allegro DVT, “Realtek Semiconductor Selects Allegro DVT’s H.266/VVC Compliance Streams”, press release, 29 October 2020.

- Allegro DVT, “H.266/VVC Elementary Streams”, Allegro DVT website, accessed 11 January 2021.

“Globally, IP video traffic will be 82 percent of all IP traffic (both business and consumer) by 2022, up from 75 percent in 2017.” … “The implications of video growth are difficult to overstate.” … “Because of video consumption patterns, the Internet now has a much busier busy hour. Because video has a higher peak-to-average ratio than data or file sharing, and because video is gaining traffic share, peak Internet traffic will grow faster than average traffic.” – Cisco Systems, November 2018 [6].