| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0023_r2 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma and H. Wang | Email: | elena.alshina@huawei.com, shanl@tencent.com, chenwei06@kwai.com, Franck.Galpin@InterDigital.com, yue.li@bytedance.com, mazhan@nju.edu.cn, hongtaow@qti.qualcomm.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Exploration Experiment 1 on Neural Network-based Video Coding. In total 6 tests have been completed within this EE between the JVET V and W meetings to study and evaluate NNVC technologies, analyze their performance and complexity aspects. Several variants of NN-based in-loop filters with complexity 20...600 kMAC/pxl providing average bit-saving in a range 1.1...8.4% (in random access configuration) have been demonstrated in this round of Exploration Experiment. Several variants of super-resolution (from available in VVC standard functionality to NN-based with signaled parameters) demonstrate 6...11% bit-saving in random access configuration for 4K content.

# Test conditions and evaluation criteria

Test in this Exploration Experimen follows AhG11 tets conditions and complexity assessment methodology [1]. Anchor is VTM11.0 with improved GOP-based temporal filter [2] enabled. Comparison of test results from last and this EE rounds shows that there is very tiny overlap between technologies under study in this EE and MCTF.  5 quality levels (corresponding QP=22, 27, 32, 37, 42) are used for tests in in-loop filters cathegory. For Super resolution cathegory even lower bit-rates diapason is used (corresponding QP= 27, 32, 37, 42, 47). Anchors were generated by AhG11 [10].

Unfotunately not all EE participants used the most recent version of results reporting template.

Most of the technologies under test in this EE are unchaged relatively to the last meeting. For them only Random Access configuration results have been up-dated, since new MCTF was enabled for both anchor and test. One participant was not able to provide results for JVET-V meeting, but provides results this meeting (only AI configuration was tested due to the limited computational resources).

# Exploration Experiment on NN-based in-loop filters

Test results for proposals in this category are shown in Table 1.

Following obsevrations can be done. NN solutions trained using DIV-2K (images) have lower performance in test configurations with motion compentsation. Content adaptivity by signaling rscaling factor for residual or/and switching between different NN-based filters models on picture or/and CTU level is source of performance improvement. Proponents keep searching for better complexity-performance trade-off in EE-related contribution [5].

Table 1. Test results in NN-based in-loop filters cathegory.

| # | Performance | Dec.T. | Features | #params | kMAC /pxl |
| --- | --- | --- | --- | --- | --- |
| JVET-W0062 | MSSSIM: AI: 4.6%(Y); 6%(Ch) L1: AI: 5.8%(Y); 13%(Ch) Anchor: VTM-12.0 w/o SAO and ALF | - | 8 Residual Blocks, normalized QP map as additional input, 2 SE (squeeze-and-excitation) blocks, 33 kernels, 64 channels (SAO and ALF are disabled in anchor and test) 2 tests: MSSSIM and L1 training. | 0.13M | 125 |
| JVET-W0130 | RA: 8.4%(Y); 22% (Ch) AI: 7.6%(Y); 19%(Ch) Anchor : VTM-11-nnvc | ×352 ×203 (CPU) | Input 10 planes: 4*Y+U+V+QP+3*Bsinfo, 72×72, #channels of all hidden layers are M=216 or K=72 ; 14 filter blocks, Residual scalling (per color, per picture, signalled); LongActivation = CONV1×1,K×M + LeakyReLu+CONV1×1,M×K ; | 8×1M | 624 |
| JVET-W0151 | RA:1.1%(Y); 4% (Ch) AI: 1.6%(Y); 5%(Ch) Anchor : VTM-11-nnvc | ×32 ×31 (CPU) | Input YUV444, 4 planes + QP map, 64×64, DepthwiseSeparableConvolutions (DSC) in 4 Residual Blocks | 23K | 23 |

NN-based filter archotectures tested in this EE are illustrated on Fig. 1, 2 and 3.

Fig. 1. Low-complexity NN-based in-loop filter with DSC from JVET-V0137.

Fig. 2. NN-based in-loop with SE blockd from JVET-W0062.

Fig. 3. "NN-based de-block" with long activation function from JVET-W0130.

For the training proponents used DIV2K [3-4] and BVI-DVC [5-6].

# Exploration Experiment on NN-based SuperRes-like techniques

Proposals in this category were tested at lower bit-rates, corresponding QP=27,..., 47. Most of propponents reported results only for classes A1 and A2 (4K). For the comparison with NN-based Super Resolution existing in VVC functionality (RPR) was test (results generated by EE coordinators and attached to this report).

Quality metrics computed unsing reconstructed videos in full resolution (so-called “PSNR2” in VTM SW). It needs to be noticed that “MS-SSIM2” computation is not supported by VTM. So, MS-SSIM was not reported for tests in this category.

Table 2. Test results of superresolution-like technologies in EE1.

| # | Performance | Dec.T. | Features | #params | kMAC /pxl |
| --- | --- | --- | --- | --- | --- |
| JVET-W0063 | RA(4K):5.6%(Y), ~ 27% drop (Ch) RA(4K):6.1%(Y), ~ 10% drop (Ch) Anchor : VTM-11-nnvc | ×2.3 ×2.3 | Test1: SuperRes CNN with const.param Test2: SuperRes CNN with signalled param NCTM5.0 used for CNN param compression; Simplified ESRGAN | 0.5M | 122 |
| JVET-W0105 | RA(4K):8.1%(Y), ~20% drop (Ch) AI(4K): 11% (Y), ~24% drop (Ch) Anchor : VTM-11-nnvc | - (×90 CPU) | RPR2 (VVC) YUV444 CNN with const.param 2 models (high / low QP) Simplified EDSR (multi-scale) | 2.7M (21.3M) | 344 |
| JVET-W0125 | RA(4K):11.2%(Y), ~15% drop (Ch) LD(4K):18%(Y), Anchor : VTM-11-nnv | no info | DCS-based Video Coding: Intra = spatial texture frames are coded in orignal resolution, Inter = temporal motion frames frames are coded 2 , synthesised with Motion Compensaiton & Texture Transfer Networks | 8.5M (24.25M) | no info |
| VVC RPR cfg#1 | RA(4K):2.3%(Y), ~31% drop (Ch) AI(4K):4.3%(Y), ~34% drop (Ch) | ×0.3 ×0.4 | All frames are coded downsampled by factor 2 vertically and horizontaly | NA | NA |
| VVC RPR cfg#3 | RA(4K):8.2%(Y), ~23% drop (Ch) AI(4K):4.7%(Y), ~35% drop (Ch) | ×0.3 ×0.4 | Adaptive selection between full size and downsampled size coding (curves overlap) | NA | NA |
| VVC RPR cfg#4 | RA(4K):6.1%(Y), ~22% drop (Ch) AI(4K):5.7%(Y), ~11% drop (Ch) | ×0.3 ×0.4 | Adaptive selection between full size and downsampled size coding (no curves overlap) | NA | NA |

Fig. 4. NN-based Super resolution based on ESRGAN from JVET-W0063.

Fig. 5. NN-based Super resolution based on EDSR from JVET-W0105 (Fig. from original paper profponent refers to).

Fig. 6. DCS-based Video Coding from JVET-W0125.

Needs to be noticed that rate-distortion curves often overlaps for tests in SuperRes category, so BD-rate measurement is no 100% reliable (Fig. 7). In additional test (performed by EE coordinatos) for VVC RPR with adaptive selection betrween full size and downsampled size coding 2 variants (with and w/o rate-distortion curves overlap) were tested. Obsevred influence of rate-distoprtion curves overlap was ~2%.

| Fig. 7. Typical examples of rate-distortion curves overlap in SuperResolution category. |  |
| --- | --- |

Proponents of JVET-W0063 and JVET-W0105 reported that they used BVI-DVC, proponents of JVET-W0125 used SJTU 4K sequences for training.

# Viewing preparation

Proponents were invited to suggest sequences and rate-point for viewing to demonstrate benefits of their proposal. By time of this document preparation following suggestions for the viewing have been done.

Among them are typical representative of NN based in-loop filter, NN-based super resolution and VVC RPR.  All sequences are 5 -10 sec , all same resolution (4K).

In-loop filter category.

| Sequence, configuration | Anchor |  | Test |  |  |
| --- | --- | --- | --- | --- | --- |
|  | QP | Bit-rate | Doc # | QP | Bit-rate |
| Campfilre, RA | 42 | 1651 | JVET-W0130 | 42 | 1651 |
| DayLightRoad, RA | 42 | 1093 | JVET-W0130 | 42 | 1094 |
| FoodMarket, RA | 42 | 1101 | JVET-W0130 | 42 | 1096 |

Super resolution category.

| Sequence, configuration | Anchor |  | Test |  |  |
| --- | --- | --- | --- | --- | --- |
|  | QP | Bit-rate | Doc # | QP | Bit-rate |
| Campfilre, RA | 45 | 1267 | JVET-W0105 | 37 | 1341 |
| FoodMarket, RA | 45 | 867 | JVET-W0105 | 37 | 925 |
| Tango, RA | 45 | 915 | JVET-W0105 | 37 | 928 |
| Campfilre, RA | 45 | 1267 | VVC RPR cfg#4 | 37 | 1332 |
| FoodMarket, RA | 45 | 867 | VVC RPR cfg#4 | 37 | 921 |
| Tango, RA | 45 | 915 | VVC RPR cfg#4 | 37 | 924 |

# Recommendation

- Organize and conduct viewing during JVET-W meeting for prepared materials (BoG probably required).

- Review EE documents for more detailed information.

# References

[1] S. Liu, A. Segall, E. Alshina, R.-L. Liao Common Test Conditions and evaluation procedures for neural network-based video coding technology JVET-V2016, JVET, 21th Meeting: by teleconference, 20-28 April. 2021.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

[3] L. Wang, S. Lin, X. Xu, S. Liu, C. Auyeung, X. Li EE1: neural network based in-loop filter using depthwise separable convolution and regular convolution JVET-W0151 JVET, 22th Meeting: by teleconference, , 07-16 July 2021.

[4] T. Ouyang, Y. Guo, Z. Chen (WHU), X. Xu, S. Liu (Tencent) EE1: SSIM based CNN model for in-loop filtering JVET-W0062, JVET, 22th Meeting: by teleconference, 07-16 July 2021 
[5] H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz (Qualcomm) EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer, JVET-W0130 JVET, 22th Meeting: by teleconference, 07-16 July 2021

[6] H. Wang,  J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz (Qualcomm) EE1-related: Neural Network-based in-loop filter with constrained computational complexity, JVET-W0131 JVET, 22th Meeting: by teleconference, 07-16 July 2021.

[7] T. Chujoh, Y. Yasugi, T. Ikai (Sharp) EE1.2.2: NN-based super resolution (JVET-V0073) JVET-W0063 JVET, 22th Meeting: by teleconference, 07-16 July 2021.

[8] A. M. Kotra, K. Reuzé, J. Chen, H. Wang, M. Karczewicz, J. Li (Qualcomm)  EE1-2.3: Neural Network-based Super Resolution JVET-W0105 JVET, 22th Meeting: by teleconference, 07-16 July 2021.

[9] Ming Lu, Zhan Ma (NJU), Zhenyu Dai, Dong Wang (OPPO) EE1-related: Tests on Decomposition, Compression and Synthesis (DCS)-based Technology (JVET-V0149) JVET-W0125 JVET, 22th Meeting: by teleconference, 07-16 July 2021.

[10] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Anchor%20performance