| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0052-v2 |
| --- | --- |

| Title: | CE-related: CABAC skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma Virginie Drugeon 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution proposes a CABAC skip mode for the high bit rate and super low delay use cases. CABAC skip mode was already proposed in JVET-M0089, JVET-N0207, and JVET-O0308 for VVC version1. The proposed scheme directly outputs binarized bins as a bitstream without any CABAC processing, therefore avoiding CABAC throughput issues without any additional building blocks. There are no technical changes compared to the previous proposals. This document reports the simulation results on high bit depth CTC. Simulation results show that CABAC skip mode can solve the throughput issues at the cost of 12%, 17%, 17% bitrate increase for AI, RA, LDB on PQ for HBD Low QP, 8%, 15%, 15% bitrate increase for AI, RA, LDB on HLG for HBD Low QP, and 3%, 3%, 3% bitrate increase for AI, RA, LDB on SVT for HBD Low QP.

Introduction

CABAC skip mode was proposed and reported several times during VVC version1 standardization [1][2][3][4][5][6]. This mode entirely disables CABAC processing, therefore avoiding CABAC throughput issues for the whole slice data. The CE for this meeting has two tests aiming to solve CABAC throughput issues by partially skipping the CABAC engine processing [7]. We suggest discussing the CABAC skip mode together with these proposals.

- CE3.1 (JVET-V0059 [8])

This contribution proposes to perform CABAC bypass alignment after the CCB threshold has been reached. It solves the throughput issue when bypassing the residual data while slightly increasing the number of bits.

- CE3.2 (JVET-V0178 [9])

This contribution proposes to always set the CCB threshold to 0 and to perform CABAC bypass alignment at the start of the residual data. It solves the throughput issue for all residual data at the price of increasing the number of bits.

Proposal

There are no technical changes compared to the previous proposals.

The proposed CABAC skip mode directly outputs binarized bins as a bitstream without any CABAC processing as shown in Figure1. This mode can be switched at stream level by a SPS flag. The binarization module can be implemented in the pipeline structure without additional buffer, so this mode can guarantee fixed processing delay regardless of the amount of bits in a stream. This mode decreases the coding efficiency, but it can avoid all CABAC throughput issues without any additional building blocks.

As for the binarization, we set the CCB threshold to 0. It means that all residual coefficients are bypass coded in both RRC and TSRC. In bypass coding, the binarization of residual data tends to use a smaller number of bins compared to normal coding. Therefore, it can be expected to reduce the loss due to skipping CABAC.

Figure1: CABAC skip mode.

Simulation results

The simulation is conducted using VTM-13.0 with the high bit depth CTC described in JVET-U2018 [10]. Table1 shows the simulation results of CABAC skip mode for Low QP condition.

Table1: Simulation results of CABAC skip mode (HBD Low QP CTC)

| HDR PQ | AI |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 13.04% | 15.22% | 15.57% | 12.82% | 14.92% | 15.39% | 85% | 82% |
| PQ422 | 11.06% | 12.39% | 12.35% | 10.85% | 12.18% | 12.26% | 90% | 81% |
| Overall | 12.05% | 13.80% | 13.96% | 11.83% | 13.55% | 13.83% | 87% | 82% |
| HDR PQ | LDB |  |  |  |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 17.98% | 22.18% | 21.48% | 17.29% | 21.52% | 21.26% | 86% | 87% |
| PQ422 | 16.97% | 18.15% | 17.54% | 16.30% | 17.67% | 17.45% | 83% | 86% |
| Overall | 17.47% | 20.17% | 19.51% | 16.79% | 19.60% | 19.35% | 84% | 87% |
| HDR PQ | RA |  |  |  |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 17.80% | 21.53% | 21.62% | 17.16% | 20.99% | 21.45% | 86% | 88% |
| PQ422 | 16.61% | 17.96% | 17.43% | 16.02% | 17.56% | 17.39% | 85% | 86% |
| Overall | 17.21% | 19.75% | 19.52% | 16.59% | 19.28% | 19.42% | 86% | 87% |
| Overall PQ | 15.58% | 17.90% | 17.66% | 15.07% | 17.48% | 17.53% | 86% | 85% |

| HDR HLG | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 7.81% | 8.45% | 9.18% | 93% | 70% |
| HLG422 | 7.41% | 7.89% | 8.47% | 86% | 70% |
| Overall | 7.61% | 8.17% | 8.82% | 89% | 70% |
| HDR HLG | LDB |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 14.15% | 16.22% | 17.13% | 77% | 80% |
| HLG422 | 15.01% | 16.29% | 16.28% | 78% | 79% |
| Overall | 14.58% | 16.26% | 16.70% | 77% | 80% |
| HDR HLG | RA |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 14.47% | 16.46% | 17.06% | 78% | 81% |
| HLG422 | 15.26% | 16.13% | 16.21% | 75% | 80% |
| Overall | 14.87% | 16.29% | 16.64% | 77% | 80% |
| Overall HLG | 12.35% | 13.57% | 14.06% | 81% | 77% |

| SVT RGB | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 2.13% | 1.84% | 1.79% | 80% | 59% |
| SVT12 | 3.40% | 2.94% | 2.92% | 82% | 63% |
| Overall | 2.77% | 2.39% | 2.36% | 81% | 61% |
| SVT RGB | LDB |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 1.68% | 1.82% | 1.82% | 72% | 70% |
| SVT12 | 3.79% | 4.00% | 3.99% | 70% | 74% |
| Overall | 2.74% | 2.91% | 2.90% | 71% | 72% |
| SVT RGB | RA |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 1.75% | 1.77% | 1.77% | 70% | 70% |
| SVT12 | 4.07% | 4.14% | 4.13% | 69% | 75% |
| Overall | 2.91% | 2.96% | 2.95% | 70% | 72% |
| Overall RGB | 2.80% | 2.75% | 2.74% | 74% | 68% |

Table2 shows the simulation results of CABAC skip mode for Low QP condition compared with the anchor of HM16.23 RExt provided in the AHG8 report [11]. This result shows that the coding performance of CABAC skip mode is similar to HEVC RExt (having CABAC) in AI conditions.

Table2: Simulation results of CABAC skip mode (HBD Low QP CTC over HM16.23)

| HDR PQ | AI |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.23 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | - | - | - | 6.25% | 7.30% | 6.75% | - | - |
| PQ422 | - | - | - | 0.80% | -2.51% | -3.33% | - | - |
| Overall | - | - | - | 3.52% | 2.39% | 1.71% | - | - |
| HDR PQ | LDB |  |  |  |  |  |  |  |
|  | Over HM16.23 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | - | - | - | 10.01% | 16.04% | 13.51% | - | - |
| PQ422 | - | - | - | 7.46% | 3.94% | 2.72% | - | - |
| Overall | - | - | - | 8.73% | 9.99% | 8.12% | - | - |
| HDR PQ | RA |  |  |  |  |  |  |  |
|  | Over HM16.23 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | - | - | - | 9.67% | 15.31% | 13.89% | - | - |
| PQ422 | - | - | - | 6.84% | 3.33% | 2.68% | - | - |
| Overall | - | - | - | 8.26% | 9.32% | 8.29% | - | - |
| Overall PQ | - | - | - | 6.84% | 7.23% | 6.04% | - | - |

| HDR HLG | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM16.23 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 3.04% | 1.93% | 2.26% | - | - |
| HLG422 | 1.87% | -0.10% | 0.35% | - | - |
| Overall | 2.45% | 0.91% | 1.31% | - | - |
| HDR HLG | LDB |  |  |  |  |
|  | Over HM16.23 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 7.60% | 6.93% | 8.48% | - | - |
| HLG422 | 8.86% | 5.18% | 7.08% | - | - |
| Overall | 8.23% | 6.06% | 7.78% | - | - |
| HDR HLG | RA |  |  |  |  |
|  | Over HM16.23 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 7.26% | 6.41% | 7.85% | - | - |
| HLG422 | 8.43% | 4.69% | 6.78% | - | - |
| Overall | 7.85% | 5.55% | 7.31% | - | - |
| Overall HLG | 6.18% | 4.17% | 5.47% | - | - |

| SVT RGB | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM16.23 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.30% | 0.24% | 0.18% | - | - |
| SVT12 | -0.78% | -0.08% | -0.18% | - | - |
| Overall | -0.54% | 0.08% | 0.00% | - | - |
| SVT RGB | LDB |  |  |  |  |
|  | Over HM16.23 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.67% | 1.00% | 1.00% | - | - |
| SVT12 | -0.59% | 2.19% | 2.17% | - | - |
| Overall | -0.63% | 1.59% | 1.59% | - | - |
| SVT RGB | RA |  |  |  |  |
|  | Over HM16.23 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.72% | 0.77% | 0.78% | - | - |
| SVT12 | -0.61% | 2.12% | 2.10% | - | - |
| Overall | -0.67% | 1.44% | 1.44% | - | - |
| Overall RGB | -0.61% | 1.04% | 1.01% | - | - |

Table3 shows the simulation results of CABAC skip mode for Normal QP condition. The coding performance loss is quite huge, but it still has enough advantage over HEVC.

Table3: Simulation results of CABAC skip mode (HBD Normal QP CTC)

| HDR | AI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 13.47% | 13.91% | 14.38% | 16.37% | 17.40% | 14.21% | 16.31% | 17.49% | 97% | 95% |
| Class H2 | - | - | - | - | - | 19.35% | 13.58% | 11.28% | 94% | 97% |
| Overall | 13.47% | 13.91% | 14.38% | 16.37% | 17.40% | 16.78% | 14.94% | 14.39% | 95% | 96% |
| HDR | RA |  |  |  |  |  |  |  |  |  |
|  | Over VTM13.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 15.86% | 18.50% | 18.51% | 15.80% | 16.99% | 18.39% | 15.76% | 16.75% | 100% | 100% |
| Class H2 | - | - | - | - | - | 22.12% | 12.68% | 10.32% | 96% | 102% |
| Overall | 15.86% | 18.50% | 18.51% | 15.80% | 16.99% | 20.25% | 14.22% | 13.54% | 98% | 101% |

Table4 shows the comparison of bit rate, Bin rate, and symbol rate using 6 sequences (in 3 QP points) of HBD Low QP condition. The symbol rate counts one syntax in the syntax table as one symbol. Each value of AI is multiplied by 8 since only one frame is coded for every eight frames in CTC. In CABAC skip mode, Bin rate is meaningless since this mode entirely disables CABAC processing, but this table has the value for the reference to compare with VTM13.0.

As for CABAC skip mode, the numbers of bypass Bin increase compared to VTM13, on the other, the numbers of context Bin and symbol drastically decrease since CCB threshold is always set equal to 0. In CABAC skip mode, the symbol rate will be the bottleneck of the throughput instead of Bin rate. The symbol rate of CABAC skip mode in this table is still huge considering 4K or higher resolutions. However, in the binarization stage, it is possible to implement the hardware as multiple symbols in one cycle hardware.

Table4: Comparison of bit rate, Bin rate, and symbol rate

|  |  | QP | VTM13.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | bit rata (kbps) | Bin rate (kbps) |  |  | symbol rate (ksps) |
|  |  |  |  | context | bypass | total |  |
| PQ 4:2:2 AI | Market3 (1920x1080, 50fps) (CTC results x 8) | -8 | 802,127 | 425,872 | 527,980 | 953,852 | 649,766 |
|  |  | 2 | 475,046 | 309,651 | 273,461 | 583,111 | 444,795 |
|  |  | 12 | 226,963 | 192,207 | 103,562 | 295,769 | 249,183 |
| PQ 4:2:2 LDB | BalloonFestival (1920x1080, 24fps) | -8 | 119,269 | 80,316 | 65,124 | 145,440 | 113,859 |
|  |  | 2 | 59,292 | 50,048 | 28,008 | 78,056 | 67,019 |
|  |  | 12 | 18,347 | 20,237 | 6,803 | 27,040 | 24,599 |
| HLG 4:4:4 AI | DayStreet (1920x1080, 60fps) (CTC results x 8) | -8 | 1,499,140 | 790,975 | 985,097 | 1,776,072 | 1,232,849 |
|  |  | 2 | 902,948 | 672,860 | 475,685 | 1,148,546 | 940,954 |
|  |  | 12 | 361,399 | 333,921 | 158,738 | 492,660 | 424,219 |
| HLG 4:4:4 LDB | PeopleInSC (1920x1080, 60fps) | -8 | 1,116,201 | 734,117 | 622,479 | 1,356,596 | 1,113,043 |
|  |  | 2 | 514,356 | 526,618 | 218,940 | 745,558 | 696,482 |
|  |  | 12 | 63,748 | 80,229 | 20,831 | 101,059 | 92,360 |
| SVT16 AI | OldTownCross (1920x1080, 50fps) (CTC results x 8) | -28 | 3,234,310 | 676,504 | 3,024,816 | 3,701,320 | 1,233,276 |
|  |  | -18 | 2,698,290 | 653,014 | 2,485,122 | 3,138,136 | 1,221,334 |
|  |  | -8 | 2,180,421 | 652,839 | 1,906,011 | 2,558,850 | 1,201,749 |
| SVT16 LDB | CrowdRun (1920x1080, 50fps) | -28 | 2,879,097 | 622,467 | 2,690,645 | 3,313,112 | 1,210,289 |
|  |  | -18 | 2,351,691 | 617,823 | 2,119,732 | 2,737,555 | 1,195,943 |
|  |  | -8 | 1,829,671 | 626,694 | 1,502,962 | 2,129,656 | 1,162,861 |

|  |  | QP | CABAC skip mode (Proposal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | bit rata (kbps) | Bin rate (kbps) (for reference) |  |  | symbol rate (ksps) |
|  |  |  |  | context | bypass | total |  |
| PQ 4:2:2 AI | Market3 (1920x1080, 50fps) (CTC results x 8) | -8 | 839,641 | 48,883 | 790,730 | 839,613 | 382,101 |
|  |  | 2 | 502,473 | 50,406 | 452,039 | 502,445 | 269,834 |
|  |  | 12 | 236,666 | 43,362 | 193,197 | 236,559 | 146,371 |
| PQ 4:2:2 LDB | BalloonFestival (1920x1080, 24fps) | -8 | 128,073 | 15,128 | 112,933 | 128,061 | 69,995 |
|  |  | 2 | 65,252 | 11,947 | 53,294 | 65,241 | 42,783 |
|  |  | 12 | 19,520 | 7,113 | 12,384 | 19,497 | 14,790 |
| HLG 4:4:4 AI | DayStreet (1920x1080, 60fps) (CTC results x 8) | -8 | 1,565,525 | 78,878 | 1,486,613 | 1,565,492 | 741,574 |
|  |  | 2 | 973,407 | 83,915 | 889,415 | 973,329 | 570,582 |
|  |  | 12 | 375,834 | 93,263 | 282,446 | 375,708 | 249,901 |
| HLG 4:4:4 LDB | PeopleInSC (1920x1080, 60fps) | -8 | 1,198,699 | 58,183 | 1,140,504 | 1,198,687 | 697,707 |
|  |  | 2 | 583,375 | 59,188 | 524,147 | 583,336 | 422,159 |
|  |  | 12 | 53,733 | 25,763 | 27,905 | 53,668 | 42,416 |
| SVT16 AI | OldTownCross (1920x1080, 50fps) (CTC results x 8) | -28 | 3,260,258 | 48,924 | 3,211,307 | 3,260,232 | 655,271 |
|  |  | -18 | 2,712,089 | 31,174 | 2,680,888 | 2,712,062 | 644,926 |
|  |  | -8 | 2,194,466 | 28,113 | 2,166,327 | 2,194,440 | 641,201 |
| SVT16 LDB | CrowdRun (1920x1080, 50fps) | -28 | 2,924,017 | 43,697 | 2,880,312 | 2,924,009 | 652,833 |
|  |  | -18 | 2,396,929 | 36,915 | 2,360,006 | 2,396,921 | 647,284 |
|  |  | -8 | 1,875,851 | 38,634 | 1,837,209 | 1,875,843 | 642,999 |

Conclusion

This contribution proposes a CABAC skip mode as was already proposed in JVET-M0089, JVET-N0207, and JVET-O0308 for VVC version1. The proposed scheme directly outputs binarized bins as a bitstream without any CABAC processing, therefore avoiding CABAC throughput issues without any additional building blocks. There are no technical changes compared to the previous proposals. This document reports the simulation results on high bit depth CTC. Simulation results show that CABAC skip mode can solve the throughput issues at the cost of 12%, 17%, 17% bitrate increase for AI, RA, LDB on PQ for HBD Low QP, 8%, 15%, 15% bitrate increase for AI, RA, LDB on HLG for HBD Low QP, and 3%, 3%, 3% bitrate increase for AI, RA, LDB on SVT for HBD Low QP. We believe that this mode is useful for the use cases of high bitrate and super low delay, and we propose to consider CABAC skip mode as a high throughput tool together with CE3 proposals.

References

[1] K. Abe, T. Toma, “Non-CE5: CABAC skip mode for super low delay”, JVET-M0089, JVET 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

[2] K. Abe, T. Toma, “AHG14: CABAC skip mode”, JVET-N0207, JVET 14th Meeting: Geneva, CH, 19–27 March 2019.

[3] K. Abe, T. Toma, D.-Y. Kim, “CABAC skip mode”, JVET-O0308, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[4] K. Abe, T. Toma, “Report of CABAC skip mode results on VTM-6.0”, JVET-P0939, JVET 16th Meeting: Geneva, CH, 1–11 October 2019.

[5] K. Abe, T. Toma, V. Drugeon, “AHG14: Report of CABAC skip mode results on VTM-7.0”, JVET-Q0149, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

[6] K. Abe, T. Toma, V. Drugeon, “AHG14: Report of CABAC skip mode results on VTM-8.0”, JVET-R0169, JVET 18th Meeting: by teleconference, 15–24 April 2020.

[7] D. Rusanovskyy, K. Naser, M. G. Sarwer, F. Wang, “Core Experiment on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-V2022, JVET 22nd Meeting: by teleconference, 20–28 Apr. 2021.

[8] M. G. Sarwer, J. Chen, Y. Ye, R.-L. Liao, “AHG8: CABAC-bypass alignment for high bit-depth coding”, JVET-V0059, JVET 22nd Meeting: by teleconference, 20–28 Apr. 2021.

[9] F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang, “AHG8: a combination of JVET-V0059 option 2 and JVET-V0122 for high bit depth and high bit rate extensions”, JVET-V0178, JVET 22nd Meeting: by teleconference, 20–28 Apr. 2021.

[10] A. Browne, T. Ikai, D. Rusanovskyy, M. G. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, JVET 21st Meeting: by teleconference, 6–15 Jan. 2021.

[11] A. Browne, T. Ikai, D. Rusanovskyy, M. G. Sarwer, X. Xiu, Y. Yu, “JVET AHG Report: High bit depth, high bit rate, and high frame rate coding”, JVET-W0008, JVET 23rd Meeting, by teleconference, 7–16 July 2021.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).