| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0057 |
| --- | --- |

| Title: | AHG11: Content-adaptive neural network post-processing filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maria Santamaria, Yat-Hong Lam, Jani Lainema, Francesco Cricri, Ramin Ghaznavi-Youvalari, Honglei Zhang, Alireza Zare, Hamed R. Tavakoli, Miska Hannuksela. | Email: | { maria.santamaria_gomez.ext, yat.lam, jani.lainema, francesco.cricri, ramin.ghaznavi-youvalari, alireza.zare, honglei.1.zhang, hamed.rezazadegan_tavakoli, miska.hannuksela}@nokia.com |
| Source: | Nokia Technologies |  |  |

_____________________________

# Abstract

This document proposes a content-adaptive post-processing filter based on Convolutional Neural Networks (CNNs). The work is based on the filter described earlier in JVET-V0075 by the same authors. The proposed filter is first trained offline on a large dataset, and then finetuned for each test video sequence. Each slice includes a scaling factor selected by the encoder and signalled in the bitstream. This scaling factor controls what portion of the filter’s impact is applied to the output. There is further CTU level control allowing the encoder to switch the filter on and off separately for luma and chroma. The proposed filter was implemented on top of VTM 11.0 and evaluated under Random Access (RA) configuration. It is reported that the proposed content-adaptive method outperforms both VTM 11.0 and the filter described earlier in JVET-V0075.

# Motivation and high-level idea

Convolutional Neural Networks (CNNs) have recently shown to be effective at removing artifacts generated by video codecs such as H.266/VVC, either as in-loop filter or as post-processing filter. These filters are usually trained on an image or video database in order to achieve good generalization ability. However, visual content can vary greatly and achieving sufficient artefact removal performance at test time may need a large training dataset and a high-capacity neural network (NN), i.e., using many learnable parameters. This results into a trade-off between coding efficiency and decoder-side computational and memory complexity. We propose a solution to this problem: having a relatively small post-processing NN which is pretrained on a sufficiently diverse dataset, and then specializing it on the input video sequence as part of the encoding operations. This way, we trade decoder-side complexity with encoder-side complexity, while achieving superior artefact removal, as we show in the experimental section.

# Network architecture and processing flow

We use two neural network architectures, which main difference is the number of layers and convolutional kernels. We refer to them as SmallNN and BigNN. The SmallNN is used for classes B, C, D, F, whereas the BigNN is used for class A. The architecture of our SmallNN is illustrated in Figure 1. The input to the NN are the YUV channels and an additional channel whose value is the normalized frame Quantization Parameter (QP). The first block comprises a convolutional layer with 64 kernels (excluding the biases), a bias layer, a LeakyReLU as the non-linear activation function. Follow 5 blocks with similar layers as in the first block, but where the input of each block is added to the block’s output (by using a skip connection) – we refer to each of these 4 blocks are a ResNet block. The last block is similar to the first block (no skip connection). The input YUV channels are summed to the output of the neural network.

Figure 1. Overview of the architecture of our neural network post-processing filter.

The BigNN is similar to the SmallNN, but the number of convolutional kernels is 512 instead of 64.

The general procedure can be divided into three main stages. The first stage consists of an offline pretraining of the neural network filter on a video dataset. The second stage consists of finetuning the pretrained NN filter on the input video sequence, obtaining a weight-update. The third stage is the actual encoding-decoding operation, which consists of encoding the video and the weight-update, decoding the encoded video and the encoded weight-update, updating the NN filter by using the decoded weight-update, using the updated NN filter for post-processing the decoded video frames.

A general overview of the workflow is illustrated in Figure 2.

Figure 2. Overview of the workflow for our proposed methodology. Taken from [2].

The following sections explain in more detail the different stages.

# Pretraining

The pretraining is performed offline, using the BVI-DVC dataset [3]. We have trained one neural network for each content class in BVI-DVC datasets, which varies in the content resolution from 4K to WQVGA. From each of the 200 video sequences, we used all 64 frames, except for class A, for which only the first 16 frames were used in order to reduce the training time. 160 randomly selected sequences were used as the training set, and 40 sequences were used as the validation set. The video sequences were encoded using VTM11.0 encoder with sequence-QP values equal to 22, 27, 32, 37 and 42 in Random Access (RA) configuration, and then reconstructed by the decoder. For each epoch, all the frames from each video sequence are selected once in a randomized order.  From each frame in these video sequences, a 128x128 patch from the reconstructed sequence was used as the input to the NN filter, and the corresponding uncompressed patch was used as the ground-truth data. The patches were cropped by choosing their position randomly within the frames. Whenever necessary, padding was performed by using boundary pixels. The normalized frame-QP was used as an additional input channel, stacked to the Y, U, V channels. The chroma channels were upsampled by duplicating the neighboring rows and columns.

A separate network was trained for each class.

As a training loss, we used weighted mean squared error (MSE) between the filtered output patches and the ground-truth patches, computed in YUV domain. The MSE values of Y, U and V channels are weighted as follows: 8:1:1.

Training was performed until the validation loss flattened or was about to flatten.

| Network Information in Pretraining Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Nvidia Tesla V100 |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 300 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted MSE |
|  | Training time: |  |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | As in CTC |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

# Finetuning for content adaptation

The finetuning stage aims to adapt the pretrained neural network filter to the target video content. In order to generate input data for the finetuning stage, all sequences in JVET-CTC video dataset were encoded by VTM 11.0 using the Random Access (RA) anchor settings with QP values equal to 22, 27, 32, 37 and 42. As a result, each encoding generated input data for obtaining a finetuned network to be used for the specific sequence with the specific sequence QP.

The reconstructed frames and original frames are cropped into 128x128 crops, forming the input data and the ground-truth data, respectively.

The frame-QP of the reconstructed patches is used as the fourth input channel. For each video sequence and each QP, we finetune the NN filter on all patches and all frames.

Starting from the pretrained NN weights, in the finetuning process only the bias terms are optimized, in order to reduce the bitrate overhead required by the update. Out of all 151,811 parameters, only 323 represent the trainable bias terms. We use the weighted MSE in YUV domain as the loss function. In each finetuning epoch, all patch-pairs are randomized and fed into the training process. Finetuning is stopped when the training loss has flattened or was about to flatten.

At the end of finetuning, a weight-update is computed as the difference between finetuned bias terms and pretrained bias terms. A single weight-update is obtained for the whole video sequence and for all Y,U,V channels in that video sequence. This weight-update is then compressed using LZMA2 algorithm.

| Network Information in Finetuning Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Nvidia Tesla V100 |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 100 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted MSE |
|  | Training time: |  |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | As in CTC |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

# Encoding and decoding

The compressed weight-update obtained from the content-adaptation process is first decompressed and then used to update the NN filter at decoder side, simply by adding the bias-update values to the original (pretrained) bias terms.

The content-adaptive post-processing filter was integrated into VTM 11.0 using TensorFlow C API (version 2.3.1) and the provided dynamic libraries [4]. The filter is applied per frame. For luma, a scaling factor is selected. For chroma, the scaling factor is always 1. The filtered output is computed as follows:

filtered=CCNrec-rec*s+rec,

where rec refers to the input to the NN filter, CCN(rec) refers to the output of the NN filter, s is the scaling factor. Although the filter is applied per frame, its usage is determined at CTU level. The bitstream syntax is modified by adding one slice parameter that determines the scaling factor to be used for the given frame and two CTU-level flags (one for luma and one for chroma) that indicate whether the NN filter is used. Finally, the decision about the value of the scaling factor and the values of the two CTU-level flags is done by means of Rate Distortion Optimisation (RDO). The scaling factor’s value can take one of the following values: 0.8, 0.9, 1.0, 1.2.

The Multiply-Accumulate per pixel is given by:

MAC per pixel=kernelSize2×∑(inputChannels×outputChannels)
=3×3×(4×64+4×64×64+64×3)

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task |  |
|  | Total Parameter Number | 151811 |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 0,579113007 |
|  | Multiply Accumulate (MAC) |  |
| Optional |  |  |
|  | Total Conv. Layers | 6 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 8 |
|  | Patch size | 128x128 |
|  | Changes to network configuration or weights required to generate rate points | 1 NN for all QPs. 1 NN per content class. |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | Padding with boundary pixels when necessary |
|  | Other information: |  |

# Experimental results

The evaluation was done using the Random Access (RA) configuration and the Common Test Conditions (CTC) for NN-based video coding technology [5].

In one of the two attached Excel tables, the anchor is VTM 11.0. In the other Excel table, the anchor is VTM 11.0 augmented with our finetuned post-processing filter with a single scaling factor (1.0). This latter case is to show the gains brought by the content-adaptation process.

Table 1 summarizes our experimental results over VTM-11.0. Table 2 summarizes our experimental results where the anchor is the case when all scaling factors are fixed to 1. This table shows the effectiveness of using RD-optimized scaling factors. Table 3 summarizes our experimental results over VTM-11.0 augmented with a pretrained post-processing filter, showing the effectiveness of the content-adaptation technique. Table 4 summarizes our experiment results over [1]. The main differences of the proposed method with respect to [1] are: (i) models were pretrained for longer time; (ii) models were finetuned for longer time; (iii) the use of RD-optimized scaling factors.

Table 1. Summarized experimental results over VTM-11.0

|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class C | -3,02 % | -5,68 % | -3,66 % | -1,14 % | -6,51 % | -4,03 % | 126 % | 47611 % | 1 % |
| Class D | -3,37 % | 0,53 % | 0,61 % | -0,16 % | -0,96 % | 0,24 % | 113 % | 17189 % | 1 % |
| Class F | 0,44 % | 1,80 % | 2,90 % | 0,11 % | -0,28 % | 1,31 % | 124 % | 26836 % | 0 % |

We may provide results for classes A and B in an update to this contribution.

Table 2. Summarized experimental results over our method with scaling factors always equal to 1 for luma and chroma

|  | BD-rate Over VTM-11.0 + NN filter single scaling factor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT |  | DecT | bit DIFF |
| Class C | -0,23 % | -0,32 % | -0,26 % | -0,07 % | -0,19 % | -0,16 % | 94 % |  | 80 % | 0 % |
| Class D | -0,20 % | -0,33 % | -0,16 % | -0,01 % | 0,04 % | 0,00 % | 98 % |  | 77 % | 0 % |
| Class F | -0,13 % | -0,08 % | -0,06 % | 0,03 % | -0,09 % | -0,07 % | 95 % |  | 86 % | 0 % |

Table 3. Summarized experimental results of [1], over VTM-11.0

|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -0,38 % | -2,99 % | -2,47 % | -1,68 % | -3,38 % | -2,21 % | 119 % | 17037 % | 2 % |
| Class A2 | 0,14 % | -2,60 % | -0,31 % | 0,86 % | -2,65 % | 0,27 % | 116 % | 15765 % | 2 % |
| Class B | -1,37 % | -7,79 % | -5,28 % | 0,30 % | -8,34 % | -5,30 % | 137 % | 50644 % | 1 % |
| Class C | -2,36 % | -5,23 % | -3,27 % | -0,32 % | -5,80 % | -3,32 % | 127 % | 69109 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1,13 % | -5,11 % | -3,19 % | -0,15 % | -5,53 % | -3,04 % | 126 % | 35037 % | 1 % |
| Class D | -2,81 % | -1,63 % | -0,09 % | 0,45 % | -2,10 % | -0,04 % | 110 % | 23799 % | 1 % |
| Class F | 0,21 % | -0,73 % | 0,16 % | 1,06 % | -1,02 % | 0,54 % | 122 % | 14340 % | 1 % |

# Conclusions

We proposed a content-adaptive neural network post-processing filter. A pretrained neural network is finetuned on the target video sequence, obtaining a weight-update for the whole video sequence and all Y,U,V channels in that video sequence. The weight-update is compressed and used at decoder-side to update the pretrained NN filter. The filter is combined with its input based on a scaling factor which is optimized on a frame-basis.

In our experimental results, we showed how finetuning boosts the performance of the pretrained NN filter, by specializing it to the target content. Also, we showed the benefit of optimizing scaling factors.

In the future, we plan to study impacts of the technique when used as a loop filter.

# References

- Y.-H. Lam, M. Santamaria, J. Lainema, F. Cricri, R. Ghaznavi-Youvalari, A. Zare, H. Zhang, H. R. Tavakoli, and M. Hannuksela. AHG11: Content-adaptive neural network post-processing filter. Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-V0075. April 2021.

- Y.-H. Lam, A. Zare, F. Cricri, J. Lainema, and M. M. Hannuksela. 2020. Efficient Adaptation of Neural Network Filter for Video Compression. In Proceedings of the 28th ACM International Conference on Multimedia (MM '20). Association for Computing Machinery, New York, NY, USA, 358–366. DOI:https://doi.org/10.1145/3394171.3413536

- Zhang, F. (Creator), Ma, D. (Creator), Bull, D. (Creator) (31 Mar 2020). BVI-DVC. University of Bristol. 10.5523/bris.3hj4t64fkbrgn2ghwp9en4vhtn

- Install TensorFlow for C, https://www.tensorflow.org/install/lang_c

- JVET-U2016. Common Test Conditions and evaluation procedures for neural network-based video coding technology.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).