| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0071-v3 |
| --- | --- |

| Title: | AHG9: Green Metadata SEI message for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Herglotz, M. Kränzler, A. Kaup (FAU) E. Francois, M. Radosavljevic, E. Reinhard (InterDigital) X. Ducloux (Harmonic) D. Menard (INSA) Yong He, Muhammed Coban, D. Rusanovskyy, Marta Karczewicz (Qualcomm) | Email: | christian.herglotz@fau.de matthias.kraenzler@fau.de andre.kaup@fau.de edouard.francois@interdigital.com milos.radosavljevic@interdigital.com xavier.ducloux@harmonicinc.com daniel.menard@insa-rennes.fr erik.reinhard@interdigital.com yonghe@qti.qualcomm.com |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU), Interdigital, INSA-Rennes / IETR, Harmonic Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an initial update of the ISO_IEC_23001-11 (Green Metadata) specification with the definition of a new Green Metadata SEI message for VVC. The update specifically addresses syntax signalling for quality recovery after low power encoding and decoder complexity metrics. For the former, it is proposed to adopt the same syntax as already used for HEVC and AVC and add syntax for signalling the quality based on SSIM, VMAF, wPSNR and WS-PSNR, which have become widely used quality metrics throughout the last years. For the latter, the update covers decoder complexity metrics for VVC, which are not covered by existing SEI messages for AVC and HEVC. The proposed syntax is based on extensive decoding complexity measurements performed on a large video dataset using VTM and VVdeC decoding. The measurement results are analyzed in detail using Tool On/Off tests and complexity modeling. As for the AVC and HEVC Green metadata SEI messages, the new SEI message is defined in ISO/IEC 23001-11, and a reference to ISO/IEC 23001-11 is inserted in the VSEI specification for the definition of Green metadata SEI message of VVC.

# Introduction

The ISO/IEC 23001-11 (Green Metadata) specification [2] defines a set of metadata characterizing video content and bitstreams, which are aimed at controlling and reducing the energy consumption in the encoding, decoding, and display process while preserving the user’s quality of experience. Especially, these metadata can help to reduce decoder power and to recover visual quality after low-power encoding, as illustrated in [2].

It was reported that the recently issued VVC video coding specification increases the complexity of software decoding by more than 75% [1] in comparison with its predecessor HEVC. As software decoding may represent an important part of the deployed decoders in end-user devices, it is desirable to update the Green Metadata specification in order to allow efficient video encoding and decoding for VVC coded bit streams.

For H.264/AVC and H.265/HEVC, such SEI messages are available and described in detail in [2]. In this contribution, the extension for VVC is proposed and described for SEI messages allowing decoder power reduction (Chapter 5 from [2]) and quality recovery after low-power encoding (Chapter 8 from [2]). The definition of this new SEI message is based on extensive decoding complexity measurements performed on a large video dataset using VTM and VVdeC decoding.

The rest of this contribution is structured as follows. Per-tool complexity measurements are described and reported in Section 2. Complexity modeling is proposed in Section 3, with recommendations on tools or set of tools to be reported in the SEI message. Section 4 summarizes the main points of the proposed updates of the green metadata specification. Section 5 gives information on a software implementation of SEI-messaging in VVC. Finally, Section 6 concludes the contribution.

# Per-tool complexity measurements

## Per-tool complexity measurements set-up

To assess the decoding complexity of VVC in detail, we performed an in-depth analysis as follows. The analysis was based on pre-encoded bitstreams corresponding to the following settings:

- Encoder: VTM11.0

- Configuration: VTM11.0 CTCs [4], random-access (RA) configuration (cfg file encoder_randomaccess_vtm.cfg), with GOP size 32.

- 4 QPs: 22, 27, 32, 37

- 12 HD (1920x1080) clips, with length 100 frames, frame rates of 30, 50 or 60 fps, generated from native UHD content downscaled to HD.

- 12 UHD (3840x2160) clips, with length 100 frames, frame rates of 30, 50 or 60 fps.

- Tested tools

  - Intra: MIP, MRL, LMchroma, ISP, Planar, DC, Directional Horiz/Vertic

  - Transform: MTS, SBT, LFNST, JCCR, DepQuant, TU32 (instead of 64 max)

  - Loop Filter: DBF, SAO, ALF+CCALF, CCALF, LMCS

  - Inter:

    - Motion-coding related: AFFINE, TMVP, SbTMVP, AMVR, MMVD, SMVD

    - Inter Blending: BCW, GPM, CIIP

    - Inter prediction refinement: DMVR, BDOF, PROF

  - Others

    - CTU64 (instead of 128)

    - All tools off (still with CTU 128 and TU 64)

- The following objective metrics between the original clips and the decoded clips were computed: PSNR-Y, PSNR-U, PSNR-V, MS-SSIM, VMAF. PSNR-YUV was also computed as the weighted average of single-component PSNRs ( 1/8 * ( 6*Y-PSNR + U-PSNR + V-PSNR) ).

Then, we measured the decoding time for VTM-13.0 (single-thread) and VVdeC v.1.0.0 (multi-thread) decoding on an Intel i7-8700. For further analysis, we measured the decoding energy as reported in [1] and calculated bitrates, PSNRs, and VMAF values for further interpretation.

## Results of per-tool complexity measurements

The results of the complexity assessment for VTM and VVdeC on the HD sequences are summarized in Table 1 and Table 2, respectively. For VVdeC, the results for the TU32 sequence are missing because it was not decodable by the current VVdeC version.

|  | BDR | BDDE | BDDT | BDRY | BDRU | BDRV | BDRVMAF | BDDEVMAF | RelDecEnergy | RelDecTime |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALLOFF | 37,38 | -37,90 | -37,31 | 29,79 | 75,23 | 139,36 | 28,46 | -40,79 | 58,84 | 59,53 |
| NODBF | 0,73 | -10,33 | -9,75 | 0,87 | -0,04 | -0,04 | 0,91 | -10,64 | 89,53 | 90,12 |
| NOALF | 5,79 | -5,67 | -5,15 | 2,61 | 19,44 | 39,02 | 5,67 | -5,81 | 92,92 | 93,53 |
| NOAffine | 2,40 | -1,36 | -0,18 | 2,52 | 1,73 | 2,47 | 2,58 | -1,41 | 98,69 | 99,99 |
| ALL | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 100,00 | 100,00 |
| NODMVR | 0,78 | -1,34 | 0,10 | 0,73 | 0,90 | 1,27 | 1,04 | -1,50 | 98,70 | 100,26 |
| NOISP | 0,34 | -0,48 | 0,65 | 0,34 | 0,08 | 0,87 | 0,45 | -0,68 | 99,45 | 100,56 |
| NOTMVP | 1,18 | -0,71 | 1,22 | 1,24 | 0,84 | 0,97 | 1,38 | -0,90 | 99,03 | 101,03 |
| MTS2 | 0,49 | 0,67 | 0,97 | 0,28 | 1,73 | 0,03 | 0,32 | 0,32 | 100,54 | 101,04 |
| NOBIO | 0,79 | -0,17 | 1,26 | 0,87 | 0,44 | 0,34 | 1,20 | -0,60 | 99,64 | 101,09 |
| NOLMCS | 0,22 | 0,11 | 1,80 | -0,04 | 1,71 | 1,76 | -1,20 | -0,22 | 99,54 | 101,18 |
| NOSAO | 0,19 | -0,40 | 1,05 | 0,11 | 0,29 | 2,80 | 0,16 | -0,17 | 99,51 | 101,18 |
| NOLFNST | 0,57 | -0,37 | 1,21 | 0,63 | 0,24 | 0,53 | 0,94 | -0,19 | 99,60 | 101,19 |
| MTS3 | -0,16 | 0,37 | 0,95 | -0,16 | -0,22 | -1,13 | -0,02 | 0,26 | 100,53 | 101,20 |
| NOPROF | 0,30 | -0,54 | 1,20 | 0,36 | -0,06 | 0,13 | 0,39 | -0,65 | 99,37 | 101,20 |
| NOJointCbCr | 0,23 | -2,05 | 1,41 | 0,33 | -0,78 | 0,56 | 0,36 | -2,33 | 97,85 | 101,40 |
| NOSBT | 0,46 | -0,31 | 1,59 | 0,54 | -0,06 | 0,46 | 0,44 | -0,38 | 99,57 | 101,50 |
| NOIMV | 1,19 | 0,81 | 1,65 | 0,94 | 2,06 | 4,93 | 1,00 | 0,54 | 100,71 | 101,58 |
| NOGPM | 0,74 | 0,75 | 1,71 | 0,70 | 0,71 | 1,15 | 0,64 | 0,64 | 100,63 | 101,69 |
| NOSbTMVP | 0,38 | 0,17 | 1,85 | 0,41 | 0,16 | 0,11 | 0,58 | 0,20 | 100,10 | 101,90 |
| NOLMChroma | 3,86 | 0,83 | 2,44 | 1,17 | 17,72 | 22,58 | 1,34 | 0,39 | 100,32 | 101,96 |
| ALLLP | 29,01 | 3,92 | 7,94 | 23,82 | 62,51 | 72,52 | 23,65 | 0,70 | 97,99 | 102,00 |
| NOMIP | 0,27 | -0,55 | 1,84 | 0,38 | -0,37 | -1,42 | 0,37 | -0,69 | 99,71 | 102,01 |
| NOMTS | 0,71 | 0,47 | 2,12 | 0,88 | -0,14 | -1,41 | 0,92 | 0,84 | 100,30 | 102,05 |
| NOCCALF | 2,61 | 1,47 | 2,89 | -0,14 | 14,68 | 30,55 | 1,36 | 1,41 | 100,81 | 102,22 |
| ALLLB | 24,13 | 4,80 | 8,22 | 19,57 | 53,73 | 58,57 | 19,43 | 1,71 | 99,08 | 102,49 |
| NOMRL | 0,48 | 0,95 | 2,88 | 0,52 | 0,06 | 0,66 | 0,31 | 0,86 | 100,63 | 102,60 |
| CTU64 | 2,32 | 2,31 | 2,55 | 1,98 | 3,53 | 5,62 | 2,10 | 2,39 | 102,18 | 102,63 |
| NOCIIP | 0,26 | 1,37 | 2,78 | 0,32 | -0,12 | 0,12 | 0,55 | 1,37 | 101,46 | 102,81 |
| NOMMVD | 0,40 | 1,11 | 2,95 | 0,42 | 0,18 | 0,81 | 0,31 | 1,02 | 101,12 | 103,02 |
| NOSMVD | 0,21 | 1,22 | 2,99 | 0,19 | 0,19 | 0,75 | 0,22 | 1,23 | 101,18 | 103,05 |
| noPLANAR | -0,12 | 0,86 | 3,00 | -0,14 | -0,16 | 0,16 | -0,08 | 0,90 | 100,93 | 103,17 |
| noDC | 0,05 | 0,54 | 3,16 | 0,06 | 0,00 | 0,05 | 0,12 | 0,58 | 100,64 | 103,23 |
| noHV | 0,23 | 0,68 | 3,38 | 0,22 | 0,09 | 0,74 | 0,28 | 0,71 | 100,53 | 103,26 |
| NODepQuant | 2,02 | 1,98 | 3,17 | 1,88 | 2,76 | 3,10 | 2,35 | 1,58 | 102,23 | 103,53 |
| NOBCW | 0,96 | 3,43 | 4,60 | 0,80 | 1,78 | 1,93 | 0,33 | 2,76 | 103,03 | 104,26 |
| TU32 | 1,10 | 3,38 | 5,29 | 0,80 | 1,86 | 4,45 | 1,04 | 3,82 | 103,40 | 105,30 |

Table 1. Complexity Assessment of VTM. The table is sorted by the relative decoding time.

|  | BDR | BDDE | BDDT | BDRY | BDRU | BDRV | BDRVMAF | BDDEVMAF | RelDecEnergy | RelDecTime |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALLOFF | 37,38 | -32,54 | -25,17 | 29,79 | 75,23 | 139,36 | 28,46 | -34,76 | 64,44 | 71,65 |
| NOALF | 5,79 | -11,52 | -10,84 | 2,61 | 19,44 | 39,02 | 5,67 | -12,33 | 87,15 | 87,73 |
| NODBF | 0,73 | -5,84 | -3,33 | 0,87 | -0,04 | -0,04 | 0,91 | -5,77 | 94,12 | 96,60 |
| ALLLP | 29,01 | 0,84 | 2,82 | 23,82 | 62,51 | 72,52 | 23,65 | -1,83 | 95,47 | 97,76 |
| NODMVR | 0,78 | -1,87 | -0,82 | 0,73 | 0,90 | 1,27 | 1,04 | -2,32 | 98,18 | 99,20 |
| ALLLB | 24,13 | 2,23 | 4,17 | 19,57 | 53,73 | 58,57 | 19,43 | -0,15 | 97,24 | 99,26 |
| MTS2 | 0,49 | -0,50 | -0,26 | 0,28 | 1,73 | 0,03 | 0,32 | -0,61 | 99,53 | 99,65 |
| ALL | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 100,00 | 100,00 |
| TU32 | 1,10 | 0,12 | 0,11 | 0,80 | 1,86 | 4,45 | 1,04 | 0,10 | 100,00 | 100,00 |
| NOSAO | 0,19 | -0,36 | 0,06 | 0,11 | 0,29 | 2,80 | 0,16 | 0,13 | 99,68 | 100,01 |
| NOAffine | 2,40 | -0,93 | 0,12 | 2,52 | 1,73 | 2,47 | 2,58 | -1,04 | 99,07 | 100,07 |
| NOLMCS | 0,22 | 0,44 | 0,87 | -0,04 | 1,71 | 1,76 | -1,20 | 0,22 | 99,70 | 100,28 |
| MTS3 | -0,16 | 0,10 | 0,37 | -0,16 | -0,22 | -1,13 | -0,02 | 0,20 | 100,28 | 100,52 |
| NOPROF | 0,30 | 0,42 | 0,71 | 0,36 | -0,06 | 0,13 | 0,39 | 0,36 | 100,50 | 100,71 |
| NOLMChroma | 3,86 | 0,41 | 1,44 | 1,17 | 17,72 | 22,58 | 1,34 | -0,02 | 99,89 | 100,83 |
| NOMIP | 0,27 | 0,59 | 0,91 | 0,38 | -0,37 | -1,42 | 0,37 | 0,67 | 100,68 | 100,95 |
| NOLFNST | 0,57 | 0,32 | 1,09 | 0,63 | 0,24 | 0,53 | 0,94 | 0,38 | 100,39 | 101,14 |
| NOMTS | 0,71 | 0,93 | 1,27 | 0,88 | -0,14 | -1,41 | 0,92 | 1,03 | 100,89 | 101,24 |
| NOTMVP | 1,18 | -0,07 | 1,32 | 1,24 | 0,84 | 0,97 | 1,38 | -0,01 | 100,00 | 101,30 |
| NOBIO | 0,79 | -0,49 | 1,37 | 0,87 | 0,44 | 0,34 | 1,20 | -0,81 | 99,41 | 101,32 |
| NOMRL | 0,48 | 1,08 | 1,49 | 0,52 | 0,06 | 0,66 | 0,31 | 1,00 | 100,97 | 101,41 |
| NOSBT | 0,46 | 0,80 | 1,90 | 0,54 | -0,06 | 0,46 | 0,44 | 0,79 | 100,82 | 101,78 |
| NOCIIP | 0,26 | 0,92 | 1,82 | 0,32 | -0,12 | 0,12 | 0,55 | 0,79 | 100,99 | 101,80 |
| NOMMVD | 0,40 | 1,40 | 2,11 | 0,42 | 0,18 | 0,81 | 0,31 | 1,17 | 101,43 | 102,06 |
| NOSbTMVP | 0,38 | 0,37 | 2,07 | 0,41 | 0,16 | 0,11 | 0,58 | 0,60 | 100,55 | 102,08 |
| NOISP | 0,34 | 0,84 | 2,16 | 0,34 | 0,08 | 0,87 | 0,45 | 0,97 | 100,91 | 102,13 |
| NOCCALF | 2,61 | 1,66 | 2,80 | -0,14 | 14,68 | 30,55 | 1,36 | 1,41 | 101,17 | 102,23 |
| NOSMVD | 0,21 | 1,66 | 2,52 | 0,19 | 0,19 | 0,75 | 0,22 | 1,46 | 101,66 | 102,49 |
| NOGPM | 0,74 | 1,94 | 2,72 | 0,70 | 0,71 | 1,15 | 0,64 | 1,79 | 101,90 | 102,73 |
| NOJointCbCr | 0,23 | 1,48 | 2,91 | 0,33 | -0,78 | 0,56 | 0,36 | 1,54 | 101,60 | 102,95 |
| NOBCW | 0,96 | 2,52 | 3,65 | 0,80 | 1,78 | 1,93 | 0,33 | 2,24 | 102,32 | 103,41 |
| noHV | 0,23 | 1,68 | 3,85 | 0,22 | 0,09 | 0,74 | 0,28 | 1,78 | 101,77 | 103,84 |
| noPLANAR | -0,12 | 1,58 | 3,80 | -0,14 | -0,16 | 0,16 | -0,08 | 1,53 | 101,59 | 103,86 |
| NODepQuant | 2,02 | 2,90 | 3,67 | 1,88 | 2,76 | 3,10 | 2,35 | 2,73 | 103,26 | 103,92 |
| noDC | 0,05 | 2,14 | 3,91 | 0,06 | 0,00 | 0,05 | 0,12 | 2,19 | 102,12 | 103,92 |
| NOIMV | 1,19 | 2,92 | 4,11 | 0,94 | 2,06 | 4,93 | 1,00 | 2,76 | 102,83 | 103,98 |
| CTU64 | 2,32 | 7,37 | 9,64 | 1,98 | 3,53 | 5,62 | 2,10 | 8,31 | 107,59 | 109,83 |

Table 2. Complexity Assessment of VVdeC. The table is sorted by the relative decoding time.

The reported values have the following meaning:

- BDR: Average BD-rate increase [3] when the tool is switched off (for YUV-PSNR) in percent.

- BDRY: Average BD-rate increase [3] when the tool is switched off (for Y-PSNR) in percent.

- BDRU: Average BD-rate increase [3] when the tool is switched off (for U-PSNR) in percent.

- BDRV: Average BD-rate increase [3] when the tool is switched off (for V-PSNR) in percent.

- BDRVMAF: Average BD-rate increase [3] when the tool is switched off (for VMAF-score) in percent.

- BDDE: Average BD-decoding energy increase when the tool is switched off. For the calculation of the BD-energy, the rate is replaced by the measured energy in percent.

- BDDT: Average BD-decoding time increase when the tool is switched off. For the calculation of the BD-time, the rate is replaced by the measured decoding time in percent.

- RelDecTime: Average decoding time change at the same QP in percent.

- RelDecEnergy: Average decoding time change at the same QP in percent.

The tools are sorted by the impact on the relative decoding time in ascending order. We can see that disabling all tools leads to highest decoding time savings. Furthermore, a high impact can be observed for the in-loop filters DBF and ALF. Finally, many tools increase the decoding time when switched off. We claim that the reason for an increased complexity is that on average, the complexity increase due to a higher number of residual coefficients and smaller block sizes is larger than the complexity decrease caused by disabling a computationally complex tool. In general, preliminary results for UHD sequences are similar and will be reported as soon as available.

Restricting the analysis to tools that have an influence on decoding complexity of at least 3 percent (VVdeC), we can see that in-loop filters, the block size, classic intra prediction modes (DC, HV, and planar), BCW, dependent quantization, and IMV show the highest influence on the decoding complexity. In general, similar tendencies can be observed for VTM decoding.

# Results from Complexity Modeling

## Complexity model

The results presented above provide a general overview on the global impact of certain coding tools. Unfortunately, they do not provide information on the complexity of a single execution of a coding tool, which might be important during decoder runtime. Hence, we train a complexity model with the measured data in order to estimate the true complexity of a single execution of a tool. The complexity model can generally be written as

Cdecode=tntct,

where Cdecode is the complexity of the decoding process in, e.g., decoding time, t is the index of a tool, nt is the frequency of occurrence of the t-th tool, and ctthe estimated complexity for a single execution of the t-th tool. In this representation, it is assumed that the overall complexity of decoding is the linear sum over the complexities related to each tool. A similar model was proposed for the decoding energy in [6] and [7] and was successfully used for reducing the processing energy on the decoder side by extending RDO in the encoder [8].

## Analysis of complexity modeling results

To determine the frequencies of occurrence nt, we use the bit stream analysis tool from [5] and count the occurrence of all supported tools in the complete data set including tool on/off bit streams. Then, we train the model using the counted tools and the measured decoding time as the complexity metric Cdecode and obtain specific decoding-time values ct for each tool. To mitigate the effect of overfitting, we reduce the number of tools by the largest possible extent  while keeping a low average estimation error.

From this analysis, we find that we can reach a mean absolute estimation error of 4.92% for VVdeC-decoding when choosing the following tools or bit stream features for modeling:

- Offset complexity

- Number of non-zero transform blocks (4 pels, 8 pels, and 16 pels, weighted by the number of pels)

- Number of non-zero transform blocks (32 pels, 64 pels, and 128 pels, weighted by the number of pels)

- Number of non-zero transform blocks (256 pels, 512 pels, and 1024 pels, weighted by the number of pels)

- Number of non-zero transform blocks (2048 pels, and 4096 pels, weighted by the number of pels)

- Number of non-zero transform coefficients

- Blocks predicted by intra-prediction modes

- Blocks predicted by bi-prediction or GPM

- Instances of the DBF

- Luma and Chroma-Instances of the SAO filter (a luma instance is weighted by 4)

- ALF instances (a luma instance is weighted by 4)

Further tools have been tested (e.g., fractional pel filtering, JCCR, LFNST, separate or merged block sizes) but did not lead to an increased estimation accuracy. Note that these results were mainly obtained for bit streams encoded with the random-access configuration such that these results might be insufficient for sequences encoded with the all-intra configuration. Respective measurements will be performed in the future.

# Proposed updates of the green metadata specification

## Proposed Syntax Elements for Decoder Power Reduction

The proposed syntax elements describe the portion of a certain tool in a predefined time interval, where the portion is related to the luma and chroma samples number to which the tool is applied. The interval is defined at the beginning of the SEI message and can span a single frame, a slice, a subpicture or a tile, or a time interval with a certain number of frames.

From the analysis shown above, we propose to specify the syntax elements in SEI messaging shown in Table 3.

Table 3 Proposed syntax elements for signaling of decoder complexity metrics.

portion_non_zero_4_8_16_blocks_area

portion_non_zero_32_64_128_blocks_area

portion_non_zero_256_512_1024_blocks_area

portion_non_zero_2048_4096_blocks_area

portion_non_zero_transform_coefficients_area

portion_intra_predicted_blocks_area

portion_bi_or_gpm_predicted_blocks_area

portion_deblocking_instances

portion_alf_blocks

portion_sao_blocks

In addition, for the intra-only case, we have currently selected the following syntax elements. For the next meeting cycle, it is planned to perform more analysis for other configurations than random-access.

Table 3 Proposed syntax elements for signaling of decoder complexity metrics for intra-only case.

portion_planar_blocks_in_intra_area

portion_dc_blocks_in_intra_area

portion_angular_hv_blocks_in_intra_area

portion_mip_blocks_in_intra_area

## Syntax for Quality Recovery after low-power Encoding

For the syntax of quality recovery after low-power encoding, we propose to follow the approach signalling the overall PSNR and add the overall wPSNR, WS-PSNR, SSIM and VMAF-score of a segment. The specification is shown in Section 8 of the Green metadata specification (see attachment).

## Detailed syntax and semantics

Green metadata specification

The detailed syntax and semantics of the new SEI message are specified in section 5 of the attached document “ISO_IEC_23001-11_2018(E)-FDIS2_Amend_VVC.docx”. Additional changes are made in a few other parts of the specification.

VSEI specification

In addition, changes to the VSEI specification are also proposed and attached to the contribution in document “JVET-V2006-v1-greenMetadataAdded.docx” (in track change mode).

VVC specification

A reference must also be added in the section D.2.1 of the VVC specification, as highlighted in yellow in the table excerpt below.

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| else if( payloadType = = 45 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 56 ) |  |
| green_metadata( payloadsize ) /* specified in ISO/IEC 23001-11 */ |  |
| else if( payloadType = = 129 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| parameter_sets_inclusion_indication( payloadSize ) |  |
| … |  |
| } |  |

# Software Implementation of SEI-messaging in VVC

A software tool counting most of the coding tools from VVC is available at https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer. The software counts the tool usage (a.k.a. features) during encoder and decoder runtime. The counted values can directly be used for SEI messaging. For future work, it is planned to also implement the SEI messaging framework into the encoder and decoder software for further referencing and further use.

# Concluding Remarks

This document proposes an initial draft for new Green metadata SEI message for VVC. The SEI message defines syntax elements enabling to ease quality recovery after low power encoding and to assess decoding complexity of the various tools or set of tools involved in VVC bitstreams decoding. They are specified based on a detailed analysis of VVC decoding, using a large set of content, bitstreams and using two software decoders, VTM-13.0 and VVdeC. A complexity model has been used to identify the most impactful tools or set of tools.

Two draft updated specification documents are provided in attachment:

- VSEI amendment specification update: JVET-V2006-v1-greenMetadataAdded.docx

- Green Metadata specification update: ISO_IEC_23001-11_2018(E)-FDIS2_Amend_VVC.docx.

The planed next steps consist of:

- Completing the decoding complexity analysis with alternate software decoders implementations

- Completing the decoding complexity analysis with additional bitstreams corresponding to All Intra and Low Delay configurations

- Implementing the SEI message in the VTM analyzer

- Consolidating the proposed specification

Acknowledgments: this work has been partially achieved in the context of the 3EMS project funded by the “Region Bretagne”.

# References

[1] M. Kränzler, C. Herglotz, A. Kaup, “Decoding Time and Energy Assessment of VTM-10.0 and VVdeC”, JVET-T0068, October 2019.

[2] Information technology - MPEG Systems Technologies - Part 11: Energy-Efficient Media Consumption (Green Metadata) ISO/IEC JTC1/SC29/WG11 Final Draft International Standard 23001-11, Oct. 2014.

[3] G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, VCEG-M33, ITU-T SG 16/Q 6, Austin, TX, Apr. 2001.

[4] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010-v1, Mar. 2019.

[5] C. Herglotz, M. Kränzler, A. Kaup, “Bit Stream Analyzer for Coding Tool Statistics”, JVET-P0085, October 2019.

[6] M. Kränzler, C. Herglotz, A. Kaup, “Modeling of the Decoding Energy for VTM-7.0 with a Bit Stream Feature-Based Model”, JVET-Q0052, January 2020.

[7] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack, and A. Kaup, „Modeling the energy consumption of the HEVC decoding process”, IEEE Transactions on Circuits and Systems for Video Technology, 28:217–229, Jan 2018.

[8] C. Herglotz, A. Heindel, and A. Kaup. Decoding-energy-rate-distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, 29:171–182, Nov 2017.

# Patent rights declarations

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The Friedrich-Alexander-Universität Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.

INSA does not have any current or pending patent rights relating to the technology described in this contribution.

Harmonic Inc does not have any current or pending patent rights relating to the technology described in this contribution

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

______________________