| # Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0092 |
| --- | --- |

| Title: | On constraining inverse transform precision for high bit depth and high bitrate video coding. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JEVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Louie Kerofsky Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | kerofsky@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

At the 22nd JVET meeting extended precision mode was adopted to VVCv2 draft specification for high bit depth and high bitrate video coding. In this mode, the precision of the inverse transform is extended to 19 bits for coding of 12 bits video and to 21 bits for coding of 16 bits video. This contribution proposes to employ Content Adaptive Transform Precision (CATP) method to constrain the bit depth of inverse transform implementation while preserving coding performance of the Anchor (VTM13.0, extended precision ON). The method proposed here is a modification of the CATP of JVET-W0050 (CE2.2) and targets complexity reduction and constraining precision used in the inverse transform.

Method has been tested under HBD / HBR CTC with 16-bit, 17-bit and 18-bit constraints vs. the Anchor. No run time increase was reported for tested configurations, no noticeable penalty is reported for Normal QP configuration. Summary of the results for Low QP are following:

Test on 16 bit constraints:

| Low QP | HDR PQ12 |  |  | HDR HLG12 |  |  | SVT12 | SVT16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Y | U | V | Aver.GBR | Aver.GBR |
| AI | -0.01% | -0.01% | -0.02% | 0.00% | -0.01% | -0.01% | 0.03% | 0.28% |
| LDB | -0.02% | 0.01% | 0.00% | 0.00% | -0.02% | -0.02% | 0.01% | 0.14% |
| RA | 0.00% | -0.01% | 0.00% | 0.00% | -0.01% | 0.00% | 0.01% | 0.15% |

Test on 17 bit constraints:

| Low QP | HDR PQ12 |  |  | HDR HLG12 |  |  | SVT12 | SVT16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Y | U | V | Aver.GBR | Aver.GBR |
| AI | -0.01% | -0.01% | -0.03% | -0.01% | -0.01% | -0.01% | 0.01% | 0.10% |
| LDB | 0.00% | -0.01% | 0.01% | 0.00% | -0.01% | -0.01% | 0.00% | 0.03% |
| RA | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% | 0.04% |

Test on 18 bit constraints:

| Low QP | HDR PQ12 |  |  | HDR HLG12 |  |  | SVT12 | SVT16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Y | U | V | Aver.GBR | Aver.GBR |
| AI | -0.01% | -0.03% | -0.03% | 0.00% | -0.01% | -0.01% | 0.00% | 0.02% |
| LDB | -0.01% | -0.01% | 0.01% | -0.01% | -0.02% | 0.00% | 0.00% | -0.01% |
| RA | 0.00% | -0.01% | 0.01% | -0.01% | -0.01% | 0.00% | 0.00% | 0.00% |

It is proposed to adopt proposed CATP method and constrained bit depth implementation of the inverse transform to VVCv2.

# Introduction

The method of Content Adaptive Transform Precision (CATP) was introduced as a coding tool in [1]. To summarize CATP, the operation of the dequantization and inverse transform of VVC includes three shifts as shown in Figure 1:

Figure 1 Block diagram of the dequantization and inverse transform processing (JVET-0133)

According to VVC specification, shift1 is computed as:

bdShift = BitDepth + rectNonTsFlag +		
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + sh_dep_quant_used_flag

And Shift2 is fixed to 7:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )

And Shift 3 is :

bdShift = ( nTbH > 1  &&  nTbW > 1 ) ? ( 20 − BitDepth ) : ( 21 − BitDepth )

The method of Content Adaptive Transform Precision (CATP) proposed in [1] targets to adapt shifts of the inverse transform (dequantization shift, mid-transform shift and normalization shift) to provide additional accuracy of calculations as shown in Figure 2.

Figure 2 Block diagram of the CATP method of JVET-0133.

The method of JVET-V0113 operates on a block of quantized coefficient levels initially dequantizing to an analysis buffer before determining the values needed for the three shift values from the largest coefficient magnitude M of the block. Repeating equations from JVET-0133 we have.

ShiftDequant=ceillog2M-1-TrDynamicRange

ShiftMid=Shift2+Shift1-ShiftDequant

ShiftMid=ceillog2B-TrDynamicRange+Shift2

ShiftNormalization=Shift1-ShiftDequant+Shift2-ShiftMid+Shift3

Where the values of shift1, shift2, and shift3 are those used in VVC at the corresponding stages and the variable TrDynamicRange is set equal to Log2TransformRange derived in clause 7.3.4.5.

Log2TransformRange = ExtendedPrecisionFlag ? Max(15, Min(20, BitDepth + 6)) : 15

In this derivation, B is the maximum coefficient after first inverse transform (before the downshift) is computed.  In practice this value B should be estimated prior to computing the first inverse transform.  In SW corresponding to JVET-0133, this value of B is replaced by a bound computed prior to the inverse transform calculation where the bound depends on the transform kernel and the residual data.  The bound B is estimated using the inequality below and bounding the output of a transform by the maximum column sums of absolute value of the transform kernel and the maximum of the transform coefficients.  The sum of transform kernel absolute values can be precomputed while the maximum transform coefficient is selected based on the content.

maxi,jMx=maxi,jk=0N-1Mi,k∙xk,j≤maxik=0N-1Mi,k∙maxk,jxk,j

# Proposed Method

In this contribution we further develop the CATP method proposed in [1]. We propose to modify the CATP method to reduce the complexity of shift derivation and to constrain the bit depth of inverse transform implementation while the entropy coding is unmodified from [2] where extended precision is used to improve coding efficiency.

A target bit depth (denoted by TDR variable in this document) of inverse transform implementation, is used to deriver shifts in the dequantization and inverse transform calculations and to determine the maximum and minimum values used for clipping specified in clause 7.4.12.11. This clipping to the target precision ensures the dynamic range of the reconstruction process does not exceed the dynamical range specified by TBR.

The processing stages of proposed method and effective bit-depths are illustrated in Figure 3. A block of quantized levels is input and then processed by dequantization, LFNST if enabled, vertical and horizontal inverse transforms to produce a residual signal.

Figure 3 Proposed CATP method. Term TDR notes the Target Bit Depth parameter.

## Derivation of dequantization and normalization shifts

The dequantization shift ShiftDequant is computed similarly to the CATP of [1] with adjustment to the target bit depth specified by the TDR instead of Log2TransformRange.

ShiftDequant=ceillog2M-1-TDR

Where M is the maximum magnitude of dequantized coefficient derived for a current TU and variable TDR denotes target bit depth of inverse transform implementation. The maximum magnitude coefficient can be computed from the quantized coefficient data without usage of additional buffer.

The mid-transform shift derivation in our proposal is simplified comparing to the CATP of [1]. Simplification is achieved by introducing a transform size dependency, instead of the content dependent derivation of the method in [1].

ShiftMid=7+ceillog2H

Where H is the height of the transform kernel.

It can be shown with this shift, a transform with coefficients of magnitude at most M increases the dynamic range by at most M+log2(H) bits, below.  With this value of mid transform shift, the first inverse transform calculation will preserve dynamic range.

maxi,jMx≤maxi,jk=0H-1Mi,k∙xk,j≤k=0H-1maxi,kMi,k∙maxk,jxk,j

maxi,jMx≤maxi,kMi,k∙maxk,jxk,j∙k=0H-11=maxi,kMi,k∙H∙maxk,jxk,j

Normalization shift are derived identically to what is proposed in CATP [1]. The change in dequantization and mid transform shifts between the proposal and current VTM is used to modify the normalization shift.

ShiftNormalization=Shift1-ShiftDequant+Shift2-ShiftMid+Shift3

The values Shift1, Shift2, and Shift3 are those used in the VTM with extended precision enabled.  Specifically,

Shift2=7

Shift3= ( 5 + maxLog2TrDynamicRange − BitDepth )

## Clipping

Relevant to this proposal are clipping in dequantization, LFNST, and first inverse transform stage. Current extended precision design computes the maximum and minimum coefficient values based on the Log2TransformRange value as specified in Clause 7.4.12.11.

In the proposed design, the clipping range is calculated based on the target bitdepth of inverse transform implementation denoted by TDR, instead of Log2TransformRange.

CoeffMin = −( 1  <<   TBR )		(189)

CoeffMax = ( 1  <<   TBR ) − 1		(190)

We note that the dequantization and mid transform shifts are chosen such that intermediate values will not be modified by clipping.

The inclusion of clipping in the process minimizes the changes relative to VVCr1 and clearly indicates the dynamic range at various intermediate points does not exceed the target dynamic range.

## Entropy coding

The entropy coding is unmodified relative to the high bit-depth design which uses extended precision [2].

## LFNST:

LFNST usage is not restricted and LFNST implementation is not modified comparing to the VVCv2 specification, expect of imposing clipping limits specified by the TDR rather than Log2TransformRange .

# Results

Proposed CATP method was implemented on top of VTM13.0 and tested under HBD / HBR CTC of JVET-T2018 in LowQP and Normal QP test configurations and tested with different bit depth of inverse transform implementation:

- CATP16: CATP with 16 bits constraint on inverse transform implementation (TDR=15).

- CATP17: CATP with 17 bits constraint on inverse transform implementation (TDR=16).

- CATP18: CATP with 18 bits constraint on inverse transform implementation (TDR=17).

## Results of CATP16:

Test 3.1 Test CATP16 vs. VTM13.0 anchor, extended precision is ON, Low QP.

| Low QP |  | HDR PQ12 |  |  |  |  |  | HDR HLG12 |  |  |  |  |  | SVT12 |  | SVT16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY |  | wU |  | wV |  | Y |  | U |  | V |  | Aver.GBR |  | Aver.GBR |  |
| AI |  | -0.01% |  | -0.01% |  | -0.02% |  | 0.00% |  | -0.01% |  | -0.01% |  | 0.03% |  | 0.28% |  |
| LDB |  | -0.02% |  | 0.01% |  | 0.00% |  | 0.00% |  | -0.02% |  | -0.02% |  | 0.01% |  | 0.14% |  |
| RA |  | 0.00% |  | -0.01% |  | 0.00% |  | 0.00% |  | -0.01% |  | 0.00% |  | 0.01% |  | 0.15% |  |
| Table 3.2. Reported run-time estimates for CATP16 vs. VTM13.0 anchor, Low QP. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | HDR PQ |  |  |  | HDR HLG |  |  |  | SVT12 RGB |  |  |  | SVT16 RGB |  |  |  |
|  |  | Enc |  | Dec |  | Enc |  | Dec |  | Enc |  | Dec |  | Enc |  | Dec |  |
| AI |  | 101% |  | 101% |  | 102% |  | 101% |  | 102% |  | 102% |  | 102% |  | 103% |  |
| LDB |  | 100% |  | 99% |  | 101% |  | 99% |  | 99% |  | 98% |  | 103% |  | 105% |  |
| RA |  | 101% |  | 98% |  | 101% |  | 100% |  | 101% |  | 100% |  | 102% |  | 106% |  |

Table 3.3. Test CATP16 vs. VTM13.0 anchor, Normal QP.

|  | HDR PQ |  |  |  |  | HDR HLG |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | wY | wU | wV | Y | U | V |
| AI | 0.05% | 0.00% | -0.01% | 0.14% | 0.10% | 0.01% | -0.06% | -0.11% |
| RA | 0.17% | -0.04% | -0.02% | -0.02% | 0.17% | 0.00% | -0.15% | 0.22% |

Table 3.4. Run-time estimates for CATP16 vs. VTM13.0 anchor, Normal QP.

|  | HDR PQ |  | HDR HLG |  |
| --- | --- | --- | --- | --- |
|  | Enc | Dec | Enc | Dec |
| AI | 100% | 100% | 101% | 98% |
| RA | 101% | 99% | 102% | 99% |

## Results of CATP17:

Test 3.5 Test CATP17 vs. VTM13.0 anchor, extended precision is ON, Low QP.

| Low QP |  | HDR PQ12 |  |  |  |  |  | HDR HLG12 |  |  |  |  |  | SVT12 |  | SVT16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY |  | wU |  | wV |  | Y |  | U |  | V |  | Aver.GBR |  | Aver.GBR |  |
| AI |  | -0.01% |  | -0.01% |  | -0.03% |  | -0.01% |  | -0.01% |  | -0.01% |  | 0.01% |  | 0.10% |  |
| LDB |  | 0.00% |  | -0.01% |  | 0.01% |  | 0.00% |  | -0.01% |  | -0.01% |  | 0.00% |  | 0.03% |  |
| RA |  | 0.00% |  | -0.01% |  | 0.00% |  | 0.00% |  | 0.00% |  | -0.01% |  | 0.00% |  | 0.04% |  |
| Table 3.6. Reported run-time estimates for CATP17 vs. VTM13.0 anchor, Low QP. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | HDR PQ |  |  |  | HDR HLG |  |  |  | SVT12 RGB |  |  |  | SVT16 RGB |  |  |  |
|  |  | Enc |  | Dec |  | Enc |  | Dec |  | Enc |  | Dec |  | Enc |  | Dec |  |
| AI |  | 100% |  | 99% |  | 101% |  | 100% |  | 101% |  | 102% |  | 102% |  | 101% |  |
| LDB |  | 101% |  | 103% |  | 100% |  | 100% |  | 99% |  | 98% |  | 102% |  | 100% |  |
| RA |  | 101% |  | 99% |  | 101% |  | 100% |  | 100% |  | 100% |  | 101% |  | 102% |  |

## Results of CATP18:

Test 3.7 Test CATP18 vs. VTM13.0 anchor, extended precision is ON, Low QP.

| Low QP |  | HDR PQ12 |  |  |  |  |  | HDR HLG12 |  |  |  |  |  | SVT12 |  | SVT16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY |  | wU |  | wV |  | Y |  | U |  | V |  | Aver.GBR |  | Aver.GBR |  |
| AI |  | -0.01% |  | -0.03% |  | -0.03% |  | 0.00% |  | -0.01% |  | -0.01% |  | 0.00% |  | 0.02% |  |
| LDB |  | -0.01% |  | -0.01% |  | 0.01% |  | -0.01% |  | -0.02% |  | 0.00% |  | 0.00% |  | -0.01% |  |
| RA |  | 0.00% |  | -0.01% |  | 0.01% |  | -0.01% |  | -0.01% |  | 0.00% |  | 0.00% |  | 0.00% |  |
| Table 3.8. Reported run-time estimates for CATP18 vs. VTM13.0 anchor, Low QP. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | HDR PQ |  |  |  | HDR HLG |  |  |  | SVT12 RGB |  |  |  | SVT16 RGB |  |  |  |
|  |  | Enc |  | Dec |  | Enc |  | Dec |  | Enc |  | Dec |  | Enc |  | Dec |  |
| AI |  | 101% |  | 102% |  | 101% |  | 101% |  | 103% |  | 104% |  | 102% |  | 103% |  |
| LDB |  | 103% |  | 105% |  | 102% |  | 100% |  | 98% |  | 97% |  | 101% |  | 99% |  |
| RA |  | 102% |  | 100% |  | 101% |  | 102% |  | 100% |  | 99% |  | 100% |  | 103% |  |

# Conclusions

This document presents CATP method to limit the bit depth needed to reconstruct residual with no coding loss (12 bits video coding) or relatively low coding loss (16 bits video coding) comparing to the anchor in HBD/HBR CTC. No run time increase on average is reported for tested configurations. It is proposed to adopt proposed CATP16 method and constrained bit depth implementation of the inverse transform to VVCv2 to 16-signed bits.

# Reference

[1] K. Naser, Franck Galpin, Fabrice Le Leannec, and Philippe de Lagrange Zhang “AHG8: Content Adaptive Transform Precision for High Bit Depth Coding JVET-V0133, Online, Apr. 2021.

[2] “VVC operation range extensions (Draft 3)”, JVET-V2005, F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, Y.-K. Wang.

[3] A. Browne, T. Ikai, D.  Rusanovskyy, M.Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, JVET, 21th Meeting: by teleconference, 20-28 April. 2021

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).