| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0048-v1 |
| --- | --- |

| Title: | CE: Film Grain Synthesis (test CE2.1 and CE2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Sean McCarthy, Walt Husak, Peng Yin, Taoran Lu, Arjun Arora, Tao Chen Dolby Laboratories, Inc Vijayakumar G R, Jay N Shingala, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam Ittiam Systems | Tel: Email: | {sean.mccarthy, fpu, pyin}@dolby.com |
| Source: | Dolby Laboratories, Inc. and Ittiam Systems |  |  |

_____________________________

# Abstract

This contribution presents test results, complexity analysis, and software for the resolution-adaptive grain block size aspects of the CE on film grain synthesis (FGS) (JVET-W2022). Resolution-adaptive grain block size was studied in CE2.1 and CE2.2. Resolution-adaptive grain block size means that 8x8 grain blocks are used for picture sizes up to 1080p (2K); 16x16 grain blocks for picture sizes greater than 2K and up to 2160p (4K); and 32x32 grain blocks for picture sizes greater than 4K (see JVET-W0095). CE2.1 and CE2.2 differ only in the 2D transform used to pre-compute the grain pattern database from which grain blocks are selected. In CE2.1, the transform specified in SMPTE RDD 5 is used. In CE2.2, the DCT2 specified in VVC is used (see JVET-V0093). CE anchor uses fixed 8x8 grain block size with SMPTE RDD 5 transform.

In complexity tests, FGS processing times were measured using JVET SDR-CTC Class A 4K test sequences (upsampled for 8K tests) following CTC random access (RA) test conditions. FGC processing times were also measured with and without SIMD enabled. FGS processing times were 2.5 to 3 times faster for SIMD enabled compared to SIMD disabled. FGS processing times relative to the anchor are summarized below and indicate that resolution-adaptive grain block size significantly reduces processing times.

| Percent reduction in FGS processing time relative to anchor (8x8 grain blocks) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4K (16x16 grain blocks) |  | 8K (32x32 grain blocks) |  |
|  | SIMD_OFF | SIMD_ON | SIMD_OFF | SIMD_ON |
| CE2.1 (SMPTE RDD 5 transform) | -20.32% | -22.54% | -52.87% | -42.51% |
| CE2.2 (VVC DCT2) | -20.39% | -23.04% | -52.74% | -42.34% |

In subjective tests, similar visual quality was observed for CE2.1 and CE2.2 compared with the anchor for 1080p and 4K test sequences selected from the JVET FTP. For test sequences with grain or noise in the source, decoded pictures with grain blending appeared more similar to the source than decoded pictures without grain blending. For test sequences without grain or noise in the source, grain blending tended to mask compression artefacts (texture loss, basis patterns etc.) when coding at low bitrate.

It is recommended that the software for resolution-adaptive grain block size and the film grain characteristics SEI rewriter be adopted into VTM. Further, it is recommended that the corresponding grain blending process be incorporated into either VSEI or a technical report.

# Introduction

Grain blending parameters can be signalled in the film grain characteristics (FGC) SEI messages specified in VSEI, HEVC, and AVC. FGC SEI parameter values can be used for grain blending, which is a post-decode process to add visual spatial temporal noise to simulate actual film grain or mask compression artefacts such as loss of texture. SMPTE RDD 5 [1] describes a method of grain blending using parameters signaled in an FGC SEI message.

In JVET-W0095 [2], resolution-adaptive grain block size was proposed as a method to make film grain synthesis more implementation friendly for 4K and 8K content. (Only 8x8 grain blocks are specified for use with SMPTE RDD 5.) It was asserted that enabling support for 16x16 grain blocks for 4K content and 32x32 grain blocks for 8K content would maintain the same relative grain block size compared to 8x8 grain blocks for 1080p while reducing the number of operations that would need to be performed per sample.

The CE for film grain synthesis (JVET-W2022 [3]) describes the core experiments to test resolution-adaptive block size. The anchor of the CE is SMPTE RDD 5 with fixed 8x8 grain block size for all resolutions (HD, 4K and 8K). In CE2, resolution-adaptive BlockSize x BlockSize grain block size was tested, where the variable BlockSize is determined as:

PicSizeInLumaSamples = PicHeightInLumaSamples * PicWidthInLumaSamples

if PicSizeInLumaSamples <= (1920 * 1080)
	BlockSize = 8
else if PicSizeInLumaSamples <= (3840 * 2160)
	BlockSize = 16
else
	BlockSize = 32

In addition, two sub-CEs tested different ways of pre-computing the grain database from which grain blocks are selected during grain blending. CE2.1 tested use of the transform specified in SMPTE RDD 5 to pre-compute the grain database. CE2.2 tested use of the DCT2 transform specified in VVC (see JVET-W0093 [4]) to pre-compute the grain database.

# Methodology

## Software

CE test software is available on Git repo: https://vcgit.hhi.fraunhofer.de/jvet-w-ce/ce-fgs/-/tree/CE-FGS (commit SHA:  81c9211c14d240de6c30444121ce2b4e5d33f232).

CE test software is based on VTM-13.2 with extensions to be tested in CE proposed in JVET-V0093 and JVET-W0095.

CE software supports additional functionality as follows:

- Film grain analysis supporting multiple intensity intervals as proposed in JVET-W0072 [5]

- FGC SEI rewriter (described in section 2.1.1) enabling FGC SEI parameter values to be modified in existing bitstreams without re-encoding

### FGC SEI rewriter tool

In order to facilitate the FGS CE experiments, an FGC SEI rewriter tool based on VTM13.2 is developed and is released with the CE software. This tool allows FGC SEI parameter values to be modified in existing bitstreams without re-encoding. It also supports inserting FGC SEI messages in a VVC bitstream originally encoded without FGC SEI message. The bitstream with the inserted FGC SEI has identical MD5sum value to the bitstream having the FGC SEI enabled during encoding. The tool supports three operational modes, controlled by SEIFilmGrainOption: 0-no change; 1-FGC SEI removal; 2-FGC SEI insertion; 3-FGC SEI rewriter.

- FGC SEI removal	(applied on bitstreams with FGC SEI)

- FGC SEI insertion	(applied on bitstreams without FGC SEI)

- FGC SEI modification	(applied on bitstreams with FGC SEI)

The two typical use cases and examples are given below:

- A bitstream (test_fg1.bit) is encoded with a pre-configured FGC SEI (fg1.cfg), but the film grain parameters are not producing satisfactory film grain effects. One can use the tool to modify the values (fg2.cfg) of the existing FGC SEI. Example command line: 
./SEIFilmGrainAppStatic -c fg2.cfg -b test_fg1.bit -o test_fg2.bit --SEIFilmGrainOption=3

- A bitstream (test.bit) is encoded without FGC SEI so it cannot take advantage of benefit of the film grain synthesis. One can use the tool to easily insert the FGC SEI (fg.cfg) into the bitstream. Example command line:

./SEIFilmGrainAppStatic -c fg.cfg -b test.bit -o test_fg.bit --SEIFilmGrainOption=2

The tool manipulates the VVC bitstream at NALU level, so the processing time is extremely fast (e.g., to rewrite FGC SEI in a 4K/8K bitstream, the entire processing time is less than 0.1 sec). The basic workflow is illustrated in Figure 1.

Figure 1 Basic workflow of FGC SEI rewriter

## Complexity Test

CE complexity tests using 4K content were conducted with the JVET SDR-CTC [6] Class A test sequences. For tests using 8K content, JVET SDR-CTC Class A test sequences were upsampled to 8K using HDRTools-v0.19.1. All tests were conducted following CTC random access (RA) test conditions [6].

CE tests were conducted for both SIMD enabled and SIMD disabled.

The three combinations of FGC SEI parameter values listed in Table 1 were tested to investigate the impact of number of intensity intervals and number of colour components on processing times for grain blending.

Table 1 Film grain configuration settings for complexity test

| Film grain configuration | # of intensity intervals | Components |
| --- | --- | --- |
| FGC_SEI_I10C3 | 10 | Luma & Chroma |
| FGC_SEI_I10C1 | 10 | Luma only |
| FGC_SEI_I1C1 | 1 | Luma only |

## Subjective Test

For subjective tests, 6 test sequences with 1080p or 4K resolution (listed in Table 2) were selected from JVET FTP: ftp://ftp.ient.rwth-aachen.de. 4K test sequences were upsampled to 8K using HDRTools-v0.19.1. Random access test conditions were used with QP32 and QP37. Film grain configurations were adjusted for each clip. Decoded YUV files with and without FGS were viewed on a BenQ pro-level SW271C 27” 4K monitor.

Table 2 Test sequences (4K and 1080p) used in subjective test

| SeqName | Category | Resolution | FPS | Frames | BitDepth | MD5 of Source YUV |
| --- | --- | --- | --- | --- | --- | --- |
| MountainBay2 | Clean | 3840x2160 | 30 | 300 | 10 | f27b6b70244fb083baac546958fcf696 |
| OberbaumSpree | Clean | 3840x2160 | 60 | 600 | 10 | 6975b81c9e63c92b3bf4223796102da1 |
| Tango2 | Clean | 3840x2160 | 60 | 294 | 10 | 0471a59c423b7059c5c6c8b395e864a9 |
| RaceNight | Noisy | 3840x2160 | 50 | 600 | 10 | c06ed896df6e7dd90b4164df39652d50 |
| TrafficFlow | Noisy | 3840x2160 | 30 | 300 | 10 | 63135fc6fd7c73675f4ec1211b11dc93 |
| Crowdrun | Noisy | 1920x1080 | 50 | 500 | 8 | da34812b5b2c316d40481c7b6c841e41 |

# Results and Analysis

## Results for FGS processing times

FGS processing times are provided for CE2.1 in Table 3 and for CE2.2 in Table 4. Absolute FGS processing time is measured with YUV output disabled. For each decoded bitstream, FGS processing time was calculated over all frames independently from decoding time. The reduction in FGS processing time is determined as follows:

Reduction in FGS processing time = (FT_CE2.x – FT_CEAnc ) / FT_CEAnc

where FT_CEAnc represents the FGS processing time for the anchor and FT_CE2.x represents the FGS processing time for CE2.1 (x = 1) and CE2.2 (x = 2).

Table 3 Average reduction in FGC processing time for CE2.1

|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Configure | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) |
| CTC4K | FGC_SEI_I10C3 | 9.99 | 7.97 | -20.16% | 3.45 | 2.70 | -21.77% |
|  | FGC_SEI_I10C1 | 6.46 | 5.13 | -20.54% | 2.21 | 1.70 | -22.92% |
|  | FGC_SEI_I1C1 | 6.86 | 5.47 | -20.25% | 2.31 | 1.78 | -22.92% |
|  | Average | 7.62 | 6.07 | -20.32% | 2.60 | 2.01 | -22.54% |
|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| Content | Configure | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) |
| CTC8K | FGC_SEI_I10C3 | 39.78 | 17.41 | -56.25% | 13.33 | 7.47 | -43.98% |
|  | FGC_SEI_I10C1 | 25.83 | 12.88 | -50.14% | 8.61 | 5.17 | -40.04% |
|  | FGC_SEI_I1C1 | 27.39 | 13.15 | -52.01% | 9.09 | 5.14 | -43.43% |
|  | Average | 30.42 | 14.34 | -52.87% | 10.14 | 5.83 | -42.51% |

Table 4 Average reduction in FGC processing time for CE2.2

|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Configure | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) |
| CTC4K | FGC_SEI_I10C3 | 9.99 | 7.94 | -20.46% | 3.45 | 2.68 | -22.15% |
|  | FGC_SEI_I10C1 | 6.46 | 5.14 | -20.45% | 2.21 | 1.70 | -22.93% |
|  | FGC_SEI_I1C1 | 6.86 | 5.47 | -20.26% | 2.31 | 1.76 | -24.01% |
|  | Average | 7.62 | 6.07 | -20.39% | 2.60 | 2.00 | -23.04% |
|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| Content | Configure | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) |
| CTC8K | FGC_SEI_I10C3 | 39.78 | 17.27 | -56.60% | 13.33 | 7.38 | -44.62% |
|  | FGC_SEI_I10C1 | 25.83 | 13.10 | -49.29% | 8.61 | 5.27 | -38.87% |
|  | FGC_SEI_I1C1 | 27.39 | 13.14 | -52.05% | 9.09 | 5.15 | -43.37% |
|  | Average | 30.42 | 14.38 | -52.74% | 10.14 | 5.85 | -42.34% |

The results in Table 3 and Table 4 indicate that resolution-adaptive grain block size results in a significant reduction in FGS processing time compared with the fixed 8x8 grain block size CE-Anchor. The average time reduction is around 20% for 4K clips (with 16x16 block size) and around 40~50% for 8K clips (with 32x32 block size).

It is noted that the FGC processing time is affected mainly by the number of colour components processed. The number of intensity intervals signalled in the FGC SEI message does not significantly affect FGS processing time. (Note, the number of intensity intervals does not affect how grain blocks are selected from the pre-computed grain pattern database. Thus, no effect on FGC processing time is expected.)

The average reduction in FGS processing time, measured as a percentage, is not strongly dependent on SIMD. However, the absolute FGS processing time for SIMD enabled tests is approximately 2.5 to 3 times faster than the FGS processing time tests in which SIMD was disabled.

The average reduction in FGS processing time is not affected by which transform is used to pre-compute the grain database from which grain blocks are selected during the FGS process, i.e., the FGS processing times for CE2.1 and CE2.2 are very similar.

## Subjective test results

For subjective tests, similar visual quality was observed for CE2.1 and CE2.2 compared with the anchor for selected 1080p and 4K test sequences. For test sequences with grain or noise in the source, decoded pictures with grain blending appeared more similar to the source than decoded pictures without grain blending. For test sequences without grain or noise in the source, grain blending tended to mask compression artefacts (texture loss, basis patterns etc.) when coding at low bitrate.

## Complexity and memory requirements

The FGS process described in SMPTE RDD 5 consists of the following steps:

- Pre-compute a grain pattern database

- Calculate an average luma value for each BlockSize x BlockSize block in the decoded picture to determine the intensity interval to which the block belongs

- Calculate pseudo-random offsets to be used to select grain blocks from the pre-computed grain pattern database

- Deblock vertical edges across adjacent grain blocks

- Add grain blocks to the decoded picture

Estimates of theoretical computational complexity for the FGS process for the luma component are enumerated in Table 5. Memory requirements are listed in Table 6. The complexity and memory requirements for chroma components can be estimated by scaling the estimate for luma based on the chroma format idc.

Following conclusions can be drawn from the analysis of normalized operations per sample listed in Table 5:

- FGS deblocking filter complexity is reduced to half for 16x16 block size compared to 8x8 block size. The filter complexity is reduced to a quarter for 32x32 block size.

- The bit masking and bit extract operations are reduced by a factor of 6x for 16x16 and 24x for 32x32 compared to 8x8 block size

- The modulo(%) operations are reduced by a factor of 4x for 32x32 block size, while they remain the same for 16x16 block size

- Rest of the processing has similar computational complexity for all the block sizes but implementation specific benefits such as effective SIMD utilization and reduced process (function) overhead for larger blocks bring in additional benefits as explained in the additional section on software (SW) benefits below

Table 5 Theoretical computational complexity for different grain block sizes

| Operation | Total Operations |  |  | Operations Per Sample |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Block size |  |  | Block size |  |  |
|  | 8x8 | 16x16 | 32x32 | 8x8 | 16x16 | 32x32 |
| Block Average (for intensity interval selection) |  |  |  |  |  |  |
| Load | 64 | 256 | 1024 | 1 | 1 | 1 |
| Addition | 64 | 256 | 1024 | 1 | 1 | 1 |
| Shift | 1 | 1 | 1 | 1/64 | 1/256 | 1/1024 |
| Grain Block (fg_block) creation using Pseudo Random Number Generated offsets x(r,ec) |  |  |  |  |  |  |
| Modulo (%) | 2 (per 16x16) | 2 | 2 | 2/256 | 2/256 | 2/1024 |
| Load/Store | 64*2 | 256*2 | 1024*2 | 2 | 2 | 2 |
| Multiply | 64 | 256 | 1024 | 1 | 1 | 1 |
| BitMask / BitExtract | 6 | 4 | 4 | 6/64 | 4/256 | 4/1024 |
| 3-Tap Deblocking Filter for vertical edges of adjacent grain blocks |  |  |  |  |  |  |
| Load/Store | 32+16 | 64+32 | 128+64 | 3/4 | 3/8 | 3/16 |
| Additions | 32 | 64 | 128 | 1/2 | 1/4 | 1/8 |
| Shift | 32 | 64 | 128 | 1/2 | 1/4 | 1/8 |
| Grain Blending |  |  |  |  |  |  |
| Load/Store | 64*3 | 256*3 | 1024*3 | 3 | 3 | 3 |
| Additions | 64 | 256 | 1024 | 1 | 1 | 1 |
| Clip | 64 | 256 | 1024 | 1 | 1 | 1 |

Table 6 Memory requirements for different grain block sizes

| Buffer | Total Memory |  |  | Memory per sample |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Block Size |  |  | Block Size |  |  |
|  | 8x8 | 16x16 | 32x32 | 8x8 | 16x16 | 32x32 |
| FG Database | X * 4096 | X * 4096 | X * 4096 | X * 4096/64 | X * 4096/256 | X * 4096/1024 |
| Recon buffer* | 64 * 2 | 256 * 2 | 1024 * 2 | 2 | 2 | 2 |
| FG Block* | 64 * 2 | 256 * 2 | 1024 * 2 | 2 | 2 | 2 |

Notes:

- X can vary based on implementation:

  - If all the 13x13 combinations of cut-off frequencies used for the grain pattern database, X = 169

  - Otherwise, X is the number of component models with different cut-off frequencies as signalled in the SEI message of current access unit (e.g., max of 10 for SMPTE-RDD 5)

- Recon buffer and FG block buffer sizes are multiplied by a factor of 2 because one current block and one previous block are required for vertical edge deblocking

- The complexity for pre-computing the grain pattern database is negligible because the grain pattern database can be performed offline and stored as lookup. Therefore, the complexities related to adaptive block size are the same for CE2.1 and CE2.2, i.e., the choice of transform matrix makes no difference.

Larger grain block sizes have additional benefits in SW due to following factors:

- SIMD benefits: Larger grain block sizes can utilize the SIMD processing more efficiently compared to smaller grain block sizes. For instance, 8x8 grain blocks use 128-bit SIMD parallelism which provides only 8x parallelism while 32x32 grain block sizes can be efficiently optimized for 256-bit (AVX2) SIMD width which provides 16x parallel processing for 16-bit input data. Even if 8x8 grain block sizes are optimized for 256-bit AVX2 SIMD, it will still need to load and store 128-bit data from different regions of memory which will increase the loads and stores by a factor of 2x compared to larger block sizes. It will also require additional overhead of scatter gather instructions and  significant restructuring of code to process adjacent 8x8 grain blocks.

- Function overheads: For a given resolution, larger grain block sizes have fewer function calls compared to smaller block sizes because they process much larger data in one function call. ‘C’ calling function overheads are typically 15 cycles – 20 cycles for most x86 architectures. These function calls can be a considerable percentage of total cycles for simple operations related to block averaging and grain blending. Compared to 8x8 grain block sizes, function calling overhead is reduced by a factor of 4x for 16x16 grain block sizes and a factor of 16x for 32x32 grain block sizes which gives additional benefits beyond SIMD processing.

The benefits of efficient SIMD processing and reduced function calling overheads are reflected in the percentage FGS processing time reduction for 4k (16x16 block size) and 8k (32x32 block sizes) CE tests as shown in Table 2 and Table 3.

# Conclusion

This contribution provides test results, complexity analysis, and software for the resolution-adaptive grain block aspects of the CE on film grain synthesis (JVET-W2022).

Test results indicate that using 16x16 grain blocks instead of 8x8 grain blocks results in an approximately 20% reduction in FGS processing time for 4K content. For 8K content, 32x32 grain blocks results in approximately 40-50% reduction in FGS processing time. The results are consistent with the complexity analysis which indicate the number of operations per sample is approximately the same for 8x8 grain blocks for 1080p, 16x16 grain blocks for 4K, and 32x32 grain blocks for 8K. Enabling SIMD resulted in approximately 2.5 to 3 times faster FGC processing times compared to SIMD disabled. No discernable difference in processing time was noted for the transform used to pre-compute the grain pattern database from which grain blocks are selected.

Subjectively, no difference was noted between the fixed 8x8 grain block anchors and resolution-adaptive grain blocks of the CE. Similarly, no subjective difference was noted for the transform selected to pre-compute the grain pattern database. For test sequences with grain or noise in the source, decoded pictures with grain blending appeared more similar to the source than decoded pictures without grain blending. For test sequences without grain or noise in the source, grain blending tended to mask compression artefacts (texture loss, basis patterns etc.) when coding at low bitrate.

It is recommended that the software for resolution-adaptive grain block size and the film grain characteristics SEI rewriter be adopted into VTM. Further, it is recommended that the corresponding grain blending process be incorporated into either VSEI or a technical report.

# Acknowledgement

We thank Alibaba and InterDigital for crosschecking the CE test results.

# Reference

- RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006.

- Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen, Edouard François, Miloš Radosavljević, Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam, “AHG9: Fixed-point grain blending process for film grain characteristics SEI message”, JVET-W0095, Teleconference, 7-16 July, 2021

- S. McCarthy, M. Radosavljević, J. Shingala, “Core Experiment on Film Grain Synthesis”, JVET-W2022, Teleconference, 7-16 July 2021.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier, “AHG9: Film grain estimation and film grain synthesis for VVC – Film grain characteristics SEI message characteristics, film grain estimation and film grain synthesis modules”, JVET-V0093, Teleconference, 20-28 April, 2021.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy and Guillaume Gautier, “AHG9: Enhancement of film grain parameter estimation for different intensity intervals”, JVET-W0072, Teleconference, 7-16 July, 2021.

- Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li, Vadim Seregin, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Teleconference, 7-16 Oct. 2020.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).