| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0055 |
| --- | --- |

| Title: | AHG11: neural network based in-loop filter with constrained storage and low complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang, Xiaozhong Xu, Shan Liu | Tel: Email: | {liqiangwang, xiaozhongxu, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

A neural network based in-loop filter with constrained storage and low complexity is proposed in this contribution. In all, only two models are used in the proposed filter design, for which each model is designed with relatively low complexity. In addition to the reconstruction image, other side information is fed into the network, such as the prediction image, partition image, slice QP and based QP. Based on the EE1 anchor, two NN filter tests are proposed with different complexity levels. For the filter test 1, { 7.15%, 19.33%, 19.11%}, {6.07%, 18.25%, 18.98%} and { 7.32%, 18.49%, 18.33%} BD-rate savings are reported with RA, LB and AI configurations, respectively. For the filter test 2, { 4.04%, 7.79%, 5.83%}, { 3.31%, 7.33%, 5.27%} and {4.82%, 8.48%, 6.84%} BD-rate savings are reported with RA, LB and RA configurations, respectively.

Introduction

Based on the network architecture in JVET-W0113[], more side information is utilized, and the design complexity is taken into consideration.

Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. In detail, as shown in Fig. 1, U and V channels are up-sampled and down-sampled in the pre-processing and post-processing, respectively.

Fig. 1 the pre-processing and post-processing units

Network architecture

The network structure of the proposed NN filter is shown in Fig. 2. Along with the reconstructed image (rec_yuv), the prediction image (pred_yuv) and the partition image (par_yuv) are also fed into the network.

Based on the un-filtered reconstructed image, the partition image is generated by setting each sample value into the average value of the whole partition block. It needs to be explained that the base QP is fed only for the network of the B slice, and the partition image is not fed for the B slice.

In addition, compared with JVET-W0113[1], the ResBlock is also modified as shown in Fig. 3. The convolutional block attention module (CBAM) is inserted, and the convolution in the skip-layer connection is canceled.

Fig. 2: Network architecture

Network information in training stage

Based on the network in the Fig. 2, two filters are proposed in the contribution, and the related information is shown in Table 1-2.

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |  |
| --- | --- | --- | --- |
|  | Filters | Filter 1 | Filter 2 |
| Mandatory | GPU Type | Tesla A100 40GB |  |
|  | Framework: | Torch v1.9.0 |  |
|  | Number of GPUs per Task | 1 |  |
|  | Epoch: | ~1000 |  |
|  | Batch size: | 64 |  |
|  | Loss function: | L1 |  |
|  | Training time: | ~150h |  |
|  | Training data information: | DIV2K, TVD, BVI-DVC |  |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |  |
| Optional |  |  |  |
|  | Number of iterations | 1200 |  |
|  | Patch size | 144x144 |  |
|  | Learning rate: | 1e-4 |  |
|  | Optimizer: | ADAM |  |
|  | Preprocessing: | flipped, rotated |  |
|  | Mini-batch selection process: | random cropped |  |
|  | Other information: |  |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Filters | Filter 1 |  | Filter 2 |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type |  | CPU only |  |
|  | Framework: |  | Torch v1.9.0 |  |
|  | Number of GPUs per Task |  | 0 |  |
|  | Number of Parameters (Each Model) |  | I: 1.56M, B: 1.55M | I: 170.2K, B: 169.39K |
|  | Total Number of Parameters (All Models) |  | 3.11M | 339.59K |
|  | Parameter Precision (Bits) |  | 32 |  |
|  | Memory Parameter (MB) |  | 13.1MB, 2 models in all | 2.91MB, 2 models in all |
|  | Multiplay Accumulate (MAC)/pixel, worst-case |  | 509K | 57K |
| Optional |  |  |  |  |
|  | Total Conv. Layers |  | 198 | 262 |
|  | Total FC Layers |  | 0 |  |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  | 1 |  |
|  | Patch size |  | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Border handling |  |  |  |
|  | Other information: |  |  |  |

Implementation

- For the I slice, Deblock and SAO are disabled, and the proposed filter is placed before ALF.

- For the B slice, Deblock and SAO are enabled, and the optimal filtered result is decided between the two outputs from the proposed filter and SAO.

- The proposed filter can be turned on/off at the CTU level and slice level.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component.

Results

Based on the EE1 anchor [][], two results of the filter 1 and 2 are shown in the section 3.1 and the section 3.2, respectively.

Results of Test 1

Table 3. Performance of the proposed method (filter 1, RA)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.71% | -13.84% | -16.21% | -9.59% | -16.37% | -17.81% | 129% | 35229% |
| Class A2 | -8.07% | -21.01% | -21.96% | -8.11% | -20.91% | -20.36% | 125% | 32578% |
| Class B | -7.04% | -21.64% | -19.27% | -7.03% | -21.53% | -19.15% | 127% | 34928% |
| Class C | -6.17% | -19.28% | -18.95% | -6.18% | -17.33% | -17.00% | 117% | 30652% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.15% | -19.33% | -19.11% | -7.53% | -19.25% | -18.55% | 124% | 33323% |
| Class D | -7.24% | -19.40% | -19.07% | -5.45% | -17.84% | -16.76% | 116% | 26564% |
| Class F | -2.36% | -10.29% | -9.62% | -2.78% | -10.90% | -10.34% | 143% | 16711% |

Table 4. Performance of the proposed method (filter 1, LB)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -5.78% | -17.65% | -19.37% | -5.93% | -16.87% | -20.44% | 126% | 32453% |
| Class C | -5.35% | -19.01% | -18.48% | -5.39% | -16.72% | -15.15% | 118% | 28930% |
| Class E | -7.51% | -18.16% | -17.60% | -7.74% | -16.90% | -15.96% | 163% | 24149% |
| Overall | -6.07% | -18.25% | -18.98% | -6.20% | -16.80% | -18.09% | 131% | 29009% |
| Class D | -6.00% | -17.63% | -17.38% | -4.48% | -15.39% | -17.10% | 116% | 27730% |
| Class F | -2.91% | -10.28% | -7.27% | -3.60% | -10.07% | -8.03% | 146% | 19017% |

Table 5. Performance of the proposed method (filter 1, AI)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.81% | -15.57% | -17.32% | -8.43% | -17.83% | -18.70% | 159% | 25777% |
| Class A2 | -6.62% | -20.14% | -19.89% | -7.51% | -21.65% | -18.62% | 131% | 20772% |
| Class B | -6.32% | -20.05% | -17.32% | -6.44% | -21.01% | -18.76% | 125% | 19977% |
| Class C | -7.57% | -17.22% | -18.79% | -7.16% | -19.37% | -20.39% | 114% | 14399% |
| Class E | -9.90% | -18.84% | -18.85% | -10.10% | -18.93% | -19.22% | 129% | 22979% |
| Overall | -7.32% | -18.49% | -18.33% | -7.72% | -19.87% | -19.16% | 129% | 19968% |
| Class D | -7.51% | -17.14% | -18.61% | -6.60% | -20.25% | -20.11% | 111% | 12868% |
| Class F | -2.90% | -7.01% | -7.68% | -3.48% | -7.99% | -8.82% | 114% | 12527% |

Results of Test 2

Table 6. Performance of the proposed method (filter 2, RA)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.69% | -5.89% | -5.31% | -5.38% | -7.23% | -5.71% | 110% | 9758% |
| Class A2 | -4.88% | -8.67% | -5.05% | -4.99% | -9.19% | -4.07% | 110% | 9038% |
| Class B | -4.08% | -8.32% | -6.51% | -4.62% | -8.58% | -6.53% | 110% | 9427% |
| Class C | -2.87% | -7.90% | -5.96% | -3.81% | -8.29% | -5.60% | 106% | 8509% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.04% | -7.79% | -5.83% | -4.63% | -8.36% | -5.63% | 109% | 9159% |
| Class D | -3.47% | -6.63% | -6.24% | -3.68% | -7.45% | -6.66% | 105% | 8128% |
| Class F | -1.17% | -4.03% | -3.32% | -1.46% | -3.55% | -3.18% | 114% | 4357% |

Table 7. Performance of the proposed method (filter 2, LB)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -3.37% | -7.15% | -5.42% | -4.14% | -7.47% | -5.97% | 109% | 8293% |
| Class C | -2.29% | -7.56% | -5.08% | -3.38% | -9.16% | -3.24% | 107% | 7992% |
| Class E | -4.58% | -6.26% | -5.89% | -5.65% | -6.16% | -5.60% | 120% | 4872% |
| Overall | -3.31% | -7.33% | -5.27% | -4.26% | -8.22% | -4.75% | 111% | 7171% |
| Class D | -2.82% | -4.42% | -4.65% | -3.32% | -6.15% | -6.77% | 105% | 7948% |
| Class F | -1.29% | -3.02% | -1.29% | -2.19% | -3.75% | -2.15% | 117% | 4622% |

Table 8. Performance of the proposed method (filter 2, AI)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.99% | -6.41% | -5.35% | -6.64% | -7.38% | -4.91% | 119% | 7352% |
| Class A2 | -4.51% | -9.09% | -6.25% | -4.78% | -8.78% | -4.94% | 111% | 5900% |
| Class B | -4.31% | -8.45% | -7.04% | -4.34% | -9.02% | -7.12% | 109% | 5653% |
| Class C | -4.55% | -9.57% | -7.18% | -4.71% | -10.07% | -7.37% | 105% | 4362% |
| Class E | -6.20% | -8.56% | -8.11% | -6.26% | -7.82% | -7.97% | 111% | 6784% |
| Overall | -4.82% | -8.48% | -6.84% | -5.20% | -8.74% | -6.58% | 110% | 5789% |
| Class D | -4.73% | -6.85% | -6.63% | -4.34% | -7.95% | -7.46% | 105% | 4033% |
| Class F | -2.33% | -5.01% | -4.25% | -2.48% | -4.83% | -3.07% | 105% | 5345% |

Conclusions

With constrained storage and low complexity, a neural network based in-loop filter is presented in this contribution. Good trade-off between performance and complexity can be achieved by applying the proposed method.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] L. Wang, W. Jiang, X. Xu, S. Liu. AHG11: neural network based in-loop filter. JVET-W0113, 23rd Meeting, by teleconference, 7–16 July 2021.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-W2016, 23rd Meeting, by teleconference, 7–16 July 2021.