| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0056-v2 |
| --- | --- |

| Title: | EE2-Related: Complexity reduction for decoder side motion derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Vadim Seregin, Zhi Zhang, Chun-Chi Chen, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents complexity reduction methods for the decoder side motion derivation tools in ECM-2.0. In the first test, early termination is applied to the DMVR and TM process. It reports 0.02%, 0.11%, 0. 11% BD rate for luma and chroma components, respectively, with 96% encoder runtime and 90% decoder runtime in random access configurations of common test condition. In the second test, on top of first test, the maximum number of iterations in square search process of DMVR is reduced and DMVR is disabled if a merge candidate has inherited additional hypothesis. It reports 0.xx%, 0.xx%, 0. xx% BD rate for luma and chroma components, respectively, with xx% encoder runtime and xx% decoder runtime.

# Introduction

In ECM-2.0, multi-pass decoder-side motion vector refinement (DMVR) and template matching (TM) are two powerful decoder side derivation tools. The multi-pass DMVR method is applied in regular merge mode if the selected merge candidate meets the DMVR conditions. In the first pass, bilateral matching (BM) is applied to the coding block. In the second pass, BM is applied to each 16x16 subblock within the coding block. In the third pass, MV in each 8x8 subblock is refined by applying bi-directional optical flow (BDOF). The TM method is applied in inter AMVP mode to derive the MVP, it’s also used in TM merge mode to refine the merge candidates.

# Proposal

In this contribution, it’s proposed to apply the following methods to reduce the complexity of the decoder side derivation tools:

- Reduce the maximum number of iterations from 26 to 8 in the 3x3 square search process of multi-pass DMVR.

- Disable multi-pass DMVR if a merge candidate has inherited additional hypothesis.

- Early termination in both multi-pass DMVR and TM.  Denote prevMinCost as the minimum cost in the previous iteration of search process and minCost as the minimum cost of current iteration, then the search process terminates if prevMinCost – minCost is less than a threshold.  In this contribution, the threshold is set equal to the area of the block.

# Test results

Two tests were conducted on top of ECM-2.0. In the first test, only early termination is applied, and it reports 0.02%, 0.11%, 0. 11% BD rate for luma and chroma components, respectively, with 96% encoder and 90% decoder run time in random access configurations of common test condition. In the second test, all proposed methods are applied. It reports 0.xx%, 0.xx%, 0. xx% BD rate for luma and chroma components, respectively, with xx% encoder runtime and xx% decoder runtime.

Table 1 Test 1 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.17% | 0.11% | 96% | 92% |
| Class A2 | 0.03% | 0.02% | 0.13% | 96% | 88% |
| Class B | 0.01% | 0.11% | 0.08% | 96% | 91% |
| Class C | 0.02% | 0.13% | 0.15% | 96% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.11% | 0.11% | 96% | 90% |
| Class D | -0.01% | -0.23% | 0.35% | 96% | 88% |
| Class F | 0.05% | 0.08% | -0.01% | 99% | 95% |

Table 2 Test 2 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusion

In this contribution, some complexity reduction methods are proposed to reduce the complexity of the decoder side derivation tools in ECM-2.0. In the first test, early termination is applied to DMVR and TM. it’s reported that it can achieve 4% encoder time reduction and 10% decoder time reduction without degrading the bd-rate performance. In the second test, all the proposed methods are applied, it reports 0.xx%, 0.xx%, 0. xx% BD rate for luma and chroma components, respectively, with xx% encoder runtime and xx% decoder runtime.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).