| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0064-v2 |
| --- | --- |

| Title: | EE1-2.2: CNN-based Super Resolution for Video Coding Using Decoded Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Kai Zhang2, Li Zhang2, 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the EE1-2.2 test results of JVET-W0099. JVET-W0099 presents a convolutional neural network-based super resolution scheme for video coding. Compared with VTM-11.0 with new MCTF enabled, the proposed method reportedly shows on average {-6.15%, 5.58%, -2.31%} and {-10.04%, 20.44%, -6.16%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

# Introduction

In JVET-W0099 [1] , a convolutional neural network-based super-resolution method is proposed for video coding. To fully utilize the decoded information and the cross-component correlation, the luma up-sampling network takes the sequence-level QP and the prediction signal as additional inputs, while the chroma up-sampling network further exploits the luma information to boost the performance. This contribution reports the test results of EE1-2.2, where the CNN-based SR method in JVET-W0099 is extended to inter slice.

# Proposed methods

Detailed information of the up-sampling network for luma and chroma components are described in the following sub-sections, respectively.

## Model for the luma component

The up-sampling model for the luma component is illustrated in Fig. 1. The input to the model consists of three parts, i.e., the low-resolution reconstruction YLRrec, the luma component of prediction frame YLRpred, and the QP map. The QP map is a matrix filled with the QP value. Those three parts are concatenated together and then fed into the first convolutional layer. The output from the first layer further goes through several residual blocks and one additional convolutional layer. Then, a shuffle layer generates the high-resolution reconstruction from the output of the last convolutional layer. To keep the complexity to be acceptable, in this contribution, N is set to 16, M is set to 96 and 64 for processing intra and inter slice, respectively.

Fig. 1 The proposed model for luma up-sampling.

## Model for chroma components

Fig. 2 shows the network structure of up-sampling model for the chroma components, which is similar to the luma up-sampling model except the input part. The input to the model consists of four parts, i.e., the low-resolution reconstruction YLRrec, the chroma components of reconstruction ULRrecand VLRrec, and the QP map. To utilize the cross-component correlation, the luma component of reconstruction is taken as one input. Specifically, the luma component YLRrec is first down-sampled to the same resolution as chroma components by a convolution layer with the stride of 2 (denoted by s2 in Fig. 2) for the 4:2:0 color format. The features extracted from the luma component are then concatenated with the features extracted from the chroma components along the channel dimension. In the simulations provided below, the M and N are set to 64 and 16, respectively. To reduce the number of parameters, a single chroma up-sampling model is utilized for both intra and inter slice.

Fig. 2 The proposed model for chroma up-sampling.

## Inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | LibTorch v1.8.1 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma up-sampling model for I slice:2.7 M luma up-sampling model for B slice: 1.37M chroma up-sampling model: 1.44M |
|  | Total Parameter Number | 5.51 M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 3 models in total: 22.04 MB |
|  | Multiply Accumulate (MAC) | 764 kMAC/pixel |
| Optional | Total Conv. Layers | 34 for up-sampling the luma, 36 for up-sampling the chroma |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

## Training details

DIV2K [3] and BVI_DVC [4] datasets are used for training. The VTM-11.0 [5] + new MCTF [6] is utilized to compress these images with QPs {22, 27, 32, 37, 42}. In the context of RPR scheme, the training data will be down-sampled by 2x before encoding. Then, we use the decoded low-resolution images and the corresponding original images for training.

Table 2 Network information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 120 |
|  | Batch size: | 16 |
|  | Training time: | 64h/model |
|  | Training data information: | DIV2K, BVI_DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

The proposed CNN-based up-sampling filter is tested on top of VTM-11.0 + new MCTF, according to the common test conditions defined in [7]. The QP values are {22, 27, 32, 37,42} for both anchor and test. The test results for RA and AI configurations are shown in Table 3 and Table 4, respectively.

Under RA configurations, the proposed method can bring {-6.15%, 5.58%, -2.31%} BD-rate changes on average for {Y, U, V} components, respectively.

Under AI configurations, the proposed method can bring {-10.04%, 20.44%, -6.16%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 3 Results of the proposed method for RA configurations.

|  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + new MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -9.37% | -22.03% | -13.07% | 110% | 29% |
|  | FoodMarket4 | -8.02% | -3.68% | -4.67% |  |  |
|  | Campfire | -18.95% | 40.95% | -11.12% |  |  |
| Class A2 4K | CatRobot1 | -0.21% | -14.75% | -3.56% | 100% | 29% |
|  | DaylightRoad2 | 8.89% | -17.76% | -1.43% |  |  |
|  | ParkRunning3 | -9.23% | 50.77% | 19.96% |  |  |
| Average on A1 |  | -12.11% | 5.08% | -9.62% | 110% | 29% |
| Average on A2 |  | -0.18% | 6.09% | 4.99% | 100% | 29% |
| overall |  | -6.15% | 5.58% | -2.31% | 105% | 29% |

Table 4 Results of the proposed method for AI configurations.

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + new MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -11.14% | -19.08% | -14.26% | 215% | 31% |
|  | FoodMarket4 | -5.95% | -1.98% | -2.11% |  |  |
|  | Campfire | -16.20% | 159.49% | -5.58% |  |  |
| Class A2 4K | CatRobot1 | -9.64% | -15.79% | -10.61% | 131% | 32% |
|  | DaylightRoad2 | -4.39% | -20.32% | -9.21% |  |  |
|  | ParkRunning3 | -12.94% | 20.33% | 4.84% |  |  |
| Average on A1 |  | -11.10% | 46.14% | -7.32% | 215% | 31% |
| Average on A2 |  | -8.99% | -5.26% | -5.00% | 131% | 32% |
| overall |  | -10.04% | 20.44% | -6.16% | 173% | 32% |

# Conclusion

This contribution presents a CNN-based super resolution method for the up-sampling filter for video coding. Different networks are designed for the luma and chroma components, respectively. The proposed method shows a promising coding gain.

# References

- AHG11: CNN-based Super Resolution for Video Coding Using Decoded Information, JVET-W0099, C. Lin, L. Zhang, K. Zhang, Y. Li

- Exploration experiments on Neural Network-based Video Coding, JVET-W2023, E. Alshina et al.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/, R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- GOP-based temporal filter improvements, JVET-V0056, P. Wennersten, C. Hollmann, J. Ström

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-W2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).