| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0096-v1 |
| --- | --- |

| Title: | AHG2/AHG9: On Multiview View Position (MVP) SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Arianne Hinds Xiang Zhang Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Three proposals on multiview view position SEI message, which was adopted into VSEI, are suggested.

Proposal-1: Extend the dimension of view positions from 1-D array to 2-D matrix.

Proposal-2: Add a constraint that a multiview view position SEI message shall not be contained in a scalable nesting SEI message.

Proposal-3: Adopt the use of multiview view position SEI message for VVC version 2.

# Proposal-1

Multiview View Position (MVP) SEI message was adopted into VSEI, at the July meeting. The MVP SEI message is useful for indicating the relative view position along a horizontal axis of 3D rendering view, especially for glassless 3D display as well as multiview-based 3D rendering. Figure 1 illustrates an example of reordering pictures according to multiview view position. When multiviews are presented, an information that indicates the display order from left to right needs to be signaled for providing the intended user experience.

However, the current MVP SEI message cannot represent 2D array of multiple views along both vertical and horizontal axes. As most state-of-the-art 3D display devices based on light-field and holography that support 2D representation of multiview, it is asserted that extending the dimension of view positions of MVP SEI message from 1D to 2D is considerable. In the MPEG workshop for Immersive Displays (https://mpeg.expert/live/nextcloud/index.php/s/sAgT8fNnqaNBaQY), various immersive media display technologies and devices that utilize fixed or configurable multiview display grid were introduced.

The proposed syntax and semantics employs a list of vertical view position (view_position_y[ i ]) as well as a list of horizontal view position (view_position_x[ i ]). num_vertical_view_positions_minus1 plus 1 and num_horizontal_view_positions_minus1 plus 1 indicates the number of vertical and horizontal view positions, respectively.

Figure 1. Example of multiview view position with 1-D array

Figure 2. Example of multiview view position with 2-D matrix

## Multiview view position SEI message ### Multiview view position SEI message syntax multiview_view_position( payloadSize ) {; Descriptor. num_views_minus1; ue(v). num_vertical_view_positions_minus1; ue(v). num_horizontal_view_positions_minus1; ue(v). for( i = 0; i <= num_views_minus1; i++ ) {. view_position[ i ]; ue(v). view_position_y[ i ]; ue(v). view_position_x[ i ]; ue(v). }. }. ### Multiview view position SEI message semantics The multiview view position SEI message specifies the relative view position along a single horizontal axis of views within a CVS. When present, the multiview view position SEI message shall be associated with an IRAP access unit. The information signalled in this SEI message applies to the entire CVS. When a CVS does not contain an SDI SEI message, the CVS shall not contain a multiview view position SEI message. num_views_minus1 plus 1 shall be equal to NumViews derived from the SDI SEI message for the CVS. num_vertical_view_positions_minus1 plus 1 specifies the number of vertical view positions. The value of num_vertical_view_positions_minus1 shall be in the range of 0 to 62, inclusive. num_horizontal_view_positions_minus1 plus 1 specifies the number of horizontal view positions. The value of num_horizontal_view_positions_minus1 shall be in the range of 0 to 62, inclusive. view_position_y[ i ] indicates the vertical order of the view with view identifier equal to ViewId[ i ] among all the views from top to bottom for the purpose of display, with the order for the top-most view being equal to 0 and the value of the order increasing by 1 for next view from top to bottom. The value of view_position_y[ i ] shall be in the range of 0 to num_vertical_view_positions_minus1, inclusive. view_position_x[ i ] indicates the horizontal order of the view with view identifier equal to ViewId[ i ] among all the views from left to right for the purpose of display, with the order for the left-most view being equal to 0 and the value of the order increasing by 1 for next view from left to right. The value of view_position_x[ i ] shall be in the range of 0 to num_horizontal_view_positions_minus1, inclusive.

# Proposal-2

In the current VSEI (JVET-W2006), there is a constraint that Scalability Dimension Information (SDI) SEI message shall not be contained in a scalable nesting SEI message. In addition, a CVS shall not contain an MVP SEI message, when the CVS does not contain an SDI SEI message. As the value of the syntax element num_views_minus1 has a semantic dependency on the value of NumViews derived form the SDI SEI message, the MVP SEI shall not be present, when the associated SDI SEI message is not present. Therefore, similarly to other Multiview-related SEI messages (e.g. SDI, MAI) adopted in VSEI, the constraint that an MVP SEI message shall not be contained in a scalable nesting SEI message is proposed.

## Multiview view position SEI message ### Multiview view position SEI message syntax multiview_view_position( payloadSize ) {; Descriptor. num_views_minus1; ue(v). for( i = 0; i <= num_views_minus1; i++ ). view_position[ i ]; ue(v). }. ### Multiview view position SEI message semantics The multiview view position SEI message specifies the relative view position along a single horizontal axis of views within a CVS. When present, the multiview view position SEI message shall be associated with an IRAP access unit. The information signalled in this SEI message applies to the entire CVS. A multiview view position SEI message shall not be contained in a scalable nesting SEI message. When a CVS does not contain an SDI SEI message, the CVS shall not contain a multiview view position SEI message.

# Proposal-3

It is also proposed to add Multiview View Position (MVP) SEI message into VVC version 2, similarly to other multiview-related SEI messages.

### General SEI payload syntax sei_payload( payloadType, payloadSize ) {; Descriptor. SeiExtensionBitsPresentFlag = 0. if( nal_unit_type = = PREFIX_SEI_NUT ). .... else if( payloadType = = 45 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. frame_packing_arrangement( payloadSize ). else if( payloadType = = 47 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. display_orientation( payloadSize ). .... else if( payloadType = = 137 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. mastering_display_colour_volume( payloadSize ). else if( payloadType = = 142 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. colour_transform_info( payloadSize ). .... else if( payloadType = = 165 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. alpha_channel_info( payloadSize ). else if( payloadType = = 168 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. frame_field_info( payloadSize ). else if( payloadType = = 177 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. depth_representation_info( payloadSize ). else if( payloadType = = 178 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. multiview_view_position ( payloadSize ). else if( payloadType = = 179 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. multiview_acquisition_info( payloadSize ). else if( payloadType = = 200 ). sei_manifest( payloadSize ). else if( payloadType = = 201 ). sei_prefix_indication( payloadSize ). else if( payloadType = = 202 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. annotated_regions( payloadSize ). else if( payloadType = = 203 ). subpic_level_info( payloadSize ). else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. sample_aspect_ratio_info( payloadSize ). else if( payloadType = = 205 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. scalability_dimension_info( payloadSize ). else if( payloadType = = 206 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. extended_drap_indication( payloadSize ). else if( payloadType = = 207 ). constrained_rasl_encoding_indication( payloadSize ). else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. reserved_message( payloadSize ). .... else /* nal_unit_type = = SUFFIX_SEI_NUT */. if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. filler_payload( payloadSize ). else if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. decoded_picture_hash( payloadSize ). else if( payloadType = = 133 ). scalable_nesting( payloadSize ). else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */. reserved_message( payloadSize ). if( SeiExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) {. if( payload_extension_present( ) ). sei_reserved_payload_extension_data; u(v). sei_payload_bit_equal_to_one /* equal to 1 */; f(1). while( !byte_aligned( ) ). sei_payload_bit_equal_to_zero /* equal to 0 */; f(1). }. }. The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 47, 129, 132, 137, 142, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 178, 179, 180, 200 to 202, inclusive, and 204 to 207, inclusive. The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 19, 45, 129, 133, 137, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 178, 179, 180, and 200 to 207, inclusive. It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in a scalable nesting SEI message: - An SEI message that has payloadType equal to 3 (filler payload), 133 (scalable nesting), 178 (multiview view position), 179 (multiview acquisition information), or 205 (scalability dimension information) shall not be contained in a scalable nesting SEI message. - When a scalable nesting SEI message contains a BP, PT, DUI, or SLI SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI).

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).