| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24rd Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0101 |
| --- | --- |

| Title: | AHG9: On the CREI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Christian Bartnik Adam Wieckowski Karsten Suehring Yago Sanchez Benjamin Bross Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

At the last JVET meeting, the Constrained RASL Encoding Indication (CREI) SEI message was adopted into the VVC v2 Draft Specification that indicates that RASL pictures within a CVS are encoded with encoder-side constraints. The constraints enable bitstream resolution switching with efficient open GOP coding structures (CRA+RASL pictures) without severe artifacts in RASL pictures.

Such bitstream resolution switching involves post-encoder usage of RPR that disables wrap-around motion compensation, even if it has been used on encoder side. In order to avoid mismatches in such situations, wrap-around motion compensation needs to be disabled on encoder side. While this is possible at PPS level, the same issues than for the other constraints indicated by the CREI SEI message arise from systems perspective, e.g. packaging such a set of bitstreams would require thorough parsing to understand the situation at each switching point in the bitstreams.

This document proposes to add a constraint to the CREI SEI message requiring disabling of wrap-around motion compensation in the PPSs referred to by the RASL pictures of a CVS. Thereby, the remaining non-RASL inter-pictures may use wrap-around motion compensation and avoid an unnecessary coding efficiency penalty while post-encoder usage of RPR by RASL pictures does not lead to severe artifacts.

# Introduction

At the last JVET meeting, the Constrained RASL Encoding Indication (CREI) SEI message was adopted into the VVC v2 Draft Specification. The CREI SEI message indicates that RASL pictures within a CVS are encoded with certain encoder-side constraints regarding (SB)TMVP, DMVR and CCLM so that bitstream resolution switching at CRA pictures does not lead to annoying visual artefacts in RASL pictures. A detailed problem description and performance analysis can be found in JVET-W0133.

Typically, the encoding of content for such applications, i.e. generating bitstreams with various picture resolutions to switch among during a streaming session, does not employ reference picture resampling (RPR) on encoder side. However, the concatenated client-side bitstream may trigger usage of RPR at bitstream switching points when decoding of RASL pictures uses reference pictures of earlier CVSs belonging to bitstream segments with a different picture resolution.

More precisely, when a reference picture is scaled with respect to the current picture, wrap-around motion compensation is disabled in Section  8.5.6.3 Fractional sample interpolation process:

The variable refWraparoundEnabledFlag is set equal to ( pps_ref_wraparound_enabled_flag && !refPicIsScaled ).

While wrap-around motion compensation can be disabled on PPS level to avoid such mismatches, the same issues than for the (SB)TMVP and DMVR constraints of the CREI SEI message arise from systems perspective, e.g. packaging such a set of bitstreams would require thorough parsing to understand the situation at each switching point in all bitstreams.

A possible work-around for the target scenario is to disable wrap-around motion compensation for the complete bitstream at SPS level. This would sufficiently avoid mismatches during bitstream switching but also would come at the cost of an unnecessary coding efficiency loss for all non-RASL inter-pictures when encoding 360-degree video content. As Video-on-Demand services typically rely on longer segment lengths and thereby relatively few RASL pictures in the bitstream, many pictures would be unnecessarily affected by such a work-around.

# Proposal

This document proposes to add a constraint to the CREI SEI message requiring disabling of wrap-around motion compensation in the PPSs referred to by the RASL pictures of a CVS.

Thereby, the remaining non-RASL inter-pictures may utilize wrap-around motion compensation and avoid an unnecessary coding efficiency penalty while post-encoder usage of RPR by RASL pictures does not lead to severe mismatches. The proposed addition is highlighted in yellow.

## D.10	Constrained RASL encoding indication SEI message

### D.10.1	Constrained RASL encoding indication SEI message syntax

| constrained_rasl_encoding_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

### D.10.2	Constrained RASL encoding indication SEI message semantics

The presence of the constrained RASL encoding indication (CREI) SEI message in a CVS indicates that a set of encoding constraints as described below applies for all RASL pictures in the CVS.

NOTE 1	In some applications such as bit-rate adaptive streaming services, when this set of encoding constraints is applied, it is expected to be possible to perform bitstream switching between different bitstreams that represent the same source video content at CRA pictures that have associated RASL pictures with fewer visually noticeable or visually annoying artefacts in the reconstructed sample values of the RASL pictures than when the RASL pictures are encoded without applying this set of constraints.

NOTE 2	CRA pictures with associated RASL pictures are sometimes referred to as open group of pictures (open-GOP) IRAP pictures.

When a CREI SEI message is present for any picture of an AU of a CVS, a CREI SEI message shall be present for the first picture of the CVSS AU. The CREI SEI message persists in decoding order from the current AU until the end of the CVS.

The presence of the CREI SEI message indicates that the following conditions all apply for each RASL picture in the CVS:

- The PH syntax structure has ph_dmvr_disabled_flag equal to 1.

- The PPS refered to by the RASL picture has pps_ref_wraparound_enabled_flag equal to 0.

- No CU in a slice with sh_slice_type equal to 0 (B) or 1 (P) has cclm_mode_flag equal to 1.

- No collocated reference picture precedes the CRA picture associated with the RASL picture in decoding order.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)