| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0108-v1 |
| --- | --- |

| Title: | Coded Picture Buffer sizes and MinCr for high bit-depth profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Adrian Browne Karl Sharman | Email: | steve.keating@sony.com adrian.browne@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This contribution describes proposed changes to maximum coded picture buffer (CPB) size for 12-bit profiles in order to guarantee that the buffer can always store a full picture when compressed at the minimum compression ratio (MinCr).

# Introduction

The document “VVC operation range extensions (Draft 4)”, JVET-W2005-v1 [1], details changes to the VVC specification for high bit-depth profiles. Table 5 specifies the CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor:

Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profiles | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10, Main 10 Still Picture, Multilayer Main 10, Main 12, Main 12 Intra, Main 12 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 10 4:4:4, Main 10 4:4:4 Still Picture, Multilayer Main 10 4:4:4, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 16 4:4:4, Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 4 000 | 4 400 | 6.000 | 0.75 |

# Problem

The same values of CpbVclFactor etc., are specified for 10-bit and 12-bit profiles, this can result in the Coded Picture Buffer being too small to contain a complete frame at the minimum compression ratio for 12-bit profiles. The table below shows the max picture size, max CPB size, max coded picture size at MinCr and the minimum number of pictures in the CPB for Main 12 profile:

| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) | Max CPB size MaxCPB (bits) | Min compression ratio MinCrBase | MinCr | Max coded luma picture size (bits) | Min coded pictures in CPB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | Main tier | Main tier | Main tier | Main tier | Main tier |
| 1 | 36,864 |  | 350 | 350,000 | 2 | 2 | 331,776 | 1.1 |
| 2 | 122,880 |  | 1,500 | 1,500,000 | 2 | 2 | 1,105,920 | 1.4 |
| 2.1 | 245,760 |  | 3,000 | 3,000,000 | 2 | 2 | 2,211,840 | 1.4 |
| 3 | 552,960 |  | 6,000 | 6,000,000 | 2 | 2 | 4,976,640 | 1.2 |
| 3.1 | 983,040 |  | 10,000 | 10,000,000 | 2 | 2 | 8,847,360 | 1.1 |
| 4 | 2,228,224 | 2048x1088 | 12,000 | 12,000,000 | 4 | 4 | 10,027,008 | 1.2 |
| 4.1 | 2,228,224 | 2048x1088 | 20,000 | 20,000,000 | 4 | 4 | 10,027,008 | 2.0 |
| 5 | 8,912,896 | 4096x2176 | 25,000 | 25,000,000 | 6 | 6 | 26,738,688 | 0.9 |
| 5.1 | 8,912,896 | 4096x2176 | 40,000 | 40,000,000 | 8 | 8 | 20,054,016 | 2.0 |
| 5.2 | 8,912,896 | 4096x2176 | 60,000 | 60,000,000 | 8 | 8 | 20,054,016 | 3.0 |
| 6 | 35,651,584 | 8192x4352 | 80,000 | 80,000,000 | 8 | 8 | 80,216,064 | 1.0 |
| 6.1 | 35,651,584 | 8192x4352 | 120,000 | 120,000,000 | 8 | 8 | 80,216,064 | 1.5 |
| 6.2 | 35,651,584 | 8192x4352 | 180,000 | 180,000,000 | 8 | 8 | 80,216,064 | 2.2 |
| 6.3 | 80,216,064 | 12288x6480 | 240,000 | 240,000,000 | 8 | 8 | 180,486,144 | 1.3 |

For level 5 the CPB size is not sufficient for a complete frame.

And the following table shows the max picture size, max CPB size, max coded picture size at MinCr and the minimum number of pictures in the CPB for Main 12 4:4:4 profile:

| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) | Max CPB size MaxCPB (bits) | Min compression ratio MinCrBase | MinCr | Max coded luma picture size (bits) | Min coded pictures in CPB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | Main tier | Main tier | Main tier | Main tier | Main tier |
| 1 | 36,864 |  | 350 | 875,000 | 2 | 1.5 | 884,736 | 1.0 |
| 2 | 122,880 |  | 1,500 | 3,750,000 | 2 | 1.5 | 2,949,120 | 1.3 |
| 2.1 | 245,760 |  | 3,000 | 7,500,000 | 2 | 1.5 | 5,898,240 | 1.3 |
| 3 | 552,960 |  | 6,000 | 15,000,000 | 2 | 1.5 | 13,271,040 | 1.1 |
| 3.1 | 983,040 |  | 10,000 | 25,000,000 | 2 | 1.5 | 23,592,960 | 1.1 |
| 4 | 2,228,224 | 2048x1088 | 12,000 | 30,000,000 | 4 | 3 | 26,738,688 | 1.1 |
| 4.1 | 2,228,224 | 2048x1088 | 20,000 | 50,000,000 | 4 | 3 | 26,738,688 | 1.9 |
| 5 | 8,912,896 | 4096x2176 | 25,000 | 62,500,000 | 6 | 4.5 | 71,303,168 | 0.9 |
| 5.1 | 8,912,896 | 4096x2176 | 40,000 | 100,000,000 | 8 | 6 | 53,477,376 | 1.9 |
| 5.2 | 8,912,896 | 4096x2176 | 60,000 | 150,000,000 | 8 | 6 | 53,477,376 | 2.8 |
| 6 | 35,651,584 | 8192x4352 | 80,000 | 200,000,000 | 8 | 6 | 213,909,504 | 0.9 |
| 6.1 | 35,651,584 | 8192x4352 | 120,000 | 300,000,000 | 8 | 6 | 213,909,504 | 1.4 |
| 6.2 | 35,651,584 | 8192x4352 | 180,000 | 450,000,000 | 8 | 6 | 213,909,504 | 2.1 |
| 6.3 | 80,216,064 | 12288x6480 | 240,000 | 600,000,000 | 8 | 6 | 481,296,384 | 1.2 |

For levels 5 and 6 the CPB size is not sufficient for a complete frame.

Main 10, Main 10 4:4:4 and Main 16 4:4:4 profiles do not have this problem. The accompanying spreadsheet allows each profile to be selected to obtain tables as show above.

# Proposed Changes

It is proposed that Table 5 is modified to specify larger values for CpbVclFactor and CpbNalFactor for 12-bit profiles as below:

Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profiles | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10, Main 10 Still Picture, Multilayer Main 10, Main 12, Main 12 Intra, Main 12 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 12, Main 12 Intra, Main 12 Still Picture | 1 500 | 1 650 | 1.875 | 1.00 |
| Main 10 4:4:4, Main 10 4:4:4 Still Picture, Multilayer Main 10 4:4:4, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 3 750 | 4 125 | 3.750 | 0.75 |
| Main 16 4:4:4, Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 4 000 | 4 400 | 6.000 | 0.75 |

This increase by a factor of 1.5 ensures that the CPB size is sufficient for 12-bit profiles.  Although a factor of 1.2 would be sufficient to fix this problem a factor of 1.5 is in line with the latest draft HEVC specification [2] for 12-bit profiles.

# Conclusion

The Specifications in Table 5 of the document “VVC operation range extensions (Draft 4)”, JVET-W2005-v1 can result in the Coded Picture buffer being too small for 12-bit profiles at MinCr.  It is therefore proposed to increase the buffer size by a factor of 1.5 for all 12-bit profiles.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “VVC operation range extensions (Draft 4)” JVET-W2005-v1, F Bossen, B Bross, T Ikai, D Rusanovskyy, G Sullivan, Y-K Wang

- “ ITU-T Rec. H.265v8 High Efficiency Video Coding (08/2021)”