| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0110-v3 |
| --- | --- |

| Title: | [EE1-1.1: Content-adaptive neural network post-processing filter] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Maria Santamaria, Jani Lainema, Francesco Cricri, Ramin G. Youvalari, Honglei Zhang, Alireza Zare, Hamed R. Tavakoli, Miska Hannuksela. | Email: | {maria.santamaria_gomez.ext, jani.lainema, francesco.cricri, ramin.ghaznavi-youvalari, honglei.1.zhang, alireza.zare, hamed.rezazadegan_tavakoli, miska.hannuksela}@nokia.com |
| Source: | [Nokia Technologies] |  |  |

_____________________________

# Abstract

This Exploration Experiment (EE) contribution studies the content-adaptive Convolutional Neural Network (CNN) post-processing filter presented in JVET-W0057. Two experiments were carried out. In the first one (EE1-1.1.1), content-adaptive CNNs were used. In the second one (EE1-1.1.2), pre-trained CNNs were used. A scaling factor determines the strength of the filter to be applied to the output. The scaling factor is selected by the encoder and signalled as a slice-level parameter within the bitstream. In addition, there is Coding Tree Unit (CTU) level control that allows the encoder to switch the filter on and off separately for luma and chroma. The evaluation was done on top of the VVC Test Model (VTM) 11.0 with Neural Network for Video Coding (NNVC) technology 1.0, using the Random Access (RA) configuration. It is reported that the proposed content-adaptation technique increases the overall BD-rate gains (over all classes) from -0.36% (Y), -0.13% (U), -0.82% (V) to -2.51% (Y), -5.86% (U), -3.86% (V).

# Introduction

This contribution studies the CNN post-processing filter in JVET-W0057 [1]. The architecture of the CNN is illustrated in Figure 1, the inputs are the YUV channels and the normalised frame Quantisation Parameter (QP). The first block comprises a convolutional layer (excluding the biases), a bias layer and a LeakyReLU as the non-linear activation function. There are four following blocks with similar layers as in the first block, but where the input of each block is added to the block’s output (by using a skip connection). The convolutional layers in the first block and in the following four blocks use 64 kernels. The last block is like the first block (no skip connection), but with 3 convolutional kernels. The input YUV channels are summed to the output of the CNN.

The CNN contains 151188 parameters and the computational complexity in MACs per pixel is 151488.

Figure 1. CNN architecture.

The CNN is pre-trained offline using a subset of training video sequences available for the JVET Common Test Conditions (CTC) for NNVC technology [2]. The BVI-DVC dataset [3] was used for classes A, B, C and D, while screen content video sequences were used for class F.

The video sequences were encoded with VTM 11.0 with NNVC 1.0, QPs 22, 27, 32, 37 and 42, and the Random Access (RA) configuration. For each epoch, all the frames from each video sequence are selected once randomly. From each frame in these video sequences, a 128x128 patch from the reconstructed sequence was used as the input to the CNN filter, and the corresponding uncompressed patch was used as the ground-truth. The patches were cropped by choosing their position randomly within the frames. Whenever necessary, padding was performed by using boundary pixels. The normalised frame-QP was used as an additional input channel, stacked to the Y, U, V channels. The chroma channels were up-sampled by duplicating the neighbouring rows and columns.

Table I reports the network information in pre-training stage.

Table I. Network information in pre-training stage

| Network Information in Pre-Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Nvidia Tesla V100 |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 300 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted MSE |
|  | Training time: | Class A: ~ 89 hours Class B: ~75 hours Class C: ~63 hours Class D: ~25 hours Class F: ~54 hours |
|  | Training data information: | BVI-DVC and screen content sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | As in JVET CTC NNVC |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

Next, the bias terms of the CNN are fine-tuned for each pair of [sequence, sequence QP]. Once the fine-tuning is done, the weight-update is calculated as the difference between the fine-tuned bias terms and the pre-trained bias terms. Finally, the weight-update is compressed with LZMA2 algorithm. The bitrate of the weight-update is considered in the bitrate and in the BD-rate computation.

The data preparation is done as for the pre-training, but in this case the patches are the frame Coding Tree Units (CTUs).

Table II reports the network information in fine-tuning stage.

Table II. Network information in fine-tuning stage

| Network Information in Fine-Tuning Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Nvidia Tesla V100 |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted MSE |
|  | Training time: | Class A: ~19.2 hours per video sequence Class B: ~5.5 hours per video sequence Class C: ~1.3 hours per video sequence Class D: ~0.4 hours per video sequence Class F: ~2.7 hours per video sequence |
|  | Training data information: | JVET-CTC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | As in JVET CTC NNVC |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

# EE1-1.1.1

This experiment uses the fine-tuned CNNs. Table III reports the network information in inference stage. The coding performance is reported in Table IV.

Table III. Network information for EE1-1.1.1 in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task |  |
|  | Total Parameter Number | 151811 |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 0,579113007 (per NN) 2,895565035 (all NNs) |
|  | Multiply Accumulate (MAC) | 151488 |
| Optional |  |  |
|  | Total Conv. Layers | 6 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 8 |
|  | Patch size | 128x128 |
|  | Changes to network configuration or weights required to generate rate points | 1 NN for each [sequence, QP] pair |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | Padding with boundary pixels when necessary |
|  | Other information: |  |

Table IV. Coding performance for EE1-1.1.1

|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -2,03 % | -2,69 % | -3,50 % | -3,80 % | -4,85 % | -4,50 % | 115 % | 22296 % | 0 % |
| Class A2 | -1,47 % | -3,19 % | -0,68 % | -1,10 % | -4,19 % | -0,87 % | 113 % | 20279 % | 0 % |
| Class B | -2,98 % | -9,56 % | -6,19 % | -1,78 % | -10,63 % | -6,86 % | 139 % | 64233 % | 0 % |
| Class C | -3,07 % | -5,61 % | -3,63 % | -1,18 % | -6,60 % | -4,10 % | 123 % | 53136 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -2,51 % | -5,86 % | -3,86 % | -1,89 % | -7,11 % | -4,45 % | 124 % | 39241 % | 0 % |
| Class D | -3,51 % | 0,36 % | 0,25 % | -0,49 % | -0,75 % | -0,07 % | 110 % | 18421 % | 1 % |
| Class F | -1,16 % | -0,85 % | 0,49 % | -0,72 % | -3,42 % | -1,87 % | 158 % | 76871 % | 1 % |

# EE1-1.1.2

This experiment uses the pre-trained CNNs. Table V reports the network information in inference stage. The coding performance is reported in Table VI.

Table V. Network information for EE1-1.1.2 in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task |  |
|  | Total Parameter Number | 151811 |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 0,579113007 (per NN) 2,895565035 (all NNs) |
|  | Multiply Accumulate (MAC) | 151488 |
| Optional |  |  |
|  | Total Conv. Layers | 6 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 8 |
|  | Patch size | 128x128 |
|  | Changes to network configuration or weights required to generate rate points | 1 NN for all QPs. 1 NN per content class. |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | Padding with boundary pixels when necessary |
|  | Other information: |  |

Table VI. Coding performance for EE1-1.1.2

|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -0,18 % | 1,30 % | -0,79 % | -1,13 % | -1,21 % | -1,59 % | 112 % | 21609 % | 0 % |
| Class A2 | -0,52 % | 3,44 % | 1,41 % | -0,66 % | 1,37 % | 0,71 % | 111 % | 18917 % | 0 % |
| Class B | -0,85 % | -3,42 % | -2,35 % | -1,02 % | -4,25 % | -3,10 % | 138 % | 58269 % | 0 % |
| Class C | 0,24 % | 0,24 % | -0,60 % | -0,78 % | -1,83 % | 0,00 % | 123 % | 47585 % | 0 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,36 % | -0,13 % | -0,82 % | -0,91 % | -1,87 % | -1,21 % | 123 % | 36150 % | 0 % |
| Class D | -2,68 % | 0,00 % | 0,28 % | -1,28 % | -1,91 % | 0,15 % | 110 % | 16389 % | 0 % |
| Class F | 35,02 % | 43,02 % | 35,39 % | 54,38 % | 17,11 % | 13,69 % | 165 % | 75387 % | 0 % |

While the usage of the pre-trained CNNs as post-processing filter brought coding gains, using content-adaptive CNNs (fine-tuned) increased the gains significantly.

# References

- M. Santamaria, Y.-H. Lam, J. Lainema, F. Cricri, R. Ghaznavi-Youvalari, H. Zhang, A. Zare, H. R. Tavakoli, M. Hannuksela. AHG11: Content-adaptive neural network post-processing filter, JVET-W0057. July 2021.

- S. Liu, A. Segall, E. Alshina, R.-L. Liao. Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-W2016. July 2021.

- Zhang, F. (Creator), Ma, D. (Creator), Bull, D. (Creator) (31 Mar 2020). BVI-DVC. University of Bristol. 10.5523/bris.3hj4t64fkbrgn2ghwp9en4vhtn.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).