| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0123-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-X0118 (EE1-3.1: BD-rate gains vs complexity of NN-based intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Johannes Sauer | Email: | johannes.sauer@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for EE-3.1 described in JVET-W2023 [1], which evaluates BD-rate gains vs complexity of NN-based intra prediction. In EE-3.1 three versions of NN-based intra prediction are tested:

- Floating point NN

- Regular integer NN

- Low complexity integer NN

Each version utilizes a set of neural networks, one network for each supported block size (unless the desired block size is obtained by transposing or down sampling an already supported size).

For the integer NN the simulation results gave an exact match with those supplied by the proponents. For the floating point NN slight differences are observed and mismatches occur when platform of encoder and decoder differ.

# Results

The code [3] was compiled using gcc 6.5.0 and tested according to the NN-CTC [2] on Linux clusters running Ubuntu 14.04.5 LTS. The results were compared with a similarly prepared EE anchor of VTM11.0-nvc [4]. EE-3.1.1 compares the two variants integer NN. EE-3.1.2 compares the regular integer NN against the floating point NN.

Results for EE-3.1.1 were found to be in exact agreement with the proponent’s results. Results for EE-3.1.2 show minor differences. This was expected since EE-3.1.2 uses a floating point implementation of the neural networks.

Decoding on Windows was additionally tested for the coded bitstreams. No mismatches occurred for EE-3.1.1. For EE-3.1.2 mismatches occurred. Again this was expected due to the floating point implementation used here.

## Test 1 (Floating point NN) vs anchor

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class A1 | -3.98% | -3.16% | -3.45% | -4.22% | -2.71% | -3.14% | 375% | 3949% | #NUM! | 3% |
| Class A2 | -2.44% | -2.40% | -2.77% | -2.47% | -1.73% | -2.05% | 400% | 4026% | #NUM! | 2% |
| Class B | -2.93% | -2.43% | -2.65% | -2.65% | -2.03% | -1.83% | 400% | 4119% | #NUM! | 2% |
| Class C | -3.09% | -2.47% | -2.68% | -3.07% | -1.83% | -1.80% | 409% | 4605% | #NUM! | 3% |
| Class E | -4.57% | -4.30% | -3.40% | -4.74% | -3.87% | -1.96% | 414% | 4829% | #NUM! | 4% |
| Overall | -3.33% | -2.87% | -2.94% | -3.32% | -2.36% | -2.10% | 400% | 4092% | #NUM! | 3% |
| Class D | -2.99% | -2.62% | -2.80% | -2.85% | -1.72% | -1.65% | 388% | 5038% | #NUM! | 2% |
| Class F | -2.25% | -1.63% | -1.73% | -2.24% | -1.46% | -1.34% | 240% | 2462% | #NUM! | 2% |
| Class H | -2.51% | -3.12% | -2.63% | -2.26% | -3.96% | -2.68% | 394% | 2491% | #NUM! | 2% |

## Test 3 (Regular integer NN) vs anchor

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class A1 | -3.93% | -3.10% | -3.37% | -4.14% | -2.68% | -3.12% | 264% | 2554% | #NUM! | 3% |
| Class A2 | -2.43% | -2.33% | -2.69% | -2.49% | -1.64% | -2.03% | 274% | 2422% | #NUM! | 2% |
| Class B | -2.92% | -2.41% | -2.57% | -2.66% | -1.98% | -1.79% | 271% | 2321% | #NUM! | 2% |
| Class C | -3.08% | -2.48% | -2.78% | -3.11% | -1.78% | -1.89% | 278% | 2427% | #NUM! | 3% |
| Class E | -4.55% | -4.29% | -3.53% | -4.76% | -3.84% | -1.94% | 282% | 2813% | #NUM! | 4% |
| Overall | -3.31% | -2.84% | -2.93% | -3.33% | -2.31% | -2.10% | 274% | 2410% | #NUM! | 3% |
| Class D | -2.98% | -2.51% | -2.73% | -2.91% | -2.11% | -1.80% | 255% | 2411% | #NUM! | 2% |
| Class F | -2.22% | -1.58% | -1.63% | -2.44% | -1.26% | -1.31% | 182% | 1408% | #NUM! | 2% |
| Class H | -2.49% | -3.18% | -2.47% | -2.24% | -4.13% | -2.86% | 280% | 1429% | #NUM! | 2% |

## Test 4 (Low complexity integer NN) vs anchor –

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class A1 | -3.76% | -2.87% | -3.21% | -3.99% | -2.43% | -3.03% | 263% | 2440% | #NUM! | 3% |
| Class A2 | -2.09% | -1.91% | -2.24% | -2.22% | -1.31% | -1.69% | 268% | 2298% | #NUM! | 2% |
| Class B | -2.49% | -1.97% | -2.10% | -2.25% | -1.60% | -1.43% | 268% | 2141% | #NUM! | 2% |
| Class C | -2.53% | -1.86% | -1.95% | -2.58% | -1.14% | -0.97% | 266% | 2054% | #NUM! | 2% |
| Class E | -3.88% | -3.39% | -2.67% | -4.05% | -3.14% | -1.23% | 270% | 2529% | #NUM! | 3% |
| Overall | -2.87% | -2.32% | -2.37% | -2.91% | -1.84% | -1.60% | 267% | 2212% | #NUM! | 2% |
| Class D | -2.47% | -2.05% | -2.08% | -2.43% | -1.56% | -1.14% | 261% | 2194% | #NUM! | 2% |
| Class F | -1.82% | -1.19% | -1.18% | -1.83% | -0.83% | -0.62% | 179% | 1256% | #NUM! | 2% |
| Class H | -2.25% | -2.84% | -2.17% | -2.10% | -3.97% | -2.63% | 273% | 1281% | #NUM! | 2% |

# References

[1] Exploration Experiments on Neural Network-based Video Coding (EE1), JVET-W2023,  E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang

[2] Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-W2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao

[3] https://vcgit.hhi.fraunhofer.de/jvet-w-ee1/VVCSoftware_VTM/-/tree/EE1-3.1

[4] https://vcgit.hhi.fraunhofer.de/jvet-w-ee1/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc