| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting: Teleconference, 6–15 October 2021 | Document: JVET-X0124-v1 |
| --- | --- |

| Title: | AHG 12: On signalling of intra template matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Luhang Xu Fan Wang Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

In this contribution, harmonization of CU-level flags of decoder-side intra mode derivation (DIMD) and intra template matching prediction (TMP) is proposed, which aims to remove redundant signalling bits in the bitstream. In the ECM-2.0 software, one dedicated flag at CU level is signalled to indicate if TMP is executed. It is proposed to skip TMP signalling flag when DIMD is in use at the current CU. This modification has been implemented in the ECM-2.0 software and the test results of class F showed, on average, coding gain of {x%, x%, x%} for AI, {x%, x%, x%} for RA, and {x%, x%, x%} for LDB.

# Introduction

At the JVET 22nd meeting, a method of intra template matching prediction (TMP) was adopted in the EE base software. This coding tool was first introduced in JVET-U0048[1] and then further tested in JVET-V0130[2]. It was reported that this is a coding tool that is mostly adapted for screen content coding. As illustrated in Figure 1 below, TMP is a special intra prediction mode that copies the best prediction block from the reconstructed part of the current frame, whose L-shaped templated matches the current template. Within a predefined search range, a prediction signal is generated at both the encoder and the decoder side by matching the L-shaped causal neighbor of the current block with another block in a predefined search area. The usage of this tool is signalled at CU level through a dedicated flag.

Figure 1. intra template matching search area used in JVET-V0130

The decoder-side intra mode derivation (DIMD) was proposed in JVET-U0100[3] and is also included in the ECM-2.0 software. When DIMD is applied, two intra prediction modes are derived from the reconstructed neighbor samples, and these two predictors are further blended by using the predictors of planar mode. The weights of these predictors are derived from their corresponding gradients’ amplitudes. A dedicated flag at CU level is signalled in the bitstream whether DIMD is applied or not.

The simplified syntax table of the current design is shown as follow.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| pred_mode_flag | ae(v) |
| … |  |
| if ( sps_dimd_enable_flag ) |  |
| cu_dimd_flag | ae(v) |
| … |  |
| if( sps_tmp_enable_flag && cbWidth <= TMP_MaxSize && cbHeight <= TMP_MaxSize ) |  |
| intra_tmp_flag | ae(v) |
| if ( !intra_tmp_flag && !cu_dimd_flag ){ |  |
| if( sps_mip_enabled_flag ) |  |
| intra_mip_flag | ae(v) |
| if( intra_mip_flag ) { |  |
| intra_mip_transposed_flag[ x0 ][ y0 ] | ae(v) |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if ( sps_timd_enable_flag && (sh_slice_type != I ¦¦ (sh_slice_type == I && cbWidth * cbHeight <=1024 ))) |  |
| intra_timd_flag | ae(v) |
| if( sps_mrl_enabled_flag && (( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx | ae(v) |
| … |  |
| } |  |
| } |  |
| } |  |

In the current design, the TMP flag will be always signalled in the bitstream regardless of DIMD is signalled or not.

# Proposed Method

This contribution proposes to skip signalling the TMP flag while DIMD is executed at the current CU.

The simpified syntax table of the proposed method is shown as follow.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| pred_mode_flag | ae(v) |
| … |  |
| if ( sps_dimd_enable_flag ) |  |
| cu_dimd_flag | ae(v) |
| … |  |
| if( sps_tmp_enable_flag && cbWidth <= TMP_MaxSize && cbHeight <= TMP_MaxSize && !cu_dimd_flag ) |  |
| intra_tmp_flag | ae(v) |
| if ( !intra_tmp_flag && !cu_dimd_flag) { |  |
| if( sps_mip_enabled_flag ) |  |
| intra_mip_flag | ae(v) |
| if( intra_mip_flag ) { |  |
| intra_mip_transposed_flag[ x0 ][ y0 ] | ae(v) |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if ( sps_timd_enable_flag && (sh_slice_type != I ¦¦ (sh_slice_type == I && cbWidth * cbHeight <=1024 ))) |  |
| intra_timd_flag | ae(v) |
| if( sps_mrl_enabled_flag && (( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx | ae(v) |
| … |  |
| } |  |
| } |  |
| } |  |

# Experimental Results

The proposed flag harmonization method is implemented in ECM-2.0. The experimental results are tested under the ECM-2.0 common test condition [3] with all intra (AI), random access (RA) and low delay B (LDB) configurations.

Tables below show the test results of the proposed method under common test conditions.

Anchor: ECM-2.0

Test: ECM-2.0 + the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusion

This contribution proposes a harmonization method for the TMP and DIMD flags, which aims to further improve the coding efficiency by removing the redundant signalling bits of TMP flag. Compared with ECM-2.0, the proposed method can bring x%, x% and x% coding gains of class F for AI, RA and LDB, respectively, without running time increasing. It is suggested to adopt this modification into the next version of ECM software.

# References

- K. Naser, et. al., “Evaluation of Template Matching Prediction for VVC”, JVET-U0048, Jan. 2021.

- K. Naser, et. al., “EE2: Intra Template Matching”, JVET-V0130, Apr. 2021.

- Y.-J. Chang, et. al., “Compression efficiency methods beyond VVC,” JVET-U0100, Jan. 2021.

- M.Karczewicz, Y.Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).