| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0131-v1 |
| --- | --- |

| Title: | Non-EE2: Low-Complexity Improvements of Intra Coding for Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Tae Meon Bae, Alexey Filippov, Vasily Rufitskiy, Young-Yoon Lee, Kalyan Goswami | Tel: Email: | duizcoll@ofinno.com, taemeon.bae@ofinno.com, afilippov@ofinno.com, vrufitskiy@ofinno.com, yylee@ofinno.com, kgoswami@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution proposes (1) a sign prediction of the IBC block vector difference method and (2) an MRLP modification for screen content coding. On top of ECM-2.0, the test results for the 1st technique reportedly show the class F compression performance improvements of -0.09%, -0.08%, -0.08% (Y/U/V) and x.xx%, x.xx%, x.xx% (Y/U/V) for AI and RA configurations, respectively. On top of ECM-2.0, the test results for the 2nd technique reportedly show the class F compression performance improvements of -0.06%, -0.04%, -0.14% (Y/U/V) and x.xx%, x.xx%, x.xx% (Y/U/V) for AI and RA configurations, respectively. The joint test results for both techniques reportedly show the class F / class TGM compression performance improvements -0.15%, -0.13%, -0.22 % (Y/U/V) and x.xx%, x.xx%, x.xx% (Y/U/V) for AI and RA configurations, respectively. The joint test results for both techniques reportedly show the class TGM compression performance improvements -0.21%, -0.14%, 0.19 % (Y/U/V) and x.xx%, x.xx%, x.xx% (Y/U/V) for AI and RA configurations, respectively.  Changes of encoding and decoding runtime for all the tests are reported to be negligible.

# Introduction

In VVC, the reference region for the IBC prediction mode is constrained to reduce the on-chip memory size and consumption [1]. The nature of the on-chip memory allows immediate access to the reconstructed samples of the neighbor CTUs for IBC prediction. A virtual buffer is introduced in VVC, which has a height equal to the CTU height (ctbSize) and a width of 256*128/ctbSize. The same virtual buffer concept is adopted in ECM 2.0 [2], increasing its size fourfold to accommodate a larger CTU size of 256x256.

The IBC reference region comprises the already encoded block in the current CTU and one or more left neighbor CTUs to the current CTU, depending on the CTU size. In order to enable friendly memory reuse, the reference area in the neighbor CTUs is constrained according to the VPDU size and the current encoding block location in the current CTU.

In IBC AMVP prediction mode, the block vector (BV) is signaled as an index to the merge/AMVP candidate list of block vector predictors (BVP) and a block vector difference (BVD). The BVD is implicitly signaled as a module of each of its components and their respective signs.

This proposal exploits the IBC reference region constrain to determine the components sign in the decoder side without explicit signaling when certain conditions are met.

Another technique used in VVC for improving compression performance of intra prediction is Multiple Reference Line Prediction (MRLP) [3]. Figure 1 illustrates a block to be predicted and a set of its 3 reference lines, one of which can be used as a reference. It is worth noting that the option of selecting a reference line out of this set is available only for intra prediction modes in an MPM list (a primary MPM list in the case of ECM [2]).

Figure 1. MRLP adopted into VVC

# Proposed methods

In this contribution, two low complexity methods for screen content coding are proposed:

- the 1st technique is based on the sign prediction of the vector differences in the IBC prediction mode,

- the 2nd technique is an MRLP modification.

## Proposed BVD sign prediction

In this proposal, the BVD components signs are derived as follows:

- Four BV candidates as build using the different sign combinations of the module of the BVD components:

- BVA= xBVP,yBVP + +|xBVD|,+|yBVD|

- BVB= xBVP,yBVP + +xBVD,-yBVD

- BVC= xBVP,yBVP + -|xBVD|,+|yBVD|

- BVD= xBVP,yBVP + -xBVD,-|yBVD|

- The four BV candidates are validated by checking them in the IBC buffer. Figure 1 illustrates a simplified example of four BV candidates where only one candidate, BVA, is validated in the IBC buffer. Candidate BVB is pointing to the non-valid reference region in the neighbor CTU, that is, the VPDU equivalent to the VPDU that the current coding block belongs to. The BVC and BVD candidates are pointing outside of the IBC buffer and consequently are invalid.

Figure 2. Example of all possible combinations of BV candidates

- Based on the validities of the BV candidates, the BVD signs can be derived. In the case of only one BV candidate is valid, the decoder can derive both signs. If two BV candidates are valid, one sign can be derived, and the other component sign is signaled. In the other case, if three BV candidates are valid, at least one component sign is signaled, and the other sign may be derived based on the sign of the previous component sign. Finally, if the four BV candidates are valid, both signs are explicitly signaled.

Table 1 illustrates an example showing all possible combinations of valid BV candidates among the four BV candidates and the sign derivation for each component of the BVD, denoted as xSign and ySign for the horizontal and vertical components, respectively.  A "0" in the condition columns indicates that the corresponding BV candidate is not valid, and a "1" that the BV candidate is valid. An "X" in the component sign columns indicates that the sign cannot be derived, and it is explicitly signaled in the bitstream, and "+" and "-" values indicate that positive or negative component sign is derived, respectively.

Table 1. BV candidate validities

*** means a non-valid combination because at least one candidate must be valid

## Proposed MRLP modifications

In this contribution, we propose a modification of the MRLP method. In contrast to the MRL prediction adopted into VVC, we adaptively adjust offsets between reference lines subject to block size as shown in Figure 2. In the proposed MRLP design, the number of reference lines and the syntax are kept unchanged. Just the semantics and reference sample processing were modified to enable these offsets between reference lines.

Figure 3. Example of the proposed MRLP modification

# Simulation results

The proposed methods have been implemented on top of the ECM-2.0 [2]. The test simulations were conducted using the Common Test Conditions as defined in [4]. Testing results under AI and RA configurations are summarized in Table 2 to Table 5. More detailed results can be found in the Excel spreadsheets accompanied.

Table 2. : BD-rate performance results on joint IBC BVD sign prediction and MRL proposals

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.09% | -0.08% | -0.08% | 101% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3. BD-rate performance results of the proposed IBC BVD sign prediction

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 4. BD-rate performance results of the proposed MRLP modification

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | -0.06 | -0.04% | -0.14% | 97% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 5. BD-rate performance results of the joint test for the IBC BVD sign prediction and the proposed MRL modification

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.15% | -0.13% | -0.22% | 98% | 100% |
| Class TGM | -0.21% | -0.14% | 0.19% | 100% | 91% |

# Conclusion

This contribution proposes a sign prediction of the IBC block vector difference method and an MRLP modification for screen content coding. The test results report that they can further improve the coding efficiency of ECM-2.0 with negligible impact on encoding and decoding run-time. We recommend to study both techniques in EE2.

# References

- Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang, "Non-CE8: Reference memory reduction for intra block copy ", JVET-N0250, March 2019.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- B. Bross, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, L. Zhao, X. Zhao, X. Li, S. Liu, Y.-J. Chang, H.-Y. Jiang, P.-H. Lin, C.-C. Kuo, C.-C. Lin, C.-L. Lin, "CE3: Multiple reference line intra prediction (Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4)", JVET-L0283, October 2018.

- Marta Karczewicz, Yan Ye, 	"Common test conditions and evaluation procedures for enhanced compression tool testing," JVET-W2017, July 2021.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).