| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0132-v1 |
| --- | --- |

| Title: | EE2-related: On MVD sign prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Zhang, Bappaditya Ray, Han Huang, Vadim Seregin, Marta Karczewicz | Email: | yzh@qti.qualcomm.com bray@qti.qualcomm.com hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results of motion vector difference sign prediction, which is initially proposed in JVET-K0067, to further include affine mode, SMVD mode, MMVD mode and affine MMVD mode. The proposed method was implemented on top of ECM-2.0, and it reports -0.11%, -0.11%, -0.10% BD rate saving for luma and chroma components, respectively, with 100% encoder and 99% decoder run time in random access configurations and -0.xx%, -0.xx%, -0.xx% BD rate saving for luma and chroma components, respectively, with xxx% encoder and xxx% decoder run time in low delay B configurations of common test condition.

# Introduction

The general idea of MVD sign prediction is to derive certain cost given hypotheses MVDs and output the correct MVD sign prediction index by comparing the true MVD with the MVD list that is sorted based on the cost. In this way, instead of directly signaling MVD signs in EP mode, the MVD sign prediction index which represents the correct MVD signs is coded with context model. The conversion from EP coding to context-based coding provides gain.

# Proposed MVD sign prediction

Motion vector difference sign prediction can be applied in regular inter mode if the motion vector difference contains non-zero component. Possible MVD sign combinations are sorted according to template matching cost and index corresponding to the true MVD sign is derived and coded with context model. At decoder side, the MVD signs are derived as following:

- Parse the magnitude of MVD components

- Parse context-coded MVD sign prediction index

- Build MV candidates by creating combination between possible signs and absolute MVD value and add it to the MV predictor

- Derive MVD sign prediction cost for each derived MV based on template matching cost and sort

- Use MVD sign prediction index to pick the true MVD sign

- Add the true MVD to the MV predictor for final MV

Bilinear filter is used in reference template generation. The template matching cost is measured by the SAD between the neighbouring samples of the current CU and their corresponding reference samples, as illustrated in Figure 1.

Figure 1 template matching cost derivation in MVSD[1]

Several other coding tools such as inter SMVD mode, affine mode, MMVD mode has been introduced during VVC standardization. In ECM [2], affine MMVD mode is additionally introduced. All these tools are either explicitly signaling the MVD signs that are part of MVD or implicitly signaling MVD signs through direction index. This indicates that MVSD could also be applied to these coding tools.

When MVD sign prediction is applied to affine and affine MMVD mode, sub-block based template is used and the template matching cost for each sub-block are accumulated for the final cost. The template matching cost derivation for affine and affine MMVD modes are depicted in Figure 2.

Figure 2 affine template matching cost derivation in MVSD[3]

In case of bi-prediction in SMVD, MMVD and affine MMVD mode, to reduce complexity, only the template from reference list 0 is used to derive the template matching cost.

When coding the MVD sign prediction index, different contexts are used depending on the magnitude of the MVD component. A pre-defined threshold is used to compare with the MVD component magnitude. Different threshold is used for translational inter mode and affine mode.

# Simulation results

The proposed method was implemented on top of the ECM-2.0 [2] .

Test was conducted using the common test conditions as defined in [4]. The BD-rate performance is shown in Table 1, -0.11%, -0.11%, -0.10% BD rate saving for luma and chroma components, respectively, with 100% encoder and 99% decoder run time in RA and -0.xx%, -0.xx%, -0.xx% BD rate saving for luma and chroma components, respectively, with xxx% encoder and xxx% decoder run time in LDB.

Table 1: Test result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.15% | -0.10% | 100% | 99% |
| Class A2 | -0.14% | -0.18% | -0.11% | 100% | 99% |
| Class B | -0.10% | -0.09% | -0.11% | 100% | 99% |
| Class C | -0.10% | -0.04% | -0.08% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.11% | -0.10% | 100% | 99% |
| Class D | -0.10% | -0.14% | 0.08% | 101% | 99% |

|  | Over ECM-2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.23% | -0.26% | -0.02% | 104% | 103% |
| Class E | -0.06% | -0.11% | 0.09% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.18% | -0.58% | -0.56% | 103% | 104% |

# Conclusion

In this contribution, motion vector difference sign prediction is proposed. It’s reported in the test results that it can further improve the coding efficiency of ECM-2.0 with almost no complexity increase. It’s suggested to study this technique in EE.

# References

- Sergey Ikonin, Jianle Chen, “CE9: Motion vector difference signs derivation (Test 4.2)”, JVET-K0067, July 2018.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- Chun-Chi Chen, Vadim Seregin, Yao-Jen Chang, Zhi Zhang, Han Huang, Yan Zhang, Marta Karczewicz, “EE2-related: Extension of template matching to Affine, CIIP, GPM merge modes, and boundary sub-blocks”, JVET-V0118, April 2021.

- Marta Karczewicz, Yan Ye	“Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).