| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0136-v1 |
| --- | --- |

| Title: | AHG8: On significance, GT1, and GT2 flag coding for high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document describes a study of the use of significance, GT1 and GT2 flags in regular residual coding at low QP and lossless operating points for bit depths beyond 10 bit. An analysis using VTM 14.0 is provided which indicates the fraction of these flags that are set for a range of different QPs within and beyond the low QP high bit depth CTC. This analysis outlines how as QPs decrease, the fraction of the flags that are set increases to almost 100%. This results in the number of coefficients at least partially coded using flags reduces as QPs become smaller. A modification is described which effectively shifts the significance, GT1 and GT2 tests left by a number of bits signalled from the encoder. A non-normative algorithm is also described which computes a suitable value for this shift in a pre-encode step. The analysis, previously done for VTM 14.0, is repeated for the modification and indicates that the fraction of flags that are set does not rise as QPs decrease, unlike VTM 14.0. Consequently the number of coefficients coded using the significance, GT1 and GT2 flags is maintained.

The following results are reported for the modification against an anchor of VTM 14.0:

| Lossless | AI | LDB | RA |
| --- | --- | --- | --- |
| PQ | -0.20% | -0.24% | -0.24% |
| HLG | -0.83% | -0.92% | -0.92% |
| SVT-12 | -1.17% | -1.35% | -1.37% |
| SVT-16 | -1.18% | -1.51% | -1.53% |

| Low QP | AI |  |  | LDB |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | U/B | V/R | Y/G | U/B | V/R | Y/G | U/B | V/R |
| PQ | -0.01% | 0.02% | 0.02% | 0.02% | 0.01% | 0.03% | 0.02% | 0.02% | 0.03% |
| HLG | -0.07% | -0.08% | -0.08% | 0.02% | 0.02% | 0.02% | 0.02% | 0.03% | 0.04% |
| SVT-12 | -0.20% | -0.19% | -0.21% | -0.05% | -0.05% | -0.05% | -0.04% | -0.02% | -0.02% |
| SVT-16 | -0.75% | -0.77% | -0.78% | -0.76% | -0.73% | -0.73% |  |  |  |

# Introduction

JVET-V0122 [1] proposed a bypass mode for coefficient coding which when enabled removed the context coded bin allowance for coefficient coding. When this mode is enabled, the significance, GT1, GT2 and parity flags are not used for both RRC and TSRC. As expected this resulted in losses for the high bit depth CTC [2] for most content over the standard and low QP ranges. However, for SVT-16 content at low QPs, and both HLG and SVT content in the lossless CTC, small gains were reported when comparing results generated using the anchor of the V CE. These gains indicate that at these operating points the coding of coefficient flags offers no benefit.

JVET-W0093 [3] provided an analysis of the usage of significance, GT1 and GT2 flags which indicated at low QPs the fraction of these flags that are set approaches 100%. This leads to a reduction in the number of coefficients that can be coded using these flags. JVET-W0093 also introduced a new technique for regular residual coding which introduced an effective left shift of these flags to reduce the fraction that are set and thus increase the number of coefficients coded using significance, GT1 and GT2.

This contribution reports further analysis and updates to the technique reported in JVET-W0093.

# Analysis of VTM14.0

As previously described in JVET-W0093, the percentage of the significance (CoeffMap), GT1 and GT2 flags that are set to 1 increases as QP decreases until the percentages for all three flags reach almost 100%. figures 1 and 2 shows the ratios observed using VTM14.0 and the HBD CTC with increased QP ranges and TransformSkip disabled.

Figure 1. Percentage of flags set for OldTownCross processed at 12 bit AI using VTM14.0

Figure 2. Percentage of flags set for OldTownCross processed at 16 bit AI using VTM14.0

One of the effects of these high ratios is that the number of coefficients at least partially coded using flags is reduced from a peak at QP 22 as shown in figure 3. When ratios are high the number of significance, GT1, parity and GT2 flags become almost equal. As these flags all contribute to a fixed maximum, an increase in the number of GT1 and GT2 flags leads directly to a reduction in the number of significance flags that can be coded.

Figures 3 and 4 also show how the number of coefficients coded without using Golumn-Rice, i.e. completely coded using flags, reduces as QPs decrease. At low QPs almost all coefficients require Golomb-Rice coding and the benefits observed for flag coding at normal VVC v1 operating points are no longer present at low QPs.

Figure 3. Number of coefficients coded in a single frame for OldTownCross processed at 12 bit using VTM14.0

Figure 4. Number of coefficients coded in a single frame for OldTownCross processed at 16 bit using VTM14.0

It is suggested that at the QPs used in the low QP range of the CTC, the individual bins are coded with increased efficiency as the QP is reduced, but the efficiency of the coding of the actual coefficient values decreases as more and more values need to be escape coded. Thus as QPs decrease the relative information about the coefficient values provided by the significance, GT1 and GT2 flags also decreases.

As a corollary to the greater number of coefficients requiring Golomb-Rice coding and the increased size of each coefficient, the number of escape coded bins increases. Figures 5 and 6 show that whilst neither the number of context-coded bins nor the number of non-escape coded EP bins increases significantly below VVC v1 operating points, the number of escape coded bins increases at an almost constant rate. It is suggested that at low QPs a large proportion of the workload on a VVC decoder is spent decoding Golomb-Rice values.

Figure 5. Number of bins coded for single frame for OldTownCross processed at 12 bit using VTM14.0

Figure 6. Number of bins coded for single frame for OldTownCross processed at 16 bit using VTM14.0

# Proposal

The aim of the proposal introduced in JVET-W0093 was to improve the coding of coefficients using context-coded bins in RRC for low (and negative) QPs at bit depths beyond 10 bit. In addition this improvement also applies to high bit depth lossless coding in VVC which at present gives only small gains over HEVC.

The technique chosen attempts to reduce the percentage of significance, GT1 and GT2 flags that are set to 1, more closely matching the percentages reported at VVC v1 operating points. By making better use of these flags this technique should allow more coefficients to be coded without requiring Golomb-Rice coding.

The effect of the technique is to shift left the significance, GT1, GT2 and parity tests used in regular residual coding by n bits as shown in in figure 7 and table 1, with a shift of 0 being equivalent to VVC v1.

Figure 7. Significance test shifted left by n bits.

| Shift | Flag |  |  |  |
| --- | --- | --- | --- | --- |
|  | Sig Map | GT1 | GT2 | Parity |
| 0 | value ≥ 1 | value ≥ 2 | value ≥ 4 | (value & 1) ≠ 0 |
| 1 | value ≥ 2 | value ≥ 4 | value ≥ 8 | (value & 2) ≠ 0 |
| 2 | value ≥ 4 | value ≥ 8 | value ≥ 16 | (value & 4) ≠ 0 |
| .. |  |  |  |  |
| n | value ≥ 2n | value ≥ 2n+1 | value ≥ 2n+2 | (value & 2n) ≠ 0 |

Table 1. Tests for flags with shift left of n bits applied

Thus every shift left of a bit doubles the thresholds for the tests which causes a reduction in the number of coefficients passing each test. Using a suitably chosen value for n, the percentages of coefficients passing these tests at low QPs can be made similar to those for higher QPs.

Rather than actually changing the sub-block coding of residuals and the significance, GT1, GT2 and parity tests used within them, the proposal is implemented by introducing a shift left of n bits of all coded values at the conclusion of residual coding for a block. The sub-block coded value thus becomes the MSBs of the full coefficient value. Once all sub-blocks are coded, the lowest n bits of the value of each coefficient in the block are then coded using EP-bins. An additional EP-bin is used to code the sign bit for coefficients with non-zero LSBs and an MSB of zero. The LSBs and MSBs are combined to give full coefficient values at the end of the transform block.

If n is non-zero, before any sub-block or the last x-y position is coded, a context coded bin is used to signal if this is a significant block. If set this flag indicates that at least one coefficient exists in the block which has a non-zero MSB and thus sub-block coding is required. If the flag is not set this indicates that all MSBs are zero, and coefficient values can be derived from the LSBs and sign bits only. In this case, the coding of the last x-y position and the sub-blocks is skipped. In the proposal, the context coded bin used for the new significant block flag is counted in the number of context coded bins for the block. Therefore there is no increase in the maximum number of context-coded bins allowed for residual coding.

An additional advantage of the proposal is the shift left of the parity flag. Using the existing VVC coding scheme this flag is effectively coding noise at QPs below 0 as below this point  the lowest bit does not typically contain structural information. This can be observed by analyzing the efficiency of parity flag coding in VTM which typically uses more than one bit to code the parity flag at low QPs. With the modification in the proposal, the parity flag has been found to require less than one bit, making context coding worthwhile.

# Determination of the value of n

The value of n, the size of the shift left of the significance test, is signalled in the slice header. Separate shift values are used for luma and chroma values. The shift values are constant for the slice and not varied in the decoder.

For some applications the values of n could be set using domain-specific rules, but for this proposal, a generic algorithm has been developed. This determines the values of n using an estimate of the residual calculated in a pre-encode phase. This allows the shift values to be signalled for the frame being currently being processed rather than the following frame unlike other approaches [4][5] which are embedded in the actual coefficient coding phase of the encoder. One advantage of using this pre-encode step is that it may be applied to still pictures and handles events such as scene changes better than techniques which signal a value based on a previous frame.

The first phase of the algorithm computes an estimate of the residual for each pixel which is defined as the minimum absolute difference between the actual pixel value for the channel and two estimates of the pixel value. These two estimates are the values of the pixels immediately to the left and immediately above.

Once each residual value is computed this is used to choose one of 96 (16 bit content) or 72 (12 bit content) histogram bins using the following formula, and the number of entries in that bin is incremented:

bin = residual > 2 ? round(log2(residual - 2)) : 0

Once this bin is calculated, a QP adjustment may be computed using the bit depth content and the slice QP as follows:

baseQP = 1 – (bitdepth -8) * 6

adjustment = sliceQP – baseQP

adjustedBin = max(0, bin - adjustment)

The effect of this change can be seen as a histogram and a cumulative frequency in figure 8.

Figure 8. Histogram and cumulative frequency for CrowdRun (16 bit AI) with and without QP adjustment.

Note: for chroma, the histogram and cumulative frequency contain entries from both chroma channels.

From the cumulative frequency the value of n is computed using three percentiles of the cumulative frequency as shown in figure 9.

Figure 9. Computation of n from the cumulative frequency

The algorithm for computing the value of n from the histogram is given below:

targetB = total / 2;   // total is number of entries in histogram

targetA = targetB - total / 8;

targetC = targetB + total / 8;

a = -1;

b = -1;

current = 0;

for (i = 0; i < 96; i++)  {

current += hist[i] // hist is the histogram computed for the slice/frame

if (current >= targetA && a == -1)    { a = i } // reached intercept a

if (current >= targetB && b == -1)    { b = i } // reached intercept b

if (current >= targetC) {                                   // reached intercept c

c = i

n = (((a + 2 * b + c) / 4) / 6)

break

}

}

As the determination of n is non-normative this algorithm is just one example method of determining n.

# Analysis of Proposal

The aim of this proposal is to improve the coding of coefficient values, and it can be seen that for this sequence (OldTownCross), the technique leads to gains at all QPs where a shift is selected by the algorithm described above.

| QP | Shift n |  |  | Reference |  |  |  | Tested |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma/G | Chroma/B+R | bits | G psnr | B psnr | R psnr | kbps | G psnr | B psnr | R psnr |
| -38 | 8 | 8 | 76487304 | 99.76 | 98.00 | 98.05 | 75696000 | 99.63 | 98.31 | 98.36 |
| -33 | 7 | 7 | 69942168 | 95.63 | 95.02 | 95.02 | 69342016 | 95.61 | 95.02 | 95.03 |
| -28 | 6 | 6 | 64544168 | 90.85 | 90.08 | 90.10 | 63999464 | 90.80 | 90.09 | 90.10 |
| -23 | 5 | 6 | 59203944 | 85.78 | 85.15 | 85.15 | 58604160 | 85.75 | 85.14 | 85.14 |
| -18 | 4 | 5 | 53917992 | 80.85 | 80.09 | 80.09 | 53373464 | 80.82 | 80.06 | 80.06 |
| -13 | 3 | 4 | 48722088 | 75.86 | 75.12 | 75.12 | 48232272 | 75.79 | 75.09 | 75.10 |
| -8 | 3 | 3 | 43559752 | 70.90 | 70.15 | 70.16 | 43071648 | 70.82 | 70.10 | 70.10 |
| -3 | 2 | 2 | 38361160 | 65.95 | 65.11 | 65.13 | 38089096 | 65.78 | 64.95 | 64.92 |
| 2 | 1 | 2 | 34170136 | 61.78 | 61.08 | 61.09 | 33896016 | 61.60 | 60.82 | 60.84 |
| 7 | 0 | 1 | 28944824 | 56.74 | 56.04 | 56.07 | 28792160 | 56.75 | 55.69 | 55.68 |

Table 2. Analysis of result for proposal (OldTownCross 16bit AI – TS disabled)

If the bin usage is analysed in the same way as for VTM14.0, the percentage of bins set to 1 of each type (significance, GT1 and GT2) is kept within bounds and does not approach 100% throughout the range of QPs for both 12 and 16 bit. This is shown for both 12 and 16 bit processing in figures 10 and 11.

Figure 10. Percentage of flags set for OldTownCross processed at 16 bit AI using proposed technique

Figure 11. Percentage of flags set for OldTownCross processed at 16 bit AI using proposed technique

As the percentages of bins set to 1 is now more consistent across the QP range, the number of coefficients coded completely using flags and without the need for Golumb-Rice escape coding is also maintained across the QP range. The total number of coefficients coded using flags is also increased due to the reduced proportion of GT1 and GT2 flags. This can be seen in figures 12 and 13. The number of coefficients that require Golumb-Rice coding in addition to flags is thus reduced and kept similar for all QPs. This implies that the workload imposed by coding Golumb-Rice values is also similar at all QPs. Although coefficients now require LSBs to be coded it is suggested that the coding of LSBs presents less of a workload to the decoder than coding Golumb-Rice values.

Figure 12. Number of coefficients coded in a single frame for OldTownCross processed at 12 bit using proposed technique

Figure 13. Number of coefficients coded in a single frame for OldTownCross processed at 16 bit using proposed technique

The number of bins of different types coded in a single frame has also been analysed. For VTM, figures 5 and 6 showed that the number of escape (Golumb-Rice) coded bins increases as QPs decrease. For the proposed technique, figures 14 and 15 show that the number of escape coded bins remains broadly similar at all low QPs. Instead the number of LSB coded bins increases as QPs decrease.

Figure 14. Number of bins coded for single frame for OldTownCross processed at 12 bit using proposed technique

Figure 15. Number of bins coded for single frame for OldTownCross processed at 16 bit using proposed technique

# Results

This section reports results for the proposal against an anchor of VTM14.0.

Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM14.0 | SigGt1Gt2Shift |  | VTM14.0 | SigGt1Gt2Shift |  | VTM14.0 | SigGt1Gt2Shift |  |
| PQ444 | 2.6 | 2.6 | -0.17% | 3.1 | 3.1 | -0.21% | 3.1 | 3.1 | -0.20% |
| PQ422 | 2.4 | 2.4 | -0.23% | 2.9 | 2.9 | -0.26% | 2.9 | 2.9 | -0.28% |
| Overall | 2.5 | 2.5 | -0.20% | 3.0 | 3.0 | -0.24% | 3.0 | 3.0 | -0.24% |
| Enc Time[%] | 96% |  |  | 98% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 101% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM14.0 | SigGt1Gt2Shift |  | VTM14.0 | SigGt1Gt2Shift |  | VTM14.0 | SigGt1Gt2Shift |  |
| HLG444 | 1.8 | 1.8 | -0.80% | 2.0 | 2.0 | -0.86% | 2.0 | 2.0 | -0.86% |
| HLG422 | 1.7 | 1.7 | -0.85% | 1.9 | 2.0 | -0.99% | 1.9 | 2.0 | -0.98% |
| Overall | 1.7 | 1.8 | -0.83% | 2.0 | 2.0 | -0.92% | 2.0 | 2.0 | -0.92% |
| Enc Time[%] | 101% |  |  | 102% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  | 99% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM14.0 | SigGt1Gt2Shift |  | VTM14.0 | SigGt1Gt2Shift |  | VTM14.0 | SigGt1Gt2Shift |  |
| SVT16 | 1.2 | 1.2 | -1.17% | 1.2 | 1.2 | -1.35% | 1.2 | 1.2 | -1.37% |
| SVT12 | 1.3 | 1.3 | -1.18% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.53% |
| Overall | 1.2 | 1.3 | -1.17% | 1.3 | 1.3 | -1.43% | 1.3 | 1.3 | -1.45% |
| Enc Time[%] | 102% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 100% |  |  |

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | 0.02% | 0.01% | -0.01% | 0.02% | 0.01% | 104% | 99% |
| PQ422 | 0.00% | 0.02% | 0.02% | -0.01% | 0.02% | 0.02% | 105% | 101% |
| Overall | -0.01% | 0.02% | 0.02% | -0.01% | 0.02% | 0.02% | 105% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | 0.01% | 103% | 100% |
| PQ422 | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% | 0.05% | 103% | 98% |
| Overall | 0.02% | 0.01% | 0.03% | 0.01% | 0.01% | 0.03% | 103% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.01% | 0.02% | 0.01% | 0.01% | 0.03% | 103% | 99% |
| PQ422 | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 103% | 100% |
| Overall | 0.02% | 0.02% | 0.03% | 0.02% | 0.02% | 0.03% | 103% | 100% |
| Overall PQ | 0.01% | 0.02% | 0.02% | 0.01% | 0.02% | 0.02% | 104% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.07% | -0.07% | 105% | 100% |
| HLG422 | -0.08% | -0.10% | -0.10% | 106% | 100% |
| Overall | -0.07% | -0.08% | -0.08% | 105% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.02% | 104% | 101% |
| HLG422 | 0.02% | 0.03% | 0.01% | 103% | 99% |
| Overall | 0.02% | 0.02% | 0.02% | 103% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.03% | 0.03% | 104% | 100% |
| HLG422 | 0.02% | 0.04% | 0.04% | 103% | 100% |
| Overall | 0.02% | 0.03% | 0.04% | 104% | 100% |
| Overall HLG | -0.01% | -0.01% | -0.01% | 104% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.75% | -0.77% | -0.78% | 100% | 101% |
| SVT12 | -0.20% | -0.19% | -0.21% | 107% | 102% |
| Overall | -0.47% | -0.48% | -0.49% | 104% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.76% | -0.73% | -0.73% | 103% | 106% |
| SVT12 | -0.05% | -0.05% | -0.05% | 105% | 103% |
| Overall | -0.39% | -0.32% | -0.33% | 104% | 105% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 |  |  |  |  |  |
| SVT12 | -0.04% | -0.02% | -0.02% | 107% | 104% |
| Overall |  |  |  |  |  |
| Overall RGB |  |  |  |  |  |

Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.05% | 0.04% | 0.04% | 0.04% | 0.05% | 0.04% | 0.04% | 0.05% | 101% | 99% |
| Class H2 |  |  |  |  |  | 0.05% | 0.06% | 0.06% | 101% | 99% |
| Overall | 0.05% | 0.04% | 0.04% | 0.04% | 0.05% | 0.05% | 0.05% | 0.05% | 101% | 99% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 105% | 102% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 104% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 104% | 102% |

The losses reported for the standard QP range are entirely down to the signalling cost. It is suggested that the modification proposed in this document is only suitable for use at low QPs and would only be available in the Main 12 4:4:4 and Main 16 4:4:4 profiles and not the Main 12 profile.

# Draft text changes

# Change clause 7.3.7 (Slice header syntax) as follows (additions are yellow-highlighted):

| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| --- | --- |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if( !sh_ts_residual_coding_disabled_flag && sps_ts_residual_coding_rice_present_in_sh_flag ) |  |
| sh_ts_residual_coding_rice_idx_minus1 | u(3) |
| if( pps_slice_header_extension_present_flag ) { |  |
| sh_slice_header_extension_length | ue(v) |
| slice_flagshift_luma | u(4) |
| slice_flagshift_chroma | u(4) |

Change clause 7.3.11.11 (Residual coding syntax) as follows (additions are yellow-highlighted):

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| flagShift = cIdx == 0 ? slice_flagshift_luma : slice_flagshift_chroma |  |
| if (flagShift > 0) { |  |
| b_coded_flag | ae(v) |
| } |  |
| if (b_coded_flag) { |  |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth = = 5 && log2TbHeight < 6 ) |  |
| log2ZoTbWidth = 4 |  |
| Else |  |

…

| log2TbWidth = log2ZoTbWidth |  |
| --- | --- |
| log2TbHeight = log2ZoTbHeight |  |
| remBinsPass1 = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| if (flagShift > 0) { |  |
| remBinsPass1-- |  |
| } |  |
| log2SbW = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| log2SbH = log2SbW |  |

…

| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| --- | --- |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if (flagShift > 0) { |  |
| for (i = 0; i < lastSubBlock; i++) { |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ][ j ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ][ j ][ 1 ] |  |
| for (int n = 0; n < numSbCoeff; i++) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| sign = 0 |  |
| if (TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] != 0) { |  |
| sign = TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] < 0 ? -1 : 1 |  |
| } |  |
| lsbs_value | u(v) |
| if (lsbs_value != 0 && sign == 0) { |  |
| lsbs_sign | u(1) |
| sign = lsbs_sign == 1 ? -1 : 1 |  |
| } |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = sign * ((abs(TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]) << flagShift) + lsbs_value) |  |
| } |  |
| } |  |
| } |  |
| } |  |  |

# In clause 7.4.8, add the following after the semantics of sh_ts_residual_coding_disabled_flag:

slice_flagshift_luma specifies the length of the syntax element lsbs_value, the number of least significant bits coded separately from the other residual coefficent data in the residual_coding( ) syntax structure, when coding luma data. When not present, the value of slice_flagshift_luma is inferred to be equal to 0.

slice_flagshift_chroma specifies the length of the syntax element lsbs_value, the number of least significant bits coded separately from the other residual coefficent data in the residual_coding( ) syntax structure, when coding luma data. When not present, the value of slice_flagshift_chroma is inferred to be equal to 0.

lsbs_value specifies the least significant bits of the coefficient value. The length of lsbs_value is specified by slice_flagshift_luma for luma data and slice_flagshift_chroma for chroma data.

lsbs_sign specifies the sign of the coefficient value when lsbs_value is non zero and the value of TransCoeffLevel[x0][y0][cIdx][x0][y0] due to sub-block coding is zero. A valueof 1 implies a negative coefficient value and a value of 0 implies a positive coefficient value.

In clause 7.4.12.11, add the following before the semantics of last_sig_coeff_x_prefix:

b_coded_flag specifies the following for the current transform block:

When b_coded_flag is equal to 0, all transform coefficient levels of the current transform block are equal to 0.

When b_coded_flag is not present, it is inferred to be equal to 1.

Change table 51, to add the b_coded_flag (additions are yellow-highlighted)::

| residual_coding( ) | b_coded_flag | Table 120a | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | last_sig_coeff_x_prefix | Table 120 | 0..22 | 23..45 | 46..68 |
|  | last_sig_coeff_y_prefix | Table 121 | 0..22 | 23..45 | 46..68 |
|  | sb_coded_flag[ ][ ] | Table 122 | 0..6 | 7..13 | 14..20 |
|  | sig_coeff_flag[ ][ ] | Table 123 | 0..62 | 63..125 | 126..188 |
|  | par_level_flag[ ] | Table 124 | 0..32 | 33..65 | 66..98 |
|  | abs_level_gtx_flag[ ][ ] | Table 125 | 0..71 | 72..143 | 144..215 |
|  | coeff_sign_flag[ ] | Table 126 | 0..5 | 6..11 | 12..17 |

Insert table 120a

Table 120a – Specification of initValue and shiftIdx for ctxIdx of  b_coded_flag

| Initialization variable | b_coded_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 35 | 35 | 35 |
| shiftIdx | 8 | 8 | 8 |

Change table 127, to add the b_coded_flag  (additions are yellow-highlighted):

| residual_coding( ) | b_coded_flag | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | last_sig_coeff_x_prefix | TR | cMax = ( log2ZoTbWidth << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2ZoTbHeight << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) ) − 1 |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) ) − 1 |
|  | sb_coded_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | par_level_flag[ ] | FL | cMax = 1 |
|  | abs_level_gtx_flag[ ][ ] | FL | cMax = 1 |
|  | abs_remainder[ ] | 9.3.3.11 | cIdx, current sub-block index i, x0, y0, xC, yC, log2TbWidth, log2TbHeight |
|  | dec_abs_level[ ] | 9.3.3.12 | cIdx, x0, y0, xC, yC, log2TbWidth, log2TbHeight |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |

# Conclusions

This document has described a modification to the coding of residual data in which the significance, GT1, and GT2 tests are effectively shifted left by a number of bits and represent the MSBs. The LSBs to the right of the new significance bit are coded separately as EP bins. The MSBs and LSBs are then combined at the transform block level to give the full coefficient values.

A non-normative algorithm has also been described which computes a suitable value for this shift in a pre-encode step. An analysis of the modification has been provided which indicates how the number of significance, GT1 and GT2 flags that are set is reduced when compared with VTM 14.0 .This increases the efficiency of coefficient coding as the flags though less efficiently coded in themselves, give more information about the coefficient value. The reduction in the fraction of set bins also leads to a decrease in the number of coefficients which require Golomb-Rice coding. It is suggested that this reduction in Golomb-Rice coding will result in a reduction in the workload on a decoder as coding LSBs is less costly than coding Golumb-Rice values.

Results are reported which show gains for lossless coding for all classes of content in the high bit depth CTC, and for lossy SVT 12 and 16 bit content in the low QP range of this CTC.

It is proposed that the modification is considered for adoption into the high bit depth extensions for a future version of VVC.

# Patent rights declaration(s)

Sony Group Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions”, JVET-V0122, F, Wang, Z. Xie, Y. Yu, H. Yu, D. Wang

[2] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu

[3] “AHG8: On significance, GT1, and GT2 flag coding for high bit depths”, JVET-W-0093, A. Browne, S. Keating, K. Sharman

[4] “AHG8: on coding of last significant coefficient position for high bit depth and high bit rate extensions”, JVET-V0121, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang

[5] “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-V0054, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang

[6] “VVC operation range extensions (Draft 4)”, JVET-W2005, F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, G. J. Sullivan, Y.-K. Wang