| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0141-v1 |
| --- | --- |

| Title: | EE2-3.1-related: CIIP with template matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2, 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to refine the motion vector of CIIP inter part with a template matching based method. The proposed method was implemented on top of EE2-3.1b, in which the intra-prediction mode of CIIP is implicitly derived by TIMD. Simulation results based on ECM-2.0 are reported as below (tool-off):

RA: -x.xx%, -x.xx%, -x.xx%, xxx%, xxx%; LB: -x.xx%, -x.xx%, -x.xx%, xxx%, xxx%.

# Introduction

The combined inter-picture merge and intra-picture prediction (CIIP) in VVC standard generates a CU prediction by weighted averaging the merge indexed inter-prediction and the planar intra-prediction.

At the last JVET meeting, a template-based intra mode derivation (TIMD) was adopted in ECM-2.0 [1]. At the same meeting, a combination of CIIP and TIMD was proposed to replace the planar mode in CIIP with the intra-prediction mode derived by TIMD method. This method is tested in EE2 as test3.1b [2].

In the ECM-2.0 software, several inter template matching methods are used for higher coding efficiency, such as TM AMVP, regular TM merge mode, GPM-TM mode, and ARMC (adaptive reordering of merge candidates with template matching).

# Proposed Solution

In this contribution, a template matching based motion refinement method, named CIIP-TM mode is proposed to further improve the performance of CIIP based on EE2-3.1b.

With CIIP-TM, a CIIP-TM merge candidate list is built for the CIIP-TM mode. The merge candidates are refined by template matching, which is the same as the regular TM mode in ECM-2.0. The CIIP-TM merge candidates are also reordered by the ARMC method as regular merge candidates in ECM-2.0. In the test, the maximum number of CIIP-TM merge candidates is set equal to two.

# Experimental Results

The proposed method was implemented on top of EE2-3.1b software, and the anchor of the test is ECM-2.0 [3]. The CIIP-TM mode is enabled only under RA configurations. The simulation results under the common test conditions [4] are tabulated in Table 1.

Table 1: Performance of the proposed method (tool-off)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.10% | -0.10% | -0.02% |  |  |  |  |  |  |  |
| Class C | -0.14% | 0.00% | 0.08% |  |  | -0.29% | -0.22% | 0.02% |  |  |
| Class E |  |  |  |  |  | -0.15% | -0.76% | -0.80% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.08% | -0.14% | 0.17% |  |  | -0.11% | -0.27% | 0.00% |  |  |
| Class F | -0.03% | -0.13% | -0.14% |  |  |  |  |  |  |  |

# Conclusions

The proposed method can achieve BD-rate gains with negligible runtime change. It is suggested adopting the proposed method to ECM.

# References

- V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-W2024, Jul. 2021.

- X. Li, R.-L. Liao, J. Chen, Y. Ye, “EE2-3.1: Combination of CIIP and DIMD/TIMD”, KVET-X0098, Oct. 2021

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-2.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, Jul. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).