| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0142-v2 |
| --- | --- |

| Title: | Non-EE2: Extended MRL candidate list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keming Cao, Yao-Jen Chang, Bappaditya Ray, Vadim Seregin, Marta Karczewicz | Tel: Email: | kemicao@qti.qualcomm.com yjchang@qti.qualcomm.com bray@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to extend the number of multiple reference lines (MRL) for intra prediction. This method was implemented on top ECM-2.0 software, it reportedly provides Y, U, V-BD rate reduction with encoder and decoder runtime as follows:

AI: {-0.10%, -0.05%, -0.01%} runtime: {101%, 98%}

RA: {-0.05%, -0.01%, 0.06%} runtime: {101%, 100%}

# Introduction

Multiple reference line (MRL) is used in VVC and ECM, MRL candidate list consists of 2 additional reference lines, i.e., {1, 2}.

# Proposed Method

This contribution proposes to extend MRL candidate lists to include more reference lines. Instead of {1, 2}, MRL candidate list {1, 3, 5, 7, 12} is used as shown in Fig.1. For template-based intra mode derivation (TIMD), instead of the full MRL candidate list, only the first two line candidates, i.e., {1, 3}, are used.

Figure 1. Extended MRL candidate list

# Results

The proposed method is implemented on top of ECM-2.0. Test results for the proposed extended MRL method are summarized the below table.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.04% | -0.06% | 99% | 97% |
| Class A2 | -0.09% | -0.10% | -0.09% | 101% | 99% |
| Class B | -0.12% | -0.04% | -0.06% | 102% | 98% |
| Class C | -0.11% | 0.00% | 0.09% | 102% | 97% |
| Class E | -0.10% | -0.16% | 0.09% | 102% | 97% |
| Overall | -0.10% | -0.05% | -0.01% | 101% | 98% |
| Class D | -0.05% | 0.06% | -0.04% | 102% | 100% |
| Class F | -0.21% | -0.16% | -0.12% | 100% | 96% |
| Class TGM | -0.20% | -0.14% | -0.09% | 99% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.03% | -0.03% | 100% | 99% |
| Class A2 | -0.05% | -0.19% | 0.06% | 99% | 97% |
| Class B | -0.06% | 0.01% | 0.03% | 102% | 101% |
| Class C | -0.04% | 0.07% | 0.14% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.01% | 0.06% | 101% | 100% |
| Class D | 0.00% | -0.24% | 0.23% | 103% | 102% |
| Class F | -0.07% | -0.06% | -0.02 | 101% | 101% |
| Class TGM | -0.11% | -0.10% | -0.12% | 100% | 101% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).