| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0149-v1 |
| --- | --- |

| Title: | AHG12: Removal of floating operations in DMD and TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Hong-Jheng Jhu, W. Chen, C.-W. Kuo, Ning Yan, Yi-Wen Chen, Xianglin Wang | Email: | {xiaoyuxiu, jhuhong-jheng, chenwei06, cheweikuo, yanning03, yiwenchen, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In ECM-2.0, DIMD and TIMD are two decoder-side coding technologies which reduces the intra coding overhead by deriving the intra prediction mode of intra CU at decoder. In the existing DIMD/TIMD design, multiple floating-point operations are involved during the derivation of the optimal intra modes, which are expensive for hardware and software implementations. In this contribution, it is proposed to replace all the floating-point operations in the DMD/TIMD by the existing lookup table used by CCLM, such that only integer additions/multiplications are needed to calculate the DIMD/TIMD parameters. Simulation results reportedly show the BD-rate changes of 0.00% and -0.01% for AI and RA configurations, respectively, without noticeable enc/dec impact.

# Introduction

In ECM-2.0 [1], two decoder-side coding tools, namely decoder-side intra mode derivation (DIMD) and template-based intra mode derivation (TIMD), are introduced to improve the intra coding efficiency. Instead of explicitly signaling intra mode, the two tools derive the information from neighboring reconstructed samples (i.e., template) of one current intra block using different approaches. Specifically, the intra modes in the DIMD are determined by analyzing the histogram of gradients (HoG) within the template samples while the TIMD searches the optimal intra modes by minimizing the SATD between the template samples and the prediction samples obtained from their references.

Additionally, in the ECM-2.0, multiple floating-point operations (e.g., divisions, multiplications and additions) are involved when calculating some intermediate parameters of the DIMD and TIMD, as described as follows:

- The derivation of the gradient orientations in the DIMD: In the HoG analysis, the gradient orientation of each template sample needs to be converted to one of the existing 65 angular intra directions. In the ECM-2.0, floating-point division are applied at sample position to calculate such orientation from its horizontal and vertical gradients, i.e.,

Orient=GyGx

- The prediction sample blending in the DIMD: In the DIMD, the prediction samples generated using the two angular intra modes (that are associated with the largest and the second largest HoG amplitudes) are blended with that of the planar mode. The weights of the two angular modes are determined based on their gradient amplitude, which are derived based on floating-point operations:

w1=23∙Amp1Amp1+Amp2, w2=23-w1

- The prediction sample blending in the TIMD: In the TIMD, the prediction samples generated using the two best intra modes can be blended together to form the final prediction samples of one TIMD block. The weights of the two intra modes are determined based on their SATD values. Similar to the DIMD, floating-point operations are used in such calculation:

w1=SATD2SATD1+SATD2, w2=1-w1

All of those floating-point operations are very expensive for the codec implementations in both hardware and software.

# Proposal

To reduce the implementation cost, it is proposed to replace all the floating-point operations in the DIMD/TIMD by the same lookup table (LUT) based integerization scheme used by the CCLM such that all the above floating-point calculations are approximated by integer multiplications/additions/shifts. Taking the DIMD as example, the following pseudo code illustrates how to derive the gradient orientations based on the LUT-based scheme:

x = Floor( Log2( Gx ) )

normDiff = ( ( Gx<< 4 ) >> x ) & 15

x +=( 3 + ( normDiff  !=  0 ) ? 1 : 0 )

Orient = (Gy* ( DivSigTable[ normDiff ] | 8 ) + ( 1<<( x-1 ) )) >> x

where

DivSigTable[16] = { 0, 7, 6, 5 ,5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }

# Simulation results

The proposed changes were implemented on top of the reference software ECM-2.0 [2] and tested using the common test conditions as defined in [3].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.11% | 100% | 101% |
| Class A2 | -0.01% | -0.05% | 0.04% | 100% | 101% |
| Class B | 0.00% | 0.02% | -0.09% | 100% | 101% |
| Class C | 0.00% | -0.05% | -0.02% | 98% | 99% |
| Class E | -0.01% | -0.09% | 0.09% | 99% | 100% |
| Overall | 0.00% | -0.03% | -0.03% | 100% | 101% |
| Class D | -0.01% | 0.08% | 0.11% | 99% | 96% |
| Class F | -0.01% | -0.07% | -0.03% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | 0.00% | 101% | 102% |
| Class A2 | 0.00% | -0.10% | 0.02% | 101% | 101% |
| Class B | -0.01% | 0.09% | -0.13% | 100% | 102% |
| Class C | -0.02% | 0.06% | 0.01% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | -0.04% | 100% | 101% |
| Class D | -0.01% | -0.34% | -0.03% | 100% | 100% |
| Class F | 0.01% | 0.22% | 0.05% | 100% | 101% |

# References

M. Coban, F. Le Léannec, J. Ström, “Algorithm description of Enhanced Compression Model 2 (ECM2)”, JVET-W2025, July, 2021, Teleconference.

ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July, 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).