| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0169 |
| --- | --- |

| Title: | Crosscheck of JVET-X0048 (CE: Film Grain Synthesis (test CE2.1 and CE2.2)) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao | Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-X0048 on CE2.1. It is confirmed that the crosscheck results match with the results from the proponents. For complexity test, the 4K and 8K sequences are encoded and the MD5 of these bitstreams are confirmed. Both anchor and CE2.1 bitstreams are decoded with SIMD on and SIMD off options. It is reported that for both SIMD on and SIMD off options, more than 30% and 40% decoding time reduction is observed for 4K and 8K sequences, respectively. For subjective test, decoded YUV of anchor and CE2.1 are viewed. Similar visual quality is observed for CE2.1 compared with the anchor.

Simulation results

For complexity test, the decoding runtime shown in Table 1 is obtained by decoding the bitstreams of anchor and CE2.1 three times and averaging the runtime.

Table 1. Simulation results of CE2.1

|  |  | SIMD off |  |  | SIMD on |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Anchor | CE2.1 | Time Saving (%) | Anchor | CE2.1 | Time Saving (%) |
| 4K | FGC_SEI_I10C3 | 7.28 | 5.09 | 30.0% | 3.43 | 2.26 | 34.0% |
|  | FGC_SEI_I10C1 | 4.60 | 3.17 | 31.0% | 2.17 | 1.43 | 34.3% |
|  | FGC_SEI_I1C1 | 4.97 | 3.48 | 29.9% | 2.31 | 1.49 | 35.5% |
| Average |  | 5.62 | 3.92 | 30.2% | 2.64 | 1.73 | 34.5% |
| 8K | FGC_SEI_I10C3 | 28.48 | 14.92 | 47.6% | 13.27 | 6.98 | 47.4% |
|  | FGC_SEI_I10C1 | 18.23 | 10.96 | 39.9% | 8.55 | 5.05 | 40.9% |
|  | FGC_SEI_I1C1 | 19.71 | 11.50 | 41.7% | 9.08 | 5.05 | 44.3% |
| Average |  | 22.14 | 12.46 | 43.7% | 10.30 | 5.69 | 44.7% |