| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0174-v1 |
| --- | --- |

| Title: | Cross-check of JVET-X0137: AHG8 and AHG10: On derivation of sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu | Email: | xiezhihuang@oppo.com yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results for JVET-X0137 which introduce a way to determining the values of two adopted VVC v2coding tools, sh_reverse_last_sig_coeff_flag and sh_ts_residual_coding_rice_idx_minus1. Tests were completed using the high bit depth / high bit rate CTC in for low QP and standard QP. In addition, some still images, SCC sequences and some extra QP sets, e.g. middle QP, very low QP, are also used to test the proposed method. This report does not verify the results for SCC sequences because we do not have those sequences available at this moment. All other simulation results matched those supplied by the proponents.

# Introduction

Two coding tools were adopted into VVC version 2 for the coding of the last significant coefficient position [1] and the Rice parameter used for transform skip residual coding [2]. Both tools signal a value in the bitstream at slice header. This contribution introduces an encoder only modifications to determine a suitable value for such signalled values.

# Procedure

One code base was provided by the proponent. The code base consisted of VTM-14.0 and the proponent’s modification. The simulations may run at different cluster, therefore, the timing reported here may not be accurate.

# Simulation results

All results matched the proponent’s supplied results. The detailed results could be seen in the attached excel files.

### RLSC

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.02% | 0.01% | 0.01% | 98% | 101% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.01% | -0.02% | -0.01% | 97% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.01% | -0.01% | 0.00% | 98% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.03% | 0.02% | 0.05% | 101% | 102% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.02% | -0.03% | 0.01% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.02% | 0.00% | 0.03% | 101% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.04% | 0.04% | 0.02% | 0.04% | 0.04% | 88% | 90% |
| PQ422 | 0.00% | -0.04% | -0.02% | 0.00% | -0.04% | -0.02% | 90% | 95% |
| Overall | 0.01% | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 89% | 92% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | -0.02% | -0.03% | 94% | 91% |
| HLG422 | 0.00% | -0.02% | -0.01% | 94% | 92% |
| Overall | -0.01% | -0.02% | -0.02% | 94% | 91% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | -0.09% | -0.04% | 103% | 106% |
| HLG422 | -0.05% | -0.17% | -0.08% | 105% | 105% |
| Overall | -0.03% | -0.13% | -0.06% | 104% | 105% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.03% | -0.05% | -0.07% | 92% | 91% |
| HLG422 | -0.02% | -0.14% | -0.03% | 93% | 91% |
| Overall | -0.03% | -0.10% | -0.05% | 93% | 91% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 108% | 96% |
| SVT12 | 0.00% | 0.00% | 0.00% | 106% | 89% |
| Overall | 0.00% | 0.00% | 0.00% | 107% | 93% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 102% | 100% |
| SVT12 | -0.01% | -0.01% | -0.01% | 103% | 101% |
| Overall | -0.01% | 0.00% | 0.00% | 103% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 |  |  |  | 88% | 84% |
| SVT12 |  |  |  | 91% | 86% |
| Overall |  |  |  | 90% | 85% |

Normal QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 116% | 110% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 115% | 108% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 116% | 109% |

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% | 110% | 105% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 109% | 104% |
| Overall | 0.00% | 0.00% | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% | 109% | 105% |

Still-Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | 0.00% | -0.02% | -0.01% | 0.00% | -0.02% | 101% | 107% |
| PQ422 | -0.02% | 0.01% | -0.03% | -0.02% | 0.02% | -0.01% | 102% | 110% |
| Overall | -0.01% | 0.01% | -0.03% | -0.02% | 0.01% | -0.01% | 102% | 108% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 103% | 107% |
| HLG422 | 0.00% | 0.00% | 0.00% | 102% | 108% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 107% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 106% | 105% |
| SVT12 | 0.00% | 0.00% | 0.00% | 108% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 107% | 105% |

Still-Mid QP

| HDR PQ |  |  | AI (QP 2 to 27) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.03% | -0.02% | -0.08% | -0.03% | -0.02% | -0.07% | 95% | 88% |
| PQ422 | -0.05% | 0.06% | -0.08% | -0.05% | 0.08% | -0.04% | 96% | 91% |
| Overall | -0.04% | 0.02% | -0.08% | -0.04% | 0.03% | -0.05% | 96% | 90% |

| HDR HLG |  |  | AI (QP 2 to 27) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.01% | 0.01% | 95% | 85% |
| HLG422 | -0.01% | -0.02% | -0.01% | 98% | 89% |
| Overall | -0.01% | 0.00% | 0.00% | 96% | 87% |

| SVT RGB | AI (QP 2 to 27 for SVT-12, QP -18 to 7 for SVT-16) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 97% | 83% |
| SVT12 | -0.05% | -0.03% | -0.04% | 93% | 83% |
| Overall | -0.03% | -0.02% | -0.02% | 95% | 83% |

### TSRC

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | -0.01% | -0.02% | -0.03% | 99% | 98% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | -0.02% | -0.04% | -0.03% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.02% | -0.03% | -0.03% | 99% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | 0.00% | -0.02% | 0.00% | 100% | 103% |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | 0.01% | -0.01% | -0.01% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.00% | -0.02% | 0.00% | 100% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM14.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | -0.01% | 0.01% | -0.01% | 0.00% | 0.01% | 91% | 90% |
| PQ422 | 0.00% | -0.01% | -0.02% | 0.00% | -0.01% | -0.02% | 92% | 97% |
| Overall | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | -0.01% | 91% | 93% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.02% | -0.02% | 98% | 98% |
| HLG422 | 0.00% | -0.01% | -0.01% | 97% | 94% |
| Overall | 0.00% | -0.02% | -0.02% | 98% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.01% | -0.01% | 106% | 113% |
| HLG422 | 0.00% | -0.01% | -0.02% | 107% | 108% |
| Overall | 0.00% | -0.01% | -0.02% | 106% | 110% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | 0.00% | 95% | 96% |
| HLG422 | -0.01% | 0.01% | 0.00% | 97% | 97% |
| Overall | -0.01% | 0.01% | 0.00% | 96% | 97% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.01% | 0.00% | 99% | 97% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.01% | 0.00% | 0.00% | 105% | 118% |
| SVT12 | 0.00% | 0.00% | 0.00% | 107% | 118% |
| Overall | 0.00% | 0.00% | 0.00% | 106% | 118% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 91% | 101% |
| SVT12 | 0.00% | 0.00% | 0.00% | 95% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 93% | 101% |

Normal QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 97% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM-14.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% | 97% | 96% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Overall | 0.00% | 0.00% | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% | 97% | 97% |

Still-Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.01% | -0.02% | -0.02% | -0.01% | -0.02% | -0.02% | 99% | 98% |
| PQ422 | -0.02% | -0.04% | -0.03% | -0.02% | -0.04% | -0.03% | 101% | 99% |
| Overall | -0.02% | -0.03% | -0.03% | -0.02% | -0.03% | -0.03% | 100% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.02% | -0.02% | 99% | 100% |
| HLG422 | 0.00% | -0.01% | -0.01% | 99% | 99% |
| Overall | 0.00% | -0.02% | -0.02% | 99% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.01% | 0.00% | 103% | 103% |
| SVT12 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 103% |

Still-Very Low QP

| HDR PQ |  |  | AI (QP -23 to 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.31% | -0.31% | -0.06% | -0.29% | -0.29% | 93% | 90% |
| PQ422 | -0.15% | -0.43% | -0.43% | -0.14% | -0.43% | -0.43% | 95% | 91% |
| Overall | -0.11% | -0.37% | -0.37% | -0.10% | -0.36% | -0.36% | 94% | 91% |

| HDR HLG |  |  | AI (QP -23 to 2) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.37% | -0.76% | -0.74% | 93% | 90% |
| HLG422 | -0.44% | -0.75% | -0.72% | 93% | 90% |
| Overall | -0.40% | -0.75% | -0.73% | 93% | 90% |

| SVT RGB | AI (QP-48 to -23 for SVT-16, QP -23 to 2 for SVT-12) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 Still |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 94% | 87% |
| SVT12 | -0.01% | 0.00% | -0.01% | 92% | 86% |
| Overall | -0.01% | 0.00% | -0.01% | 93% | 86% |

# Conclusion

The results obtained in the cross-check match the results presented in JVET-X0136.

# References

[1] “AHG8: on coding of last significant coefficient position for high bit depth and high bit rate extensions”, JVET-V0121, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang

[2] “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-V0054, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang