| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0180 |
| --- | --- |

| Title: | Crosscheck of JVET-X0072 (EE2-related: PDPC-skip scheme for angular intra modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-X0072. The simulation results reportedly match those provided by the proponents.

# Verification

JVET-X0072 [1] is proposed a PDPC skip scheme, which makes use of the results of texture analysis in DIMD method to decide whether skip PDPC or not. There are three methods with different PDPC skip conditions in the proposal.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The coding results of method 1, 2, and 3 under the ECM CTC are summarized in Table 1-3.

Table 1: Performance of method 1 over ECM-2.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.12% | 0.02% | 96% | 93% |
| Class A2 | 0.06% | 0.07% | 0.09% | 96% | 93% |
| Class B | 0.03% | 0.02% | 0.07% | 96% | 92% |
| Class C | -0.02% | 0.11% | 0.08% | 96% | 92% |
| Class E | -0.01% | -0.03% | 0.03% | 96% | 93% |
| Overall | 0.02% | 0.06% | 0.06% | 96% | 92% |
| Class D | -0.05% | 0.18% | -0.05% | 96% | 91% |
| Class F | -0.16% | -0.21% | -0.20% | 97% | 95% |

Table 2: Performance of method 2 over ECM-2.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | -0.04% | 97% | 95% |
| Class A2 | 0.03% | 0.03% | 0.08% | 96% | 93% |
| Class B | 0.03% | 0.06% | -0.04% | 97% | 94% |
| Class C | -0.01% | 0.01% | 0.13% | 96% | 93% |
| Class E | 0.02% | -0.10% | 0.19% | 97% | 94% |
| Overall | 0.02% | 0.01% | 0.05% | 97% | 94% |
| Class D | -0.02% | 0.14% | -0.06% | 96% | 94% |
| Class F | -0.02% | -0.12% | -0.07% | 97% | 94% |

Table 3: Performance of method 3 over ECM-2.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-2.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.19% | -0.01% | 96% | 93% |
| Class A2 | 0.09% | 0.11% | 0.16% | 96% | 92% |
| Class B | 0.08% | 0.14% | 0.15% | 96% | 92% |
| Class C | 0.00% | 0.09% | 0.11% | 96% | 92% |
| Class E | 0.05% | 0.08% | 0.29% | 97% | 94% |
| Overall | 0.06% | 0.12% | 0.14% | 96% | 92% |
| Class D | -0.03% | 0.14% | -0.02% | 95% | 92% |
| Class F | -0.08% | -0.16% | -0.09% | 96% | 94% |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

Jinrong Zhang, Chuan Zhou, Zhuoyi Lv, “EE2-related: PDPC-skip scheme for angular intra modes”, JVET-X0072, October 2021.