| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0194-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-X0135 (Non-EE2: Adaptive intra MTS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group. |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-X0135. The partial simulation results confirm that BD-rate results match with the proponent’s results. The runtime results are also aligned with proponent’s results.

# Introduction

JVET-X0135 proposes to use variable number of MTS candidates for intra coded blocks depending on the position of last significant coefficient. The variable number of MTS candidates i.e., 1, 4 or 6 are signaled depending on the position of the last significant coefficient in the scanning order (lastScanPos),

# Cross-check reports

The simulation results of JVET-X0135 are verified. The tests were conducted under the common test conditions (CTC) defined in [1]. The partial results show the BD-rate results match exactly with proponent’s results. Table 1 shows the crosscheck results.

Table 1: Cross-check results of JVET-X0135

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.01% | -0.04% | 98% | 98% |
| Class A2 | -0.14% | -0.07% | -0.06% | 100% | 97% |
| Class B | -0.12% | -0.04% | -0.17% | 100% | 99% |
| Class C | -0.06% | -0.07% | -0.02% | 102% | 98% |
| Class E | -0.10% | -0.14% | 0.01% | 100% | 98% |
| Overall | -0.10% | -0.06% | -0.07% | 100% | 98% |
| Class D | -0.07% | 0.04% | -0.05% | 103% | 99% |
| Class F | -0.15% | -0.29% | -0.17% | 100% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.12% | 0.00% | 95% | 98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.04% | 0.03% | -0.03% | 94% | 99% |
| Class C | -0.02% | -0.08% | -0.06% | 94% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.48% | 0.31% | 95% | 100% |
| Class F | 0.00% | 0.01% | -0.26% | 95% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.19% | 0.18% | 0.20% | 100% | 99% |
| Class E | 0.02% | 0.28% | -0.29% | 101% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.36% | -0.51% | 102% | 98% |
| Class F | -0.15% | -0.39% | 0.02% | 101% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

M. Karczewicz, Y. Yen, "Common Test Conditions and evaluation procedures for enhanced compression tool testing", JVET-V2017, 22nd meeting, by teleconference, 20 – 28 Apr. 2021.

# Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.