| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-X0209-v1 |
| --- | --- |

| Title: | EE1-related: Report on results of JVET-X remote viewing session |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

Remote expert viewing tests were conducted during the 24th JVET meeting for the exploration activity on DNN-based coding tools on UHD test sequences as defined in EE1. Calls for participation in the subjective viewing were issued on the JVET reflector and verbally during JVET sessions. Overall, 17 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test.

The results reveal some performance improvements for the proposals or similar performance compared to the VTM anchor. In one case, a slight loss is observed. In the super-resolution category, the visual benefit of JVET-X0064 appears to be comparable to the visual benefit observed for the RPR configuration under test reported in JVET-X0117. The results for the loopfilter category suggest some slight visual benefit of JVET-X0066 over the VTM anchor.

# Introduction

Remote expert viewing tests were conducted during the 24th JVET meeting for the exploration activity on DNN-based coding tools defined in EE1 [2]. Calls for participation in the subjective viewing were issued on the JVET reflector. Overall, 17 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test [1].

# Test arrangement

EE1-participants were requested to prepare and submit mp4 files of rate points which they considered to be suitable for visually demonstrating the proposed scheme. All sequences were at UHD resolution and taken from the EE1 test set using the RA configurations. According to the EE1 test configuration, the sequences are of different length. For the tests a consistent length of approximately 5 s was used for all sequences under test. For the Campfire sequence (10 s), the last 5 s were evaluated

Table 1: Pairs of mp4 files assessed in the visual test

| Anchor | Proposal |
| --- | --- |
| VTM/Campfire_3840x2160_30fps_10bit_420_qp37.mp4 | JVET-X0117/Campfire_3840x2160_30fps_10bit_420_qp30.mp4 |
| VTM/Campfire_3840x2160_30fps_10bit_420_qp37.mp4 | JVET-X0064/Campfire_3840x2160_30fps_10bit_420_qp30.mp4 |
| VTM/Campfire_3840x2160_30fps_10bit_420_qp42.mp4 | JVET-X0117/Campfire_3840x2160_30fps_10bit_420_qp36.mp4 |
| VTM/Campfire_3840x2160_30fps_10bit_420_qp42.mp4 | JVET-X0064/Campfire_3840x2160_30fps_10bit_420_qp36.mp4 |
| VTM/CatRobot_3840x2160_60fps_10bit_420_qp37.mp4 | JVET-X0066/CatRobot_3840x2160_60fps_10bit_420_qp37.mp4 |
| VTM/CatRobot_3840x2160_60fps_10bit_420_qp42.mp4 | JVET-X0066/CatRobot_3840x2160_60fps_10bit_420_qp42.mp4 |
| VTM/DaylightRoad2_3840x2160_60fps_10bit_420_qp37.mp4 | JVET-X0066/DaylightRoad2_3840x2160_60fps_10bit_420_qp37.mp4 |
| VTM/DaylightRoad2_3840x2160_60fps_10bit_420_qp42.mp4 | JVET-X0066/DaylightRoad2_3840x2160_60fps_10bit_420_qp42.mp4 |
| VTM/ParkRunning3_3840x2160_50fps_10bit_420_qp37.mp4 | JVET-X0064/ParkRunning3_3840x2160_50fps_10bit_420_qp30.mp4 |
| VTM/ParkRunning3_3840x2160_50fps_10bit_420_qp37.mp4 | JVET-X0117/ParkRunning3_3840x2160_50fps_10bit_420_qp30.mp4 |
| VTM/ParkRunning3_3840x2160_50fps_10bit_420_qp42.mp4 | JVET-X0064/ParkRunning3_3840x2160_50fps_10bit_420_qp35.mp4 |
| VTM/ParkRunning3_3840x2160_50fps_10bit_420_qp42.mp4 | JVET-X0117/ParkRunning3_3840x2160_50fps_10bit_420_qp35.mp4 |
| VTM/Tango_3840x2160_60fps_10bit_420_qp37.mp4 | JVET-X0064/Tango_3840x2160_60fps_10bit_420_qp31.mp4 |
| VTM/Tango_3840x2160_60fps_10bit_420_qp37.mp4 | JVET-X0117/Tango_3840x2160_60fps_10bit_420_qp31.mp4 |
| VTM/Tango_3840x2160_60fps_10bit_420_qp37.mp4 | JVET-X0066/Tango_3840x2160_60fps_10bit_420_qp37.mp4 |
| VTM/Tango_3840x2160_60fps_10bit_420_qp42.mp4 | JVET-X0066/Tango_3840x2160_60fps_10bit_420_qp42.mp4 |
| VTM/Tango_3840x2160_60fps_10bit_420_qp42.mp4 | JVET-X0117/Tango_3840x2160_60fps_10bit_420_qp36.mp4 |
| VTM/Tango_3840x2160_60fps_10bit_420_qp42.mp4 | JVET-X0064/Tango_3840x2160_60fps_10bit_420_qp36.mp4 |

Note: It was decided before the test to present the ParkRunning3 sequence at 60 Hz rather than 50 Hz in order to enable a consistent frame rate relation.

# Test procedure

## Test session setup

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM anchor. It was held as an online session via Zoom with the participants having downloaded the (anonymized) mp4 files before hand in a password protected zip file. The password was provided to the participants before the start of the viewing.

In the following, video A and video B denote either the anchor or the proposal from the pairs of mp4 files listed in Table 1. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects. A single viewing session with a total of 25 BTCs was conducted. Each basic test cell (BTC) was constructed as follows:

- “Original” (on a mid-gray background, 1 s presentation)

- Original Video

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “Vote N” (on a mid-gray background, 5 s presentation)

The test subjects were asked to rate the video sequences presented in each test cell as follows:

- “A was much better than B”

- “A was better than B”

- “B was better than A”

- “B was much better than A”

The used voting sheet is presented in the figure below. The participants were supposed to fill the voting sheet and enter their results into an online form after completion of the session. The participants were also asked to enter the used screen size and any specific observations in a comment field of the online form.

The voting sheets were used in order to simplify the workload for the participants during the session.

Figure 1: Voting sheet

Before the start of the test session, the viewers were provided a demo playlist in order to familiarize themselves with the procedure of the test. The viewers were verbally instructed about the procedure and the rating scale in the introduction of the session. They were asked not to stop the video during the test sessions but to use the assigned voting time to record their judgement.

## Voting scores

For evaluation, the votes of the test subjects were mapped to MOS numbers as follows:

- +3	: “Proposal much better than anchor”

- +1	: “Proposal better than anchor”

- -1	: “Anchor better than proposal”

- -3	: “Anchor much better than proposal”

From these values, the MOS and the 95% confidence intervals were calculated. A resulting MOS value of zero indicates that anchor and proposal were rated to have the same visual quality. The scores of the four participants with the lowest Pearson correlation were removed from the evaluation.

## Participants comments

TBD

# Test results

The MOS values resulting from the evaluation are reported in Table 2. According to the assignment of voting scores, a positive number indicates a visual benefit for the proposal over the anchor while a negative number indicates the proposal to be inferior to the anchor. The test included a stabilization phase and two hidden checks test, “VTM vs Orig” and “VTM vs VTM”.

Table 2: MOS results for the A/B comparisons under test

| Test | Sequence | QP | Name | MOS | CI |
| --- | --- | --- | --- | --- | --- |
| JVET-X0064 | Campfire | 37 | Campfire QP37 | 1.6 | 0.6 |
| JVET-X0064 | Campfire | 42 | Campfire QP42 | 1.1 | 1.0 |
| JVET-X0064 | ParkRunning3 | 37 | ParkRunning3 QP37 | -0.9 | 0.5 |
| JVET-X0064 | ParkRunning3 | 42 | ParkRunning3 QP42 | 1.1 | 0.5 |
| JVET-X0064 | Tango | 37 | Tango QP37 | 0.3 | 0.5 |
| JVET-X0064 | Tango | 42 | Tango QP42 | 0.9 | 0.8 |
| JVET-X0066 | CatRobot | 37 | CatRobot QP37 | 0.4 | 0.5 |
| JVET-X0066 | CatRobot | 42 | CatRobot QP42 | 1.3 | 0.6 |
| JVET-X0066 | DaylightRoad2 | 37 | DaylightRoad2 QP37 | 0.6 | 0.6 |
| JVET-X0066 | DaylightRoad2 | 42 | DaylightRoad2 QP42 | 0.6 | 0.6 |
| JVET-X0066 | Tango | 37 | Tango QP37 | 0.6 | 0.4 |
| JVET-X0066 | Tango | 42 | Tango QP42 | 1.1 | 0.5 |
| JVET-X0117 | Campfire | 37 | Campfire QP37 | 1.3 | 0.8 |
| JVET-X0117 | Campfire | 42 | Campfire QP42 | 1.7 | 0.8 |
| JVET-X0117 | ParkRunning3 | 37 | ParkRunning3 QP37 | 0.1 | 0.7 |
| JVET-X0117 | ParkRunning3 | 42 | ParkRunning3 QP42 | 0.6 | 0.6 |
| JVET-X0117 | Tango | 37 | Tango QP37 | 0.1 | 0.5 |
| JVET-X0117 | Tango | 42 | Tango QP42 | 0.4 | 0.5 |
| VTM vs. Orig | DaylightRoad2 | 37 | DaylightRoad2 QP37 | -1.2 | 0.6 |
| VTM vs VTM | CatRobot | 42 | CatRobot QP42 | 0.4 | 0.5 |

Figure 2: MOS plots

For JVET-X0066, the plots reveal a minor or slight visual benefit of the proposal over the VTM anchor. For CatRobot QP37 and Tango QP37, the benefit is not significant. For JVET-X0064, a loss is reported for ParkRunning QP37, while Tango QP37 is not significant. For the other cases, a minor or slight benefit is indicated. The control test with VTM-based tools in JVET-X0117 shows similar behavior as JVET-X0064, with no loss for ParkRunning QP37.

Multiple participants claimed too high quality for the presented rate points which impacted on the ability to differentiate between the two variants. Participants found the faster ParkRunning sequence challenging for viewing.

# Conclusions

The results reveal some performance improvements for the proposals or similar performance compared to the VTM anchor. In one case, a slight loss is observed. In the super-resolution category, the visual benefit of JVET-X0064 appears to be comparable to the visual benefit observed for the RPR configuration under test reported in JVET-X0117. The results for the loopfilter category suggest some slight visual benefit of JVET- X0066 over the VTM anchor.

# References

- J. Jung, M. Wien, V. Baroncini, “Draft SC29/AG5 Guidelines for remote experts viewing sessions,” Doc. JVET-W0185, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 23rd meeting, online, July 2021.

- E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang, „ EE1: Summary of Exploration Experiments on Neural Network-based Video Coding,“ Doc. JVET-W0023, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 23rd meeting, online, Apr. 2021.