| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X1005-v1 |
| --- | --- |

| Title: | New level for HEVC (Draft 1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, JAPAN | Tel: Email: | teruhiko.s@sony.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes to the High Efficiency Video Coding (HEVC) standard (ITU-T H.265 | ISO/IEC 23008-2), for the addition of Level 6.3.

# Changes to the specification text:

In clause A.4.1, change Table A.8 as follows (additions are yellow-highlighted):

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.3 | 80 216 064 | 240 000 | 1 600 000 | 600 | 22 | 20 |

In clause A.4.2, change Table A.9 as follows (additions are yellow-highlighted):

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |  |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |  |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |  |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |  |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |  |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |  |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |  |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |  |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |  |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |  |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |  |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |  |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |  |

In clause A.4.3, change Tables A.11 and A.12 as follows (additions are yellow-highlighted):

Table A.11 – Maximum picture rates (pictures per second) at levels 1 to 4.1 for some example picture sizes
when MinCbSizeY is equal to 64

| Level: |  |  |  | 1 | 2 | 2.1 | 3 | 3.1 | 4 | 4.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max luma picture size (samples): |  |  |  | 36 864 | 122 880 | 245 760 | 552 960 | 983 040 | 2 228 224 | 2 228 224 |
| Max luma sample rate (samples/sec) |  |  |  | 552 960 | 3 686 400 | 7 372 800 | 16 588 800 | 33 177 600 | 66 846 720 | 133 693 440 |
| Format nickname | Luma width | Luma height | Luma picture size |  |  |  |  |  |  |  |
| SQCIF | 128 | 96 | 16 384 | 33.7 | 225.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QCIF | 176 | 144 | 36 864 | 15.0 | 100.0 | 200.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QVGA | 320 | 240 | 81 920 | - | 45.0 | 90.0 | 202.5 | 300.0 | 300.0 | 300.0 |
| 525 SIF | 352 | 240 | 98 304 | - | 37.5 | 75.0 | 168.7 | 300.0 | 300.0 | 300.0 |
| CIF | 352 | 288 | 122 880 | - | 30.0 | 60.0 | 135.0 | 270.0 | 300.0 | 300.0 |
| 525 HHR | 352 | 480 | 196 608 | - | - | 37.5 | 84.3 | 168.7 | 300.0 | 300.0 |
| 625 HHR | 352 | 576 | 221 184 | - | - | 33.3 | 75.0 | 150.0 | 300.0 | 300.0 |
| Q720p | 640 | 360 | 245 760 | - | - | 30.0 | 67.5 | 135.0 | 272.0 | 300.0 |
| VGA | 640 | 480 | 327 680 | - | - | - | 50.6 | 101.2 | 204.0 | 300.0 |
| 525 4SIF | 704 | 480 | 360 448 | - | - | - | 46.0 | 92.0 | 185.4 | 300.0 |
| 525 SD | 720 | 480 | 393 216 | - | - | - | 42.1 | 84.3 | 170.0 | 300.0 |
| 4CIF | 704 | 576 | 405 504 | - | - | - | 40.9 | 81.8 | 164.8 | 300.0 |
| 625 SD | 720 | 576 | 442 368 | - | - | - | 37.5 | 75.0 | 151.1 | 300.0 |
| 480p (16:9) | 864 | 480 | 458 752 | - | - | - | 36.1 | 72.3 | 145.7 | 291.4 |
| SVGA | 800 | 600 | 532 480 | - | - | - | 31.1 | 62.3 | 125.5 | 251.0 |
| QHD | 960 | 540 | 552 960 | - | - | - | 30.0 | 60.0 | 120.8 | 241.7 |
| XGA | 1 024 | 768 | 786 432 | - | - | - | - | 42.1 | 85.0 | 170.0 |
| 720p HD | 1 280 | 720 | 983 040 | - | - | - | - | 33.7 | 68.0 | 136.0 |
| 4VGA | 1 280 | 960 | 1 228 800 | - | - | - | - | - | 54.4 | 108.8 |
| SXGA | 1 280 | 1 024 | 1 310 720 | - | - | - | - | - | 51.0 | 102.0 |
| 525 16SIF | 1 408 | 960 | 1 351 680 | - | - | - | - | - | 49.4 | 98.9 |
| 16CIF | 1 408 | 1 152 | 1 622 016 | - | - | - | - | - | 41.2 | 82.4 |
| 4SVGA | 1 600 | 1 200 | 1 945 600 | - | - | - | - | - | 34.3 | 68.7 |
| 1080 HD | 1 920 | 1 080 | 2 088 960 | - | - | - | - | - | 32.0 | 64.0 |
| 2Kx1K | 2 048 | 1 024 | 2 097 152 | - | - | - | - | - | 31.8 | 63.7 |
| 2Kx1080 | 2 048 | 1 080 | 2 228 224 | - | - | - | - | - | 30.0 | 60.0 |
| 4XGA | 2 048 | 1 536 | 3 145 728 | - | - | - | - | - | - | - |
| 16VGA | 2 560 | 1 920 | 4 915 200 | - | - | - | - | - | - | - |
| 3616x1536 (2.35:1) | 3 616 | 1 536 | 5 603 328 | - | - | - | - | - | - | - |
| 3672x1536 (2.39:1) | 3 680 | 1 536 | 5 701 632 | - | - | - | - | - | - | - |
| 3840x2160 (4*HD) | 3 840 | 2 160 | 8 355 840 | - | - | - | - | - | - | - |
| 4Kx2K | 4 096 | 2 048 | 8 388 608 | - | - | - | - | - | - | - |
| 4096x2160 | 4 096 | 2 160 | 8 912 896 | - | - | - | - | - | - | - |
| 4096x2304 (16:9) | 4 096 | 2 304 | 9 437 184 | - | - | - | - | - | - | - |
| 7680x4320 | 7 680 | 4 320 | 33 423 360 | - | - | - | - | - | - | - |
| 8192x4096 | 8 192 | 4 096 | 33 554 432 | - | - | - | - | - | - | - |
| 8192x4320 | 8 192 | 4 320 | 35 651 584 | - | - | - | - | - | - | - |
| 11520×6480 | 11 520 | 6 480 | 74 649 600 | - | - | - | - | - | - | - |
| 12288×6144 | 12 288 | 6 144 | 75 497 472 | - | - | - | - | - | - | - |
| 12288×6480 | 12 288 | 6 480 | 79 626 240 | - | - | - | - | - | - | - |

Table A.12 – Maximum picture rates (pictures per second) at levels 5 to 6.3 for some example picture sizes
when MinCbSizeY is equal to 64

| Level: |  |  |  | 5 | 5.1 | 5.2 | 6 | 6.1 | 6.2 | 6.3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max luma picture size (samples): |  |  |  | 8 912 896 | 8 912 896 | 8 912 896 | 35 651 584 | 35 651 584 | 35 651 584 | 80 216 064 |
| Max luma sample rate (samples/sec) |  |  |  | 267 386 880 | 534 773 760 | 1 069 547 520 | 1 069 547 520 | 2 139 095 040 | 4 278 190 080 | 4 812 963 840 |
| Format nickname | Luma width | Luma height | Luma picture size |  |  |  |  |  |  |  |
| SQCIF | 128 | 96 | 16 384 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| QCIF | 176 | 144 | 36 864 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| QVGA | 320 | 240 | 81 920 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 525 SIF | 352 | 240 | 98 304 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| CIF | 352 | 288 | 122 880 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 525 HHR | 352 | 480 | 196 608 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 625 HHR | 352 | 576 | 221 184 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| Q720p | 640 | 360 | 245 760 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| VGA | 640 | 480 | 327 680 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 525 4SIF | 704 | 480 | 360 448 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 525 SD | 720 | 480 | 393 216 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 4CIF | 704 | 576 | 405 504 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 625 SD | 720 | 576 | 442 368 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 480p (16:9) | 864 | 480 | 458 752 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| SVGA | 800 | 600 | 532 480 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| QHD | 960 | 540 | 552 960 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| XGA | 1 024 | 768 | 786 432 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 720p HD | 1 280 | 720 | 983 040 | 272.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 4VGA | 1 280 | 960 | 1 228 800 | 217.6 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| SXGA | 1 280 | 1 024 | 1 310 720 | 204.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 525 16SIF | 1 408 | 960 | 1 351 680 | 197.8 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 16CIF | 1 408 | 1 152 | 1 622 016 | 164.8 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 4SVGA | 1 600 | 1 200 | 1 945 600 | 137.4 | 274.8 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 1080 HD | 1 920 | 1 080 | 2 088 960 | 128.0 | 256.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 2Kx1K | 2 048 | 1 024 | 2 097 152 | 127.5 | 255.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 2Kx1080 | 2 048 | 1 080 | 2 228 224 | 120.0 | 240.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 4XGA | 2 048 | 1 536 | 3 145 728 | 85.0 | 170.0 | 300.0 | 300.0 | 300.0 | 300.0 | 960.0 |
| 16VGA | 2 560 | 1 920 | 4 915 200 | 54.4 | 108.8 | 217.6 | 217.6 | 300.0 | 300.0 | 960.0 |
| 3616x1536 (2.35:1) | 3 616 | 1 536 | 5 603 328 | 47.7 | 95.4 | 190.8 | 190.8 | 300.0 | 300.0 | 858.9 |
| 3672x1536 (2.39:1) | 3 680 | 1 536 | 5 701 632 | 46.8 | 93.7 | 187.5 | 187.5 | 300.0 | 300.0 | 844.1 |
| 3840x2160 (4*HD) | 3 840 | 2 160 | 8 355 840 | 32.0 | 64.0 | 128.0 | 128.0 | 256.0 | 300.0 | 576.0 |
| 4Kx2K | 4 096 | 2 048 | 8 388 608 | 31.8 | 63.7 | 127.5 | 127.5 | 255.0 | 300.0 | 573.5 |
| 4096x2160 | 4 096 | 2 160 | 8 912 896 | 30.0 | 60.0 | 120.0 | 120.0 | 240.0 | 300.0 | 540.0 |
| 4096x2304 (16:9) | 4 096 | 2 304 | 9 437 184 | - | - | - | 113.3 | 226.6 | 300.0 | 510.0 |
| 4096x3072 | 4 096 | 3 072 | 12 582 912 | - | - | - | 85.0 | 170.0 | 300.0 | 382.5 |
| 7680x4320 | 7 680 | 4 320 | 33 423 360 | - | - | - | 32.0 | 64.0 | 128.0 | 144.0 |
| 8192x4096 | 8 192 | 4 096 | 33 554 432 | - | - | - | 31.8 | 63.7 | 127.5 | 143.4 |
| 8192x4320 | 8 192 | 4 320 | 35 651 584 | - | - | - | 30.0 | 60.0 | 120.0 | 135.0 |
| 11520×6480 | 11 520 | 6 480 | 74 649 600 | - | - | - | - | - | - | 64.0 |
| 12288×6144 | 12 288 | 6 144 | 75 497 472 | - | - | - | - | - | - | 63.7 |
| 12288×6480 | 12 288 | 6 480 | 79 626 240 | - | - | - | - | - | - | 60.0 |

____________