| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li K. Naser H. Yang | Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@tencent.com karam.naser@interdigital.com haitao.yang@huawei.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 24th meeting by teleconference, 6–15 October 2021, and the 25th meeting by teleconference, 12-21 January 2022.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM2 algorithm description JVET-W2025.

- Study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Refine test conditions in JVET-W2017, generate anchors, identify new test sequences to be added, especially high resolution ones in 8K, in coordination with AHG4.

- Analyse the results of exploration experiments described in JVET-W2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The Common Test Conditions were updated to fix mislabelled sequences in the excel sheet and clarify configurations used for SCC (JVET-X2017). The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-X2024). The indication of the achievable improvement over VVC are the results of the EE2 software base (ECM-3.0). The combined improvement of the ECM-3.0 over VTM-11.0 with the improved MCTF from JVET-V0056 for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.98% | -12.44% | -16.63% | 311% | 241% |
| Class A2 | -6.44% | -13.13% | -11.99% | 298% | 232% |
| Class B | -6.05% | -13.75% | -14.21% | 338% | 257% |
| Class C | -7.06% | -10.21% | -10.67% | 322% | 252% |
| Class E | -7.58% | -11.55% | -12.41% | 322% | 294% |
| Overall | -6.75% | -12.28% | -13.16% | 320% | 255% |
| Class D | -5.86% | -8.45% | -8.23% | 316% | 265% |
| Class F | -10.80% | -15.69% | -15.66% | 244% | 298% |
| Class TGM | -15.62% | -18.73% | -18.25% | 232% | 293% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.95% | -16.42% | -21.85% | 367% | 466% |
| Class A2 | -15.84% | -20.38% | -20.42% | 364% | 511% |
| Class B | -13.57% | -19.94% | -19.32% | 384% | 493% |
| Class C | -15.35% | -17.27% | -16.98% | 387% | 427% |
| Class E |  |  |  |  |  |
| Overall | -14.77% | -18.61% | -19.42% | 377% | 473% |
| Class D | -16.10% | -17.65% | -16.99% | 378% | 459% |
| Class F | -13.54% | -18.02% | -17.76% | 345% | 407% |
| Class TGM | -14.65% | -18.96% | -19.20% | 348% | 304% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -12.28% | -24.89% | -24.11% | 302% | 331% |
| Class C | -12.51% | -18.81% | -18.94% | 315% | 294% |
| Class E | -12.11% | -17.24% | -19.15% | 298% | 296% |
| Overall | -12.31% | -20.95% | -21.14% | 305% | 310% |
| Class D | -14.09% | -20.17% | -19.36% | 303% | 296% |
| Class F | -12.15% | -18.99% | -18.67% | 295% | 324% |
| Class TGM | -14.02% | -21.79% | -22.13% | 269% | 255% |

# Contributions

In addition to 18 EE2 contributions, 42 related contributions were received which can be subdivided as follows:

## Partitioning (1)

JVET-Y0174, "EE2-1.1-related: additional tests on partitioning flexibility", F. Urban, K. Naser, F. Galpin (Interdigital), K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang (Bytedance)

## In Loop Filters (1)

JVET-Y0148, "Non-EE2: Spatial-Temporal Adaptive Loop Filter", W. Yin, K. Zhang, Y. Li, H. Liu, L. Zhang (Bytedance)

## Intra (11)

JVET-Y0055, "AHG12: Slope adjustment for CCLM", J. Lainema, A. Aminlou, P. Astola, R. G. Youvalari (Nokia)

JVET-Y0092, "Non-EE2: On chroma intra prediction mode", X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-Y0097, "AhG12: Removed DIMD from MPM list of TIMD", K. Naser, T. Dumas, Y. Chen, F. Galpin (InterDigital)

JVET-Y0109, "AhG12: Neural Network-based intra prediction",	T. Dumas, F. Galpin, P. Bordes, F. Mom, E. Francois (InterDigital)

JVET-Y0119, "EE2-related: On Extended MRL Intra Prediction", K. Sato, Y. Yu, H. Yu, Z. Xie, L. Xu, F. Wang, H. Huang, J. Gan, D. Wang (OPPO)

JVET-Y0130, "EE2-related : Unification of availability check for intra mode coding", S. Yoo, H. Jang, J. Nam, J. Choi, J. Lim, S. Kim (LGE)

JVET-Y0131, "EE2-related : Clean-up on DIMD", S. Yoo, H. Jang, J. Nam, J. Choi, J. Lim, S. Kim (LGE)

JVET-Y0140, "EE2-related: On the LCU boundary processing by intra-prediction tools",	A. Filippov, V. Rufitskiy, D. Ruiz Coll (Ofinno)

JVET-Y0144, "Non-EE2: DIMD Flag Signalling Clean-up", J. Zhao, S. Kim (LGE)

JVET-Y0149, "EE2-related: Modifications of the extended MRL candidate list",	A. Filippov, V. Rufitskiy, E. Dinan (Ofinno)

JVET-Y0203, "EE2-related: a combination of the extended MRL candidate list (JVET-Y0149) modifications with the extended LCU boundary processing area (JVET-Y0140)", A. Filippov, V. Rufitskiy, D. Ruiz Coll, E. Dinan (Ofinno)

## Inter and RPR (12)

JVET-Y0076, "Non-EE2: Template Matching-based OBMC Design",Z. Lv, C. Zhou, J. Zhang (vivo)

JVET-Y0089, "Non-EE2: DMVR with BCW enabled",	P. Bordes, A. Robert, Y. Chen, F. Galpin (Interdigital)

JVET-Y0091, "Non-EE2: MVP refinement for regular AMVP mode", C. Zhou, Z. Lv, J. Zhang (vivo)

JVET-Y0120, "EE2-related: Non-adjacent temporal MVP", F. Wang, Z. Xie, Y. Yu, H. Yu, L. Xu, K. Sato, J. Gan, D. Wang (OPPO)

JVET-Y0125, "AHG12: Enhanced bi-directional motion compensation",	Y.-W. Chen, C.-W. Kuo, N. Yan, W. Chen, X. Xiu, X. Wang (Kwai Inc.)

JVET-Y0129, "Non-EE2: MVD and merge index signaling of AMVP-merge mode", Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)

JVET-Y0135, "Non-EE2: Template matching based reordering for GPM split modes", C.-C. Chen, H. Huang, Y. Zhang, Z. Zhang, Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-Y0139, "Non-EE2: On the extended number of active reference pictures and reference picture reordering", 	H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-Y0151, "Non-EE2: Adaptive re-ordering of merge candidates with refined motion", Y. Wang, K. Zhang, N. Zhang, Z. Deng, L. Zhang (Bytedance)

JVET-Y0154, "AHG12: Bilinear Interpolation Filtering for ARMC", W. Chen, X. Xiu, H. Gao, Y.-W. Chen, X. Wang (Kwai)

JVET-Y0161, "EE2-3.12-related: Extensions of history-parameter-based affine model inheritance", K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang (Bytedance)

JVET-Y0172, "Non-EE2: Long tap interpolation filtering on chroma components", X. Xie, K. Zhang, L. Zhang, Junru Li, Meng Wang, Shiqi Wang (Bytedance)

## RPR (2)

JVET-Y0095, "Non-EE2: RPR with luma-only re-scaling", P. Bordes, F. Galpin, E. Francois (Interdigital)

JVET-Y0128, "Non-EE2: fixing issues for RPR enabling and non-CTC configuration in ECM", Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm), P.Bordes (InterDigital)

## Entropy and Coefficient Coding (5)

JVET-Y0114, "Non-EE2: Dependent quantization with 4 states for chroma components",	Y. Chen, E. Francois, F. Galpin, P. de Lagrange (InterDigital)

JVET-Y0121, "EE2-4.2-related: On adaptive sign prediction position selection",	L. Xu, Y. Yu, H. Yu, Z. Xie, F. Wang, D. Wang (OPPO)

JVET-Y0141, "EE2-4.3 related: More combined test results for sign prediction",	J. Chen, Y. Ye, R.-L. Liao, X. Li (Alibaba), X. Xiu, Y.-W. Chen, N. Yan, C.-W. Kuo, H.-J. Jhu, W. Chen, X. Wang (Kwai)

JVET-Y0157, "AHG12: Improved probability estimation for CABAC",	X. Xiu, Y.-W. Chen, W. Chen, H. Gao, H.-J. Jhu, C.-W. Kuo, X. Wang (Kwai)

JVET-Y0181	"AHG12: CABAC initialization from previous inter slice", V. Seregin, J. Dong, N. Hu, M. Karczewicz (Qualcomm)

## Transform Coding (1)

JVET-Y0159, "EE2-related: inter MTS refinement on adaptive intra MTS (EE2-4.4)", T. Hashimoto, T. Ikai (Sharp)

## Screen Content Coding (5)

JVET-Y0062, "Non-EE2: Cross-component palette coding", B. Vishwanath, K. Zhang, L. Zhang (Bytedance)

JVET-Y0088, "EE2-related: IBC with Template Matching", A. Robert, K. Naser, T. Poirier, Y. Chen, F. Galpin (InterDigital)

JVET-Y0124, "Non-EE2: Intra Block Copy with An Extended Reference Area",	J. Xu (ByteDance)

JVET-Y0133, "EE2-related: BVP candidate adjustment based on IBC reference region implemented on top of test EE2-3.13",	D. Ruiz Coll, A. Filippov, V. Rufitskiy, T.M. Bae (Ofinno)

JVET-Y0160, "EE2-3.13-related: Enlarged HMVP table for IBC", N. Zhang, K. Zhang, L. Zhang (Bytedance)

## ECM Encoder and Test Conditions (4)

JVET-Y0102, "On the balance of ECM coding gains between luma and chroma",	F. Le Léannec, P. Andrivon, E. Thomas(Xiaomi)

JVET-Y0113, "Adjusting luma/chroma BD-rate balance in ECM", 	Y. Chen, E. Francois, P. Nikitin (InterDigital)

JVET-Y0108, "AhG3/AhG12 Modification of JVET CTC for environmental considerations",	F. Galpin, M. Radosavljevic, E. Francois (InterDigital)

JVET-Y0117, "AhG12: ECM coding performance for HDR/WCG content and suggested common test conditions", 	T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby), N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

# Recommendations

The AHG recommends to:

- To review all the related contributions.

- To encourage contribution of new test sequences, especially 8k resolution, which might be included in future test conditions.