| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0023_r1 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | input to JVET |  |  |
| Purpose: | Exploration Experiment report |  |  |
| Author(s) or Contact(s): | E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma and H. Wang | Tel: Email: | elena.alshina@huawei.com, shanl@tencent.com, chenwei06@kwai.com, Franck.Galpin@InterDigital.com, yue.li@bytedance.com, mazhan@nju.edu.cn, hongtaow@qti.qualcomm.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Exploration Experiment 1 on Neural Network-based Video Coding. Originally 7 NN-based technologies (plus VVC RPR) have been included into EE1. Among them: 3 NN-based filter, 3 NN-based super resolution and one NN-based Intra techniques. One test (super resolution) was dropped, 3 EE related contributions with further development tools under study are submitted to this meeting. Several variants of NN-based in-loop filters with complexity 500...2500 kMAC/pxl providing average bit-saving in a range 2...11% (in random access configuration) have been demonstrated in this round of Exploration Experiment. Several variants of super-resolution (from available in VVC standard functionality to NN-based) demonstrate 3...6% bit-saving in random access configuration for 4K content.   NN-based Intra tool improves Intra coding by ~3%.

- Test conditions and evaluation criteria

Tests in this Exploration Experiment follow AhG11 testing conditions and complexity assessment methodology [1]. Anchor is VTM11.0 with improved GOP-based temporal filter [2] enabled. In total 5 quality levels (corresponding QP=22, 27, 32, 37, 42) are used for objective performance measurement.

Additionally in this round of EE several proposals have been tested on top of ECM SW. Some proponents used ECM3.0, another ECM3.1 as SW basis/anchor, so very accurate performance comparison is not possible. For best NN-based in-loop filters gain on top of AhG11 anchor is 8-10%, but drops to 5-7% on top of ECM. So, overlap in performance between NN-based under study in EE1/AhG11 and “classical” tools under study in EE2/AhG12 is about 2-3%.

AhG11 has agreed before [3] to summarize results in graphic forms, showing compression performance (BD-rate) vs kMAC/pxl and BD-rate vs Total number of model parameters (considering those two not only, but the most important complexity characteristics).

Fig. 1 shows average BD-rate in Random Access configuration vs kMAC/pxl for different EE1 and related proposals. Fig. 2 shows average BD-rate in Random Access configuration vs Total number of parameters for different EE1 and related proposals.

Super-resolution proposals do not appear on those plots. Two proposals ,which are based on VVC RPR (2D separable filter of up-sampling), which is applied adaptively and so show average BD-rate gain, have too low complexity to be show next to NN-based technologies. Two NN-based Super-resolution proposals included into this EE are used unconditionally for all sequences, so show BD-rate drop in average. Merging of two gain bringing aspects (adaptive resolution coding and NN-based up-sampling) is still subject for further study.

Fig. 1. Average BD-rate in Random Access configuration vs kMAC/pxl for different EE1 and related proposals. The dash line indicates capability of NVIDIA RTX3080 for 4K@60 processing.

Fig. 2. Average BD-rate in Random Access configuration vs Total number of parameters for different EE1 and related proposals.

In total 7 tests in this EE have been cross-checked (thanks Ericsson, OPPO and Huawei for their efforts and comments). Addressing comments from cross-checker should greatly help collaborative work on NN-based tools.

Majority of EE1 participants used PyTorch/libtorch (v1.3, 1.4, 1.6, 1.8, 1.9) for their training/tests and float-point implementation. Conducting cross-checks or/and collaborative development on NN-based tools w/o agreement on particular version of libtorch (if it will decided to use libtorch for future development) is complicated.

Due to the float point arithmetic if (for example) encoding is done on linux, decoding on windows then mismatch is observed.

One proponent used SADL library [4] and int16 implementation. Cross-check for this proposal was successful (matching results, no issue with encoding decoding on different platforms) even at JVET-X meeting.

# Exploration experiments on Enhancement filters

Two proposals perform significantly better than other in this category. It makes sense to compare them with more details in Table 1.

| Test | EE1-1.1 (Tencent) JVET-Y0078 | EE1-1.2 (Bytedance and Qualcomm) JVET-Y0143 |
| --- | --- | --- |
| Filter-structure |  |  |
| Attention module | Use prediction and reconstruction |  |
|  | both are as inputs to NN | weight between NN-module input and output for F_out = F_in × f (Rec, Pred, BS, QP) + F_in f comprises 2 convolutional layers |
|  | partitioning map, QP | BS, QP |
| Stride2 convolution | yes | yes |
| Colors | 3 colors processed together | separate processing for Luma and Chroma |
| N – number of Res Blocks | 24 | 8 |
| M – number of channels / features | 64 (192) goes up before and then goes down after the activation layer | 96 |
| Position | I-slice: replaces DF, SAO B-slice: switching between proposed NN-filter and DF & SAO | replaces DF, SAO |
| Control | CTU and slice level on/off | Filter selected from 3 candidates {q, q - 5, q - 10} on/off control as well as the conditional parameter index signalled per 128x128 |
| Content adaptation | scaling factor signaled in slice header | scaling factor signaled in picture header for each color component |
| Total number of models | 2 | 4 |
| Block extension | 8 samples each side | 8 samples each side |
| kMAC/pxl | 509 | 539 |
| Total Parameters (Millions) | 3.1 | 6.3 |
| Implementation | Torch v1.9 / float32 | Pytorch v1.6 / float32 |
| BD-rate vs VTM | 8.6% (Luma) / 20% (Chroma) | 9.8% (Luma) / 21% (Chroma) |
| BD-rate vs ECM | 5.3% (Luma) / 15% (Chroma) | 7.2% (Luma) / 18% (Chroma) |

From this analysis one can see that filters are conceptually very similar. Both were included into subjective quality assessment test. Both have attracted attention of cross-checkers and there are EE1-releated contribution which further improve both filters.

If combined with EE2 tools available in ECM3.1 then JVET-Y0143 can provide 21% (Luma) / 33% (Chroma) gain over VTM (in RA configuration, QP=22,...,42).

Random Access cfg, QP=22, 27, 32, 37, 42

|  | ECM3.1 over VTM-11.0(w/NewMCTF) |  |  |  |  | ECM3.1 & EE1-1.2 over VTM-11.0(w/NewMCTF) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -16.4% | -16.4% | -22.3% | 4.6 | 4.8 | -22.4% | -27.7% | -34.8% | 5.5 | 476 |
| Class A2 | -16.6% | -21.2% | -20.7% | 4.5 | 5.2 | -23.1% | -33.7% | -36.1% | 5.2 | 462 |
| Class B | -13.7% | -20.9% | -20.0% | 4.2 | 4.9 | -19.6% | -35.0% | -35.0% | 4.8 | 422 |
| Class C | -15.0% | -17.2% | -16.4% | 4.0 | 4.7 | -21.2% | -31.6% | -32.1% | 4.3 | 331 |
| Overall | -15.2% | -19.1% | -19.6% | 4.3 | 4.9 | -21.3% | -32.4% | -34.4% | 4.9 | 413 |
| Class D | -15.4% | -16.9% | -15.8% | 3.9 | 4.9 | -22.7% | -33.0% | -33.4% | 4.1 | 296 |
| Class F | -13.6% | -19.4% | -19.2% | 3.4 | 3.9 | -16.8% | -27.2% | -27.2% | 4.7 | 181 |

Random Access cfg, QP=22, 27, 32, 37 (same as EE2 ctc)

|  | ECM3.1 over VTM-11.0(w/NewMCTF) |  |  |  |  | ECM3.1 & EE1-1.2 over VTM-11.0(w/NewMCTF) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -16.4% | -16.4% | -22.3% | 4.6 | 4.8 | -22.4% | -27.7% | -34.8% | 5.5 | 476 |
| Class A2 | -16.6% | -21.2% | -20.7% | 4.5 | 5.2 | -23.1% | -33.7% | -36.1% | 5.2 | 462 |
| Class B | -13.7% | -20.9% | -20.0% | 4.2 | 4.9 | -19.6% | -35.0% | -35.0% | 4.8 | 422 |
| Class C | -15.0% | -17.2% | -16.4% | 4.0 | 4.7 | -21.2% | -31.6% | -32.1% | 4.3 | 331 |
| Overall | -15.2% | -19.1% | -19.6% | 4.3 | 4.9 | -21.3% | -32.4% | -34.4% | 4.9 | 413 |
| Class D | -15.4% | -16.9% | -15.8% | 3.9 | 4.9 | -22.7% | -33.0% | -33.4% | 4.1 | 296 |
| Class F | -13.6% | -19.4% | -19.2% | 3.4 | 3.9 | -16.8% | -27.2% | -27.2% | 4.7 | 181 |

Series of tests from Dahua EE1-1.3.1, 2 and 3 show not that good complexity performance trade-off, but provides the group with useful information about performance impact of taking Chroma information for Luma NN-based enhancement and Luma information for Chroma  NN-based enhancement:

(a)

(b)

Table below demosntrates that Chroma as extra input for Luma NN-based in-loop filtering provides ~0.3% Luma gain (with negligible increment of complexity). Luma as extra input for Chroma NN-based in-loop filtering brings 11% Chroma gain, while computational complexity roughly doubles.

| Test | Description | Total Number of Parameters (Millions) | Worst Case Complexity (kMAC/pixel) | Random Access (CTC) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | Cb | Cr |
| EE1-1.3.1 | Luma only (a) without chroma input | 12.2 | 1902.6 | -2.3% | -0.2% | -0.3% |
| EE1-1.3.2 | Luma only (a) with chroma input | 12.2 | 1904.4 | -2.6% | -0.2% | -0.3% |
| EE1-1.3.3 | (a) + (b) | 20.0 | 2510.6 | -2.7% | -11.8% | -11.2% |

- Super resolution category specific

There are proposals which use VVC RPR content adaptively and so show gain (or at least no loss) for all sequences. And there are proposals which encode all content in smaller resolution (so in average show drop). Before those two gain-bringing aspects will be combined one can compare technologies in this category only looking on 4K results (Fig. 3 and Fig. 4).

One of EE1-related proposals shows reasonable-speed encoder which decides about content adaptive enabling of RPR, and so allows to obtain compression gain and have no cross of RD-curves w/o multi-pass. It is highly recommended to combine this of similar technology with NN-based super resolution in future EE (if any) to be able eventually compare average (not only 4K ) gain of all NN-based tools (including super-resolution).

Fig. 3. Average BD-rate for 4K sequences in Random Access configuration vs kMAC/pxl for different EE1 and related proposals in super resolution category. The dash line indicates capability of NVIDIA RTX3080 for 4K@60 processing.

Fig. 4. Average BD-rate for 4K sequences in Random Access configuration vs Total number of parameters for different EE1 and related proposals in super resolution category.

Observations of cross-checkers in this category is as follows. NN-based super-resolution processing is often not performed after decoding (PSRN, MS-SSIM metrics are measured based on encoder reconstruction), and so NN-based processing doesn’t appear in decoding run-time. This must be fixed in a future.

- NN-Intra  category specific

It makes sense to look closely onto test results in “all-intra” configuration while discussing Intra tools. The comparison of NN-filter and NN-Intra tests in this EE is shown in Fig. 5.

Fig. 5.  Average BD-rate in “all intra” configuration vs kMAC/pxl for different EE1 proposals. The dash line indicates capability of NVIDIA RTX3080 for 4K@60 processing.

The only NN-based Intra tool was implemented using SADL library [4] using 16 bits integer implementation. Complexity of NN-based intra tools (measured in kMAC/pxl) is significantly lower than complexity of NN-based filter, while gain is substantial. Likely NN-based Intra and NN-based filters utilize different sources of compression gain. It make sense to test combination.

If combined with EE2 tools available in ECM3.1 then JVET-Y0082 can provide 8.5% (Luma) / 15% (Chroma) gain over VTM (in All Intra configuration, QP=22,...,42).

All Intra cfg, QP=22, 27, 32, 37, 42

|  | ECM3.1 over VTM-11.0(w/NewMCTF) |  |  |  |  | ECM3.1 & EE1-3.1 over VTM-11.0(w/NewMCTF) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.1% | -13.8% | -17.3% | 3.9 | 2.7 | -9.0% | -14.9% | -18.4% | 6.0 | 12 |
| Class A2 | -6.5% | -14.3% | -12.4% | 3.8 | 2.6 | -7.7% | -15.1% | -13.2% | 6.1 | 15 |
| Class B | -6.2% | -15.1% | -15.3% | 3.7 | 2.7 | -7.7% | -16.0% | -16.2% | 6.3 | 16 |
| Class C | -7.2% | -11.3% | -11.6% | 3.5 | 2.6 | -8.8% | -12.4% | -12.6% | 5.8 | 19 |
| Class E | -7.6% | -12.0% | -13.6% | 3.5 | 2.9 | -9.8% | -13.7% | -14.4% | 5.8 | 20 |
| Overall | -6.9% | -13.4% | -14.0% | 3.7 | 2.7 | -8.5% | -14.5% | -15.0% | 6.0 | 16 |
| Class D | -6.1% | -8.6% | -8.3% | 3.4 | 2.5 | -7.6% | -9.6% | -9.5% | 5.5 | 22 |
| Class F | -11.1% | -17.0% | -17.2% | 2.4 | 3.1 | -11.9% | -17.3% | -17.7% | 3.7 | 11 |

All Intra cfg, QP=22, 27, 32, 37 (same as EE2 ctc)

|  | ECM3.1 over VTM-11.0(w/NewMCTF) |  |  |  |  | ECM3.1 & EE1-3.1 over VTM-11.0(w/NewMCTF) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.0% | -12.4% | -16.6% | 3.8 | 2.6 | -8.9% | -13.6% | -17.8% | 6.0 | 13 |
| Class A2 | -6.4% | -13.1% | -12.0% | 3.7 | 2.5 | -7.7% | -14.0% | -12.9% | 5.9 | 15 |
| Class B | -6.1% | -13.8% | -14.2% | 3.7 | 2.6 | -7.5% | -14.7% | -15.1% | 6.2 | 16 |
| Class C | -7.1% | -10.2% | -10.7% | 3.5 | 2.5 | -8.6% | -11.3% | -11.7% | 5.7 | 19 |
| Class E | -7.6% | -11.6% | -12.4% | 3.5 | 2.8 | -9.9% | -13.3% | -13.6% | 5.7 | 19 |
| Overall | -6.7% | -12.3% | -13.2% | 3.6 | 2.6 | -8.4% | -13.4% | -14.2% | 5.9 | 16 |
| Class D | -5.9% | -8.4% | -8.2% | 3.4 | 2.5 | -7.4% | -9.6% | -9.5% | 5.4 | 21 |
| Class F | -10.8% | -15.7% | -15.7% | 2.4 | 2.9 | -11.6% | -16.0% | -16.1% | 3.6 | 11 |

# Visual test

Two best performing NN-based filter, VVC RPR and typical representative of NN-based super-resolution technology were recommended to be include into the viewing.

Sequences and QP for viewing were discussed at AhG11/EE1 teleconference conducted at Nov. 16.  Details on this discussion are summarized in [5]. Viewing was conducted in 3 different sessions: 2160p, 1080p, 720p. Each video was limited to be 5 seconds only (to shorten overall viewing time). Several sequences out of AhG11 CTC were included into viewing sessions.

Several issues have been identified during viewing preparation:

- ffmpeg version was not the same for different proponents who prepared viewing and naming and organization of mp4 files was inconsistent across test points;

- way of cutting 5 second part of sequences was not the same for different proponents who prepared viewing.

Proposed solutions:

- Specify ffmpeg version in viewing preparation instruction (latest 64bit-version recommended)

- In order to cut 5 sec part of the video first cut then create mp4. Example for creation “last 5 sec” video  from Campfire sequence:

  - ffmpeg -s:v 3840x2160 -c:v rawvideo -pix_fmt yuv420p10le -i Campfire_3840x2160_30fps_bt709_420_videoRange.yuv -filter:v select="between(n\, 150\,  299)" temp.yuv

  - ffmpeg -s:v 3840x2160 -c:v rawvideo -pix_fmt yuv420p10le -r 30 -i temp.yuv -c:v libx265 -crf 15 -tag:v hvc1 -pix_fmt yuv420p10le Campfire_3840x2160_30fps_10bit_420.mp4

- Specify a precise naming convention and directory structure for provided mp4 files.

- Specify a time line for provision of test material including crosscheck among participating proponent organizations.

Viewing was conducted on Jan. 10th and Jan. 11th, results are reported in [6].

- Recommendation

- Review all EE1 documents for more detailed information.

- Review results of viewing conducted prior JVET-Y meeting for prepared materials.

- References

[1] S. Liu, A. Segall, E. Alshina, R.-L. Liao Common Test Conditions and evaluation procedures for neural network-based video coding technology JVET-W2016.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

[3] A. Segall, BoG Report: EE1 Viewing Preparation and Neural Networks Video Coding Results Analysis JVET-X0188.

[4] F. Galpin, T. Dumas, P. Bordes, P. Nikitin, F. Le Leannec, E. Francois (InterDigital) AHG11: Small Ad-hoc Deep-Learning Library JVET-W0181 https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl .

[5] E. Alshina, M. Wien, A. Segall, J. Sauer   AhG11/EE1 viewing preparation report JVET-Y0045

[6] M. Wien, AHG4: REV Result for AHG11/EE1 and AHG10/Deblocking, JVET-Y0212