| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0052-v1 |
| --- | --- |

| Title: | AHG11: CNN post-processing filter based on depthwise separable convolution and attention mechanism |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Dan Zou Ming Li | Email: | 13227706628@163.com zhengzk@xidian.edu.cn zoudan@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes weakly connected dense attention neural network for compression artifact removal, called WCDANN. WCDANN is a post-processing filter based on convolutional neural networks (CNNs) that does not need any change of codecs. WCDANN consists of several weakly connected dense attention blocks (WCDABs), which is based on residual learning by taking the compressed video after codecs as the input. We use depthwise separable convolution as the basic convolution unit for WCDANN to generate a lightweight model. Moreover, we introduce attention mechanism into the proposed filter to emphasize important features. Compared with VTM-11.0-NNVC, WCDANN achieves average 4.78%, 4.06% and 4.33% BD-rate reductions for Y channel on C and D classes, under AI, RA and LDP configurations, respectively.

Introduction

In this contribution, we report a CNN post-processing filter based on depthwise separable convolution and attention mechanism, named WCDANN. WCDANN aims to learn effective residual information and improve the quality of the input compressed image. WCDANN is composed of two basic modules WCDAB and RAB to extract the important residual features from the input image. WCDANN includes residual connections in each module to promote the circulation of residual information, while using two attention modules CAB and CSAB to enhance the important residual features in the output of RAB and WCDAB, respectively. We use depthwise separable convolution as the basic convolution unit to greatly reduce the amount of model parameters and generate a lightweight model. Besides, we train WCDANN for different quantization parameters (QPs).

Proposed method

Network Architecture

The architecture of the proposed CNN post-processing filter (WCDANN) is shown in Fig. 1. The filter consists of three parts: Head, backbone, and reconstruction. The head part consists of two convolutional layers, which are used to extract the shallow features of the input image. The backbone network consists of M weakly connect dense attention blocks (WCDABs) containing 4 residual attention blocks (RABs). The reconstruction part is structurally symmetrical to the head part. In addition, a residual connection is used between input and output to learn global residual information and enhance the quality of input image.

Fig. 1 Architecture of WCDANN. WCDANN consists of M WCDABs. “3ⅹ3 Conv” denotes 3ⅹ3 common convolution layer. ReLU denotes Rectified Linear Unit for activation. Each Conv has 64 channels. WCDABM denotes the M-th weakly connect dense attention block.

The architecture of the proposed WCDAB is shown in Fig. 2. WCDAB consists of four RABs and one channel-spatial joint attention block (CSAB). Some residual and concatenate connections are used in WCDAB to promote the circulation of residual information and prevent the gradient disappearance during training. We use CSAB in the end of each WCDAB to emphasize spatial and channel-wise feature refinement. The architecture of CSAB is shown in Fig. 3, which consists of two parallel branches: channel attention branch and spatial attention branch. Through these two branches, the channel attention map and spatial attention map corresponding to the input features are obtained, and then they are fuse through the addition operation to obtain the channel-spatial attention map. Note that all convolutions in the spatial attention block are depthwise convolutions.

Fig. 2 Architecture of WCDAB. WCDAB consists of M WCDABs. “1ⅹ1 Conv” denotes 1ⅹ1 common convolution layer. RABi denotes i-th residual attention block (i=1, …,4). CSAB denotes channel-spatial joint attention block.

Fig. 3 (a) Architecture of CAB. (b) Architecture of CSAB. (c) Architecture of SAB. “3x3 depthwise conv” denotes 3x3 depthwise convolution layer. “1x1 depthwise conv” denotes 1x1 depthwise convolution layer. ReLU denotes Rectified Linear Unit for activation. Sigmoid denotes sigmoid function.

The architecture of the proposed RAB is shown in Fig. 4(a). Each RAB has a dual-branch structure to increase the receptive field and capture multi-level information. All the convolution layers in these two branches are depthwise separable convolution, and their structure is shown in Fig. 4(b). The ReLU activation function can be used after only the pointwise convolutional layer, instead of the depthwise convolutional layer. We use a channel shuffle layer after two branches to fully integrate the features of different receptive fields at the channel level. Furthermore, we use a channel attention module (CAB) [2] in the end of each RAB to emphasize channel-wise feature refinement.

Fig. 4 (a) Architecture of RAB. “3x3 DSconv” denotes 3x3 depthwise separable convolution layer. Channel shuffling denotes channel shuffling operation. CAB denotes channel attention block. (b) Structure of the “DSconv” in implementation.

Training

In the training phase, we use two neural network architectures, and their main difference is the number of WCDABs. We refer to them as WCDANNsmall and WCDANNbig. WCDANNsmall includes 3 WCDABs (M=3) and WCDANNbig includes 6 WCDABs (M=6). Specifically, small networks are used to enhance the quality of video using low QP compression (QP= {22, 27}), and large networks are used to enhance the quality of video with high QP compression (QP= {32,37,42}). DIV2K and BVI-DVC [3] datasets are used to train the proposed WCDANN. All pictures are compressed using VTM 11.0- NNVC under the condition of QP = {22, 27, 32, 37, 42} to train the corresponding QP networks, respectively. All pictures are cropped into 128×128 patches in 125 steps to form the training set. We only use the Y channel of each image in the training set to train WCDANN. The total epochs are 50, and batch size is set to 8, L1 loss and L2 loss are used to train WCDANN. For training, we use L1 loss in the first 40 epochs and L2 loss in the last 10 epochs. It is worth noting that we generate five models (i.e. WCDANNQP22, WCDANNQP27, WCDANNQP32, WCDANNQP37, WCDANNQP42) during training according to five QPs (22, 23, 32, 37, 42) and the parameters for the five models are fixed. Since WCDANN is a post-processing filter used after the VTM decoder, there is no need to transmit the network parameters from the encoder to the decoder through bitstream or generate them in the decoder side.

Table 1. Network information of the proposed post-processing filter in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 8 |
|  | Training time: | 115h/model for WCDANNbig 60h / model for WCDANNsmall |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1 and L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | - |
|  | Other information: | - |

Inference

In the inference phase, we use WCDANN according to base QPs [4] as follows:

WCDANN=WCDANNQP22,  &QPbase≤24.5WCDANNQP27,  &24.5<QPbase≤29.5WCDANNQP32,  &29.5<QPbase≤34.5WCDANNQP37,  &34.5<QPbase≤39.5WCDANNQP42,  &QPbase>39.5

Specifically, the Y channel of the compressed video is input into WCDANN to obtain the enhanced Y channel, and then combined with the original UV channel to obtain the enhanced video. Unlike [5], WCDANN is a post-processing filter, thus resulting in the VTM encoding and decoding time and bitrate unchanged. Therefore, we do not report the change in the VTM encoding and decoding time in the table. In addition, since the proposed filter works outside the VTM, we set the bit depth of the compressed video by VTM to be the same as the bit depth of the input video.

Table 2. Network Information for NN-based post-processing filter in Inference Stage

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Filters | WCDANNsmall |  | WCDANNbig |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type |  | GPU: GeForce RTX 3090 |  |
|  | Framework: |  | PyTorch v1.9 |  |
|  | Number of GPUs per Task |  | 1 |  |
|  | Number of Parameters (Each Model) |  | 430897 | 786721 |
|  | Memory Parameter (MB) |  | 1,863777 | 3,420289 |
|  | Multiply Accumulate (MAC) |  |  |  |
| Optional |  |  |  |  |
|  | Total Conv. Layers |  | 100 | 196 |
|  | Total FC Layers |  | 30 | 60 |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  | 1 |  |
|  | Patch size |  |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Border handling |  |  |  |
|  | Other information: |  |  |  |

Experimental results

The proposed CNN post-processing filter is evaluated based on VTM-11.0-NNVC according to the common test conditions (CTCs) defined in [6]. We perform the evaluation in the Random Access (RA), All Intra (AI) and Low Delay P (LDP) configurations. Experimental results are shown in Tables 3, 4 and 5. In the AI configuration, the proposed filter achieves 4.78 % BD-rate reduction on average for Y channel. In the RA configuration, the proposed filter achieves 4.06% BD-rate reduction on average for Y channel. In the LDP configuration, the proposed filter achieves 4.33% BD-rate reduction on average for Y channel.

Table 3 BD rate of WCDANN over VTM-11.0 NNVC in RA configuration.

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |  |
| Class C | -3,36 % | 0,00 % | 0,00 % | -2,62 % | 0,00 % | 0,00 % | 0 % |  |
| Class D | -4,76% | 0,00 % | 0,00 % | -2,68 % | 0,00 % | 0,00 % | 0 % |  |

Table 4 BD rate of WCDANN over VTM-11.0 NNVC in AI configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |  |
| Class C | -4,58 % | 0,00 % | 0,00 % | -3.99 % | 0,00 % | 0,00 % | 0 % |  |
| Class D | -4,97% | 0,00 % | 0,00 % | -3,70 % | 0,00 % | 0,00 % | 0 % |  |

Table 5 BD rate of WCDANN over VTM-11.0 NNVC in LDP configuration.

|  | Low Delay P Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |  |
| Class C | -3.93 % | 0,00 % | 0,00 % | -3.10 % | 0,00 % | 0,00 % | 0 % |  |
| Class D | -4.72 % | 0,00 % | 0,00 % | -3.17 % | 0,00 % | 0,00 % | 0 % |  |

Conclusions

This contribution presents a CNN post-processing filter based on depthwise separable convolution and attention mechanism, named WCDANN. WCDANN consists of two novel modules WCDAB and RAB to effectively extract the residual features from the input image. It uses the attention mechanism in the two modules to emphasize important features. Moreover, the depthwise separable convolution is used as the basic convolution unit to make WCDANN lightweight. Experimental results demonstrate that WCDANN achieves average 4.78%, 4.06% and 4.33% BD-rate reductions over VTM-11.0-NNVC for Y channel on C and D classes in AI, RA and LDP configurations, respectively.

We propose including the proposed CNN post-processing filter in EE1 and recommending further study on it.

References

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- S. Woo, J. Park, J.-Y. Lee, I. Kweon. “CBAM: Convolutional block attention module.” In Proc. ECCV (2018)

- Di Ma, Fan Zhang, and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

- F. Zhang, C. Feng and D. R. Bull, "Enhancing VVC Through Cnn-Based Post-Processing," 2020 IEEE International Conference on Multimedia and Expo (ICME), 2020, pp. 1-6, doi: 10.1109/ICME46284.2020.9102912.

- M. Santamaria, Y.-H. Lam, J. Lainema, F. Cricri, R. Ghaznavi-Youvalari, A. Zare, H. Zhang, H. R. Tavakoli, and M. Hannuksela, “AHG11: Content-adaptive neural network post-processing filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-W0057, April 2021.

- S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, April 2021.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).