| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0055-v1 |
| --- | --- |

| Title: | AHG12: Slope adjustment for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Alireza Aminlou Pekka Astola Ramin G. Youvalari | Email: | firstname.lastname@nokia.com |
| Source: | Nokia Technologies |  |  |

_____________________________

# Abstract

This contribution proposes to signal an adjustment for the slope parameter(s) used in cross-component linear model (CCLM) prediction of ECM. The adjustment is “tilting” the linear function which maps luma values to chroma values with respect to a center point determined by the average luma value of the reference samples. The encoder selectable adjustment is proposed to be an integer value between -4 and 4 in 1/8th sample units and is signalled at the PU level for each CCLM model which uses both top and left reference samples. It is reported that the proposed modification to ECM provides -0.05%, -1.10% and -0.98% BD-rate impact in AI configuration for Y, U and V, respectively. In RA configuration the impact is reportedly -0.01%, -0.44% and -0.48% for Y, U and V, respectively. Impact on encoder and decoder runtimes is asserted not significant.

# Introduction

Cross-component linear model prediction (CCLM) performs well if there is a strong linear correlation between the luma and chroma channels. However, the reference samples that are used to generate the linear model parameters can be noisy or those may not represent well the content inside the actual prediction block. In these cases, the prediction may fail and the process results in suboptimal coding efficiency.

This contribution attempts to address such issues by introducing an adjustment term that can be used to tune the CCLM model to better fit the linear mapping to the content of the prediction block.

# Proposed approach

Basic principle

CCLM uses a model with 2 parameters to map luma values to chroma values. The slope parameter “a” and the bias parameter “b” define the mapping as follows:

chromaVal = a * lumaVal + b

It is proposed signal an adjustment “u” to the slope parameter to update the model to the following form:

chromaVal = a’ * lumaVal + b’

where

a’ = a + u

b’ = b - u * yr

With this selection the mapping function is tilted or rotated around the point with luminance value yr. It is proposed to use the average of the reference luma samples used in the model creation as yr in order to provide a meaningful modification to the model. Picture below illustrates the process.

Figure 1: Illustration of the effect of the slope adjustment parameter “u”. Left: model created with the current CCLM. Right: model updated as proposed.

Implementation

Slope adjustment parameter is provided as an integer between -4 and 4, inclusive, and signaled in the bitstream. The unit of the slope adjustment parameter is 1/8th of a chroma sample value per one luma sample value (for 10-bit content).

Adjustment is available for the CCLM models that are using reference samples both above and left of the block (“LM_CHROMA_IDX” and “MMLM_CHROMA_IDX”), but not for the “single side” modes. This selection is based on coding efficiency vs. complexity trade-off considerations.

When slope adjustment is applied for a multimode CCLM model, both models can be adjusted and thus up to two slope updates are signaled for a single chroma block.

Encoder approach

The proposed encoder approach performs an SATD based search for the best value of the slope update for Cr and a similar SATD based search for Cb. If either one results as a non-zero slope adjustment parameter, the combined slope adjustment pair (SATD based update for Cr, SATD based update for Cb) is included in the list of RD checks for the TU.

# Experimental results

Tables below summarize the coding efficiency impact for AI and RA configurations. Decodings have been done under similar cluster loads and thus decoder runtimes can be considered relatively reliable. During encodings cluster loads have varied and runtimes should not be considered accurate.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03 % | -1.79 % | -1.64 % | 101 % | 100 % |
| Class A2 | -0.06 % | -1.19 % | -0.48 % | 102 % | 101 % |
| Class B | -0.05 % | -1.34 % | -1.38 % | 100 % | 99 % |
| Class C | -0.06 % | -0.87 % | -0.94 % | 99 % | 99 % |
| Class E | -0.02 % | -0.23 % | -0.20 % | 100 % | 99 % |
| Overall | -0.05 % | -1.10 % | -0.98 % | 100 % | 100 % |
| Class D | -0.04 % | -0.62 % | -0.57 % | 101 % | 99 % |
| Class F | -0.10 % | -0.71 % | -0.96 % | 100 % | 98 % |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01 % | -0.65 % | -0.81 % | 98 % | 100 % |
| Class A2 | -0.02 % | -0.34 % | -0.09 % | 97 % | 101 % |
| Class B | 0.01 % | -0.66 % | -0.75 % | 99 % | 100 % |
| Class C | -0.02 % | -0.09 % | -0.18 % | 98 % | 100 % |
| Class E |  |  |  |  |  |
| Overall | -0.01 % | -0.44 % | -0.48 % | 98 % | 100 % |
| Class D | 0.04 % | -0.30 % | -0.13 % | 99 % | 100 % |
| Class F | -0.01 % | -0.30 % | -0.52 % | 99 % | 99 % |

Full simulation results are provided in the attached Excel sheet.

# Requested action

It is proposed to include the proposed technique to the next round of Exploration Experiments on enhanced compression beyond VVC capability.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).