| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0065 |
| --- | --- |

| Title: | EE2-3.1: GPM with inter and intra prediction (JVET-X0166) |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan Hyeongmun Jang Seung-hwan Kim Jaehyun Lim (LGE) 19, Yangjae-daero 11-gil, Seocho-gu, Seoul, Republic of Korea Zhipin Deng Kai Zhang Li Zhang (Bytedance) 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: Tel: Email: Email: | +81 70 3508 5829 yo-kidani@kddi.com +02 10 8505 0793 hm.jang@lge.com zhipin.deng@bytedance.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) LG Electronics Inc. Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2 Test 3.1 on geometric partitioning mode (GPM) with inter and intra prediction. With the proposed methods, GPM can generate the final prediction samples with not only inter predicted samples but also intra predicted samples. It means the proposed method allows to enable GPM in LP configuration where the conventional GPM is disabled by default (since bi-prediction is restricted). Furthermore, the restriction of GPM with intra and intra prediction, a direct motion vector and intra prediction mode (IPM) storage, several IPM derivation schemes, and combination of GPM with MMVD and/or GPM with TM are introduced to maximize the coding performance.

Simulation results of over all classes on the top of ECM-3.1 (tool-off) in RA/LB/LP are reported as follows.

- EE2-3.1a:

  - RA: BD-rate YUV: -0.10%, -0.29%, -0.21%; EncT: 101%; DecT: 100%

  - LB: BD-rate YUV: -0.18%, -0.50%, -0.53%; EncT: 101%; DecT: 100%

  - LP: BD-rate YUV: -0.42%, -1.30%, -1.17%; EncT: 106%; DecT: 99%

- EE2-3.1b:

  - RA: BD-rate YUV: -0.14%, -0.40%, -0.34%; EncT: 101%; DecT: 100%

  - LB: BD-rate YUV: -0.29%, -0.70%, -0.93%; EncT: 101%; DecT: 99%

  - LP: BD-rate YUV: -0.65%, -1.81%, -1.36%; EncT: 115%; DecT: 100%

- EE2-3.1c:

  - RA: BD-rate YUV: -0.17%, -0.43%, -0.36%; EncT: 101%; DecT: 100%

  - LB: BD-rate YUV: -0.29%, -0.74%, -0.76%; EncT: 101%; DecT: 100%

  - LP: BD-rate YUV: -0.67%, -1.97%, -1.57%; EncT: 116%; DecT: 101%

- EE2-3.1d:

  - RA: BD-rate YUV: -0.16%, -0.41%, -0.48%; EncT: 101%; DecT: 100%

  - LB: BD-rate YUV: -0.31%, -0.67%, -0.49%; EncT: 102%; DecT: 100%

  - LP: BD-rate YUV: -0.71%, -1.97, -1.61%; EncT: 121%; DecT: 101%

Introduction

In current ECM design, geometric partitioning mode (GPM) generates the final prediction samples only with inter predictions, which may have room to improve the prediction accuracy. To tackle the problem, GPM with inter and intra prediction methods were proposed in JVET-X0166 [1].

In this contribution, a total of four tests on GPM with inter and intra prediction are conducted per the EE2 description [2]. Especially, the purpose of the Test 3.1c and 3.1d is to study the effect of the proposed elements regarding the template matching according to the comment of experts in the previous meeting.

Proposed methods

Test 3.1a: GPM with inter and intra prediction

In GPM with inter and intra prediction, the final prediction samples are generated by weighting inter predicted samples and intra predicted samples for each GPM-separated region. The inter predicted samples are derived by the same scheme as the GPM in the current ECM whereas the intra predicted samples are derived by an intra prediction mode (IPM) candidate list and an index signaled from the encoder. The IPM candidate list size is pre-defined as 3. The available IPM candidates are the parallel angular mode against the GPM block boundary (Parallel mode), the perpendicular angular mode against the GPM block boundary (Perpendicular mode), and the Planar mode as shown Fig. 1a ~ c, respectively. Furthermore, GPM with intra and intra prediction as shown Fig. 1d is restricted in the proposed method to reduce the signalling overhead for IPMs and avoid an increase in the size of the intra prediction circuit on the hardware decoder. In addition, a direct motion vector and IPM storage on the GPM-blending area is introduced to further improve the coding performance.

Figure 1 (a) ~ (c) Available IPM candidates for the Test 3.1a. (d) Example of GPM with intra and intra prediction.

Test 3.1b: Test 3.1a + DIMD and neighboring mode based IPM derivation + Combination of GPM-intra and GPM-MMVD

In the Test 3.1b, the following two modifications are introduced into the Test 3.1a method to achieve more coding performance.

- DIMD and neighboring mode based IPM derivation

- Combination of GPM-intra and GPM-MMVD

The IPM candidate list size is the same as the Test. 3.1a and the Parallel mode is registered first. Therefore, max two IPM candidates derived from the decoder-side intra mode derivation (DIMD) method in the ECM-3.1 and/or the neighboring blocks can be registered if there is not the same IPM candidate in the list. As for the neighboring mode derivation, there are five positions for available neighboring blocks at most, but they are restricted by the angle of GPM block boundary as shown in Table 1, which has been already used for GPM with template matching (GPM-TM) in the ECM-3.1.

Different from the Test 3.1a, GPM with intra prediction (GPM-intra) can be utilized with GPM with merge with motion vector difference (GPM-MMVD) which has been already implemented in the ECM-3.1.

Table 1 : The position of available neighboring blocks for IPM candidate derivation based on the angle of GPM block boundary. A and L denotes the above and left side of the prediction block.

| Angle of GPM | 0 | 2 | 3 | 4 | 5 | 8 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |
| Partition angle | 16 | 18 | 19 | 20 | 21 | 24 | 27 | 28 | 29 | 30 |
| 1st partition | A | A | A | A | L+A | L+A | L+A | L+A | A | A |
| 2nd partition | L+A | L+A | L+A | L | L | L | L | L+A | L+A | L+A |

Test 3.1c: Test 3.1b + TIMD-based IPM derivation

In the Test 3.1c, template-based intra mode derivation (TIMD) in the ECM-3.1 can be additionally utilized for IPM candidates of GPM-intra to further improve the coding performance. The IPM candidate list size is also the same as the Test. 3.1a. The Parallel mode can be registered first, then IPM candidates of TIMD, DIMD, and neighboring blocks in this order.

Test 3.1d: Test 3.1c + Combination of GPM-intra and GPM-TM

In the Test 3.1d, GPM-intra with GPM with template matching (GPM-TM) can be utilized in addition to the Test 3.1c to increase the application rates of GPM-intra blocks.

Simulation Results

The proposed methods are implemented on top of ECM-3.1 [3]. Tool-off test results under RA/LB/LP configurations are summarized in Table 2~Table 5, respectively. The simulation tests were conducted following the common test condition and evaluation procedures for enhanced compression tool testing [4]. More detailed results can be found in the attached spreadsheets.

Table 2 : Simulation results of Test 3.1a on the top of ECM-3.1 (tool-off test).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.06% | -0.31% | -0.36% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.05% | -0.14% | -0.12% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.07% | -0.27% | -0.13% | 101% | 100% | -0.20% | -0.71% | -0.52% | 101% | 99% | -0.38% | -1.12% | -1.47% | 106% | 99% |
| C | -0.19% | -0.41% | -0.28% | 101% | 100% | -0.31% | -0.72% | -0.77% | 101% | 99% | -0.67% | -1.88% | -1.72% | 107% | 98% |
| E |  |  |  |  |  | 0.02% | 0.12% | -0.21% | 100% | 101% | -0.16% | -0.84% | 0.07% | 105% | 100% |
| Overall | -0.10% | -0.29% | -0.21% | 101% | 100% | -0.18% | -0.50% | -0.53% | 101% | 100% | -0.42% | -1.30% | -1.17% | 106% | 99% |
| D | -0.09% | -0.14% | 0.07% | 101% | 100% | -0.14% | -0.80% | -1.29% | 101% | 99% | -0.29% | -1.18% | -0.95% | 107% | 99% |
| F | -0.04% | -0.23% | -0.15% | 99% | 100% | -0.18% | -0.71% | -0.39% | 100% | 100% | -0.28% | -0.66% | -0.77% | 103% | 100% |

Table 3 : Simulation results of Test 3.1b on the top of ECM-3.1 (tool-off test).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.09% | -0.42% | -0.49% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.10% | -0.29% | -0.15% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.13% | -0.35% | -0.33% | 101% | 100% | -0.29% | -0.84% | -0.85% | 101% | 99% | -0.56% | -1.69% | -1.48% | 113% | 100% |
| C | -0.23% | -0.53% | -0.38% | 101% | 100% | -0.46% | -1.08% | -1.28% | 101% | 98% | -0.94% | -2.44% | -2.10% | 120% | 99% |
| E |  |  |  |  |  | -0.05% | 0.03% | -0.60% | 100% | 100% | -0.41% | -1.19% | -0.18% | 114% | 101% |
| Overall | -0.14% | -0.40% | -0.34% | 101% | 100% | -0.29% | -0.70% | -0.93% | 101% | 99% | -0.65% | -1.81% | -1.36% | 115% | 100% |
| D | -0.15% | -0.18% | -0.15% | 101% | 100% | -0.24% | -1.26% | -1.51% | 101% | 99% | -0.55% | -1.56% | -1.57% | 118% | 100% |
| F | -0.06% | -0.27% | -0.28% | 99% | 100% | -0.23% | -0.93% | -0.37% | 100% | 99% | -0.31% | -0.94% | -0.87% | 108% | 100% |

Table 4 : Simulation results of Test 3.1c on the top of ECM-3.1 (tool-off test).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.12% | -0.45% | -0.52% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.12% | -0.34% | -0.20% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.15% | -0.52% | -0.29% | 101% | 100% | -0.27% | -0.89% | -0.75% | 101% | 100% | -0.57% | -1.71% | -1.51% | 113% | 101% |
| C | -0.26% | -0.38% | -0.44% | 101% | 100% | -0.47% | -1.06% | -1.10% | 101% | 99% | -1.03% | -2.50% | -2.30% | 120% | 101% |
| E |  |  |  |  |  | -0.07% | -0.05% | -0.31% | 100% | 101% | -0.35% | -1.70% | -0.69% | 114% | 101% |
| Overall | -0.17% | -0.43% | -0.36% | 101% | 100% | -0.29% | -0.74% | -0.76% | 101% | 100% | -0.67% | -1.97% | -1.57% | 116% | 101% |
| D | -0.17% | -0.43% | -0.20% | 101% | 100% | -0.24% | -0.52% | -1.28% | 101% | 99% | -0.52% | -2.08% | -0.89% | 118% | 103% |
| F | -0.08% | -0.21% | -0.25% | 100% | 100% | -0.13% | -1.15% | -1.04% | 101% | 99% | -0.29% | -1.17% | -0.79% | 110% | 101% |

Table 5 : Simulation results of Test 3.1d on the top of ECM-3.1 (tool-off test).

|  |  |  | RA |  |  |  |  | LB |  |  |  |  | LP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.10% | -0.45% | -0.48% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| A2 | -0.11% | -0.34% | -0.28% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| B | -0.14% | -0.40% | -0.31% | 102% | 100% | -0.26% | -0.88% | -0.43% | 102% | 100% | -0.61% | -1.73% | -1.55% | 119% | 101% |
| C | -0.26% | -0.43% | -0.43% | 102% | 100% | -0.52% | -1.02% | -0.76% | 102% | 99% | -1.08% | -2.64% | -2.49% | 126% | 101% |
| E |  |  |  |  |  | -0.13% | 0.13% | -0.25% | 102% | 101% | -0.38% | -1.47% | -0.55% | 117% | 101% |
| Overall | -0.16% | -0.41% | -0.37% | 101% | 100% | -0.31% | -0.67% | -0.49% | 102% | 100% | -0.71% | -1.97% | -1.61% | 121% | 101% |
| D | -0.18% | -0.17% | -0.27% | 102% | 101% | -0.33% | -0.57% | -1.37% | 102% | 100% | -0.50% | -2.44% | -1.40% | 123% | 102% |
| F | -0.05% | -0.28% | -0.33% | 101% | 100% | -0.38% | -0.54% | -0.96% | 102% | 100% | -0.09% | -1.16% | -1.25% | 112% | 101% |

Conclusion

In this contribution, GPM with inter and intra prediction methods were proposed. The proposed methods can bring some coding gains with a good trade-off between coding performance and complexity. It is recommended to adopt the proposed methods into the next ECM.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- Y. Kidani, H. Kato, K. Kawamura, H. Jang, S. Kim, J. Lim, Z. Deng, K. Zhang, and L. Zhang, “EE2-related: Combination of JVET-X0078 (Test 7/8), JVET-X0147 (Proposal-2), and GPM direct motion storage,” JVET-X0166, by teleconference, 6–15 October 2021.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-X2024, by teleconference, 6–15 October 2021.

- https://vcgit.hhi.fraunhofer.de/ecm/jvet-x-ee2/ECM/-/tags/ECM-3.1

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-X2017, by teleconference, 6-15 October. 2021.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).