| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0071 |
| --- | --- |

| Title: | New Test Content for Video Conferencing Applications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zeina Sinno Gregory Desgouttes Alexis Michael Tourapis David Singer | Tel: Email: | +1 408 228 7983 atourapis@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This document presents 4 new video conferencing sequences that could be used for the development of new Video Coding standards by MPEG and the JVET. The sequences included were captured on a mobile device at 30fps and at a 1080x1920 resolution. They are provided in both an HDR (Non Constant Luminance YCbCr 420 BT.2100 HLG) and an SDR HDR (Non Constant Luminance YCbCr 420 BT.709) representation.

# Introduction

In this contribution we introduce 4 new video sequences that can be used for the evaluation of video coding tools in the context of video conferencing applications. These new sequences were captured at almost lossless quality on a mobile device in an HDR video format (Non Constant Luminance YCbCr 420 BT.2100 HLG) but are also provided in an SDR version. All sequences involve a single subject walking or standing outdoors at different locations. Capture was performed in a portrait format, at 30fps, and at 1080x1920 resolution. The sequences are provided under the Creative Commons Attribution-ShareAlike 4.0 International license [1].

# Sequence Description

The 4 sequences are released under our "Gregory" collection of test content. The first sequence is named as GregoryScarf and involves our subject wearing a bright red scarf and walking outdoors. Highlights of this sequence involve colourful bushes, a fence, and big blue sky areas. The sequence is a total of 748 frames. Two snapshots from these sequence are shown in Figure 1.

Figure 1. Snapshots from sequence GregoryScarf

The second video sequence is named GregoryFence and mostly involves the subject talking in front of a house fence. The scene is characterized by also a lot of global motion, whereas the subject also starts moving towards the end of the segment. The sequence is a total of 631 frames.

Figure 1. A snapshot from sequence GregoryFence

The third clip is named GregoryCactus and involves our subject conversing in front of a cactus. This segment consists of a total of 516 frames.

Figure 1. A snapshot from sequence GregoryCactus

The fourth clip is named GregoryWalk and involves our subject walking on a street while conversing on his mobile device. Other than the subjects face/head one can see prominently the sky, moving trees, signs, and buildings.

Figure 1. A snapshot from sequence GregoryWalk

# Copyright information

Copyright © 2021 Apple, Inc. All Rights Reserved.

Apple, Inc. provides this test material under the following license.

https://creativecommons.org/licenses/by-sa/4.0/

Owners of these sequences: Apple Inc.

Production: Apple Inc.

# References

- Creating Commons License CC BY-SA 4.0, https://creativecommons.org/licenses/by-sa/4.0/

# Patent rights declaration(s)

Apple Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).