| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0079-v3 |
| --- | --- |

| Title: | EE1-1.1-related: the result of neural network based in-loop filter on ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang Xiaozhong Xu Shan Liu | Email: | liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the result of JVET-Y0078 [] on ECM. The proposed filter in EE1-1.1.1 is integrated on ECM-3.0, and the corresponding 2 models are both used directly without any other retaining processes or refinements. Compared with ECM-3.0, {5.32%, 13.97%, 14.62%}, {4.48%, 13.03%, 13.73%} and {4.54%, 13.07%, 12.87%} BD-rate savings are reported with RA, LB and AI configurations, respectively. Compared with EE1 anchor, ECM3.0 with the proposed method can achieve {19.64%, 29.89%, 31.22%}, {11.14%, 24.34%, 24.98%} BD-rate savings with RA and AI configurations, respectively.

Introduction

In JVET-Y0078, a neural network based in-loop filter with constrained storage and low complexity is proposed. In all, only two models are used in the proposed filter design, for which each model is designed with relatively low complexity. In order to study the performance of neural network based in-loop filter on ECM, the proposed filter in JVET-Y0078 is integrated on ECM-3.0 for evaluation.

Proposed method

Network architecture

The network structure of the proposed NN filter is shown in Fig. 1. Along with the reconstructed image (rec_yuv), the prediction image (pred_yuv) [] and the partition image (par_yuv) [] are also fed into the network.

Based on the un-filtered reconstructed image, the partition image is generated by setting each sample value into the average value of the whole partition block. It needs to be explained that the base QP is fed only for the network of the B slice, and the partition image is not fed for the B slice.

The 2 models in JVET-Y0078 are both used directly without any other retaining processes or refinements.

Fig. 1: Network architecture

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Torch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time: | ~150h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Torch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | I: 1.56M, B: 1.55M |
|  | Total Number of Parameters (All Models) | 3.11M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | ~6.5MB/model, 2 models in all |
|  | Multiplay Accumulate (MAC)/pixel, worst-case | 509K |
| Optional |  |  |
|  | Total Conv. Layers | 198 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Results

Based on the ECM3.0 anchor [][][], {5.32%, 13.97%, 14.62%}, {4.48%, 13.03%, 13.73%} and {4.54%, 13.07%, 12.87%} BD-rate savings are reported with RA, LB and AI configurations, respectively.

Based on the EE1 anchor [][], ECM3.0 with the proposed method can achieve {19.64%, 29.89%, 31.22%}, {11.14%, 24.34%, 24.98%} BD-rate savings with RA and AI configurations, respectively.

Results of EE1-1.1.1 on ECM3.0

Table 3. Performance of the proposed method (ECM, RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-3.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.98% | -5.91% | -9.13% | -11.22% | 106% | 6938% |
| Class A2 | -9.17% | -6.53% | -15.85% | -18.35% | 105% | 6255% |
| Class B | -7.73% | -5.09% | -16.11% | -15.18% | 106% | 7177% |
| Class C | -6.58% | -4.24% | -13.52% | -13.65% | 104% | 6635% |
| Class E |  |  |  |  |  |  |
| Overall | -7.56% | -5.32% | -13.97% | -14.62% | 105% | 6791% |
| Class D | -7.12% | -4.85% | -13.16% | -14.70% | 104% | 5860% |
| Class F | -2.07% | -1.12% | -5.09% | -4.77% | 115% | 2453% |

Table 4. Performance of the proposed method (ECM, LB)

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-3.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6.75% | -4.44% | -13.34% | -14.01% | 106% | 8947% |
| Class C | -6.08% | -3.77% | -12.64% | -13.39% | 101% | 8412% |
| Class E | -8.20% | -5.48% | -16.45% | -16.30% | 116% | 9287% |
| Overall | -6.70% | -4.48% | -13.03% | -13.73% | 107% | 8847% |
| Class D | -6.26% | -4.26% | -11.21% | -13.32% | 102% | 7730% |
| Class F | -2.43% | -1.23% | -6.42% | -5.65% | 113% | 3210% |

Table 5. Performance of the proposed method (ECM, AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-3.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.01% | -4.44% | -10.04% | -11.40% | 116% | 8653% |
| Class A2 | -6.88% | -4.18% | -14.62% | -15.36% | 109% | 7729% |
| Class B | -5.81% | -3.73% | -13.29% | -10.82% | 108% | 5786% |
| Class C | -6.52% | -4.34% | -12.67% | -13.48% | 105% | 5269% |
| Class E | -8.62% | -6.63% | -14.74% | -14.44% | 110% | 7423% |
| Overall | -6.65% | -4.54% | -13.07% | -12.87% | 109% | 6629% |
| Class D | -7.00% | -4.56% | -13.58% | -15.10% | 104% | 4919% |
| Class F | -2.24% | -1.65% | -4.03% | -3.99% | 112% | 978% |

Results of EE1-1.1.1 + ECM3.0 beyond EE1 anchor

Table 6. Performance of the proposed method (VTM, RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -22.76% | -21.28% | -23.55% | -30.91% | 454% | 33096% |
| Class A2 | -24.88% | -21.90% | -32.85% | -34.77% | 439% | 32538% |
| Class B | -21.65% | -18.07% | -33.01% | -31.75% | 406% | 33505% |
| Class C | -21.07% | -18.66% | -28.53% | -28.12% | 390% | 29728% |
| Class E |  |  |  |  |  |  |
| Overall | -22.37% | -19.64% | -29.89% | -31.22% | 417% | 32184% |
| Class D | -21.79% | -19.59% | -28.19% | -28.59% | 379% | 28407% |
| Class F | -16.76% | -14.58% | -23.55% | -23.06% | 367% | 9161% |

Table 7. Performance of the proposed method (VTM, AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -14.54% | -11.33% | -21.71% | -26.70% | 397% | 22332% |
| Class A2 | -14.31% | -10.44% | -26.25% | -25.63% | 363% | 19097% |
| Class B | -13.60% | -9.75% | -25.97% | -24.30% | 343% | 14694% |
| Class C | -14.20% | -11.28% | -22.45% | -23.49% | 319% | 12748% |
| Class E | -16.66% | -13.78% | -24.88% | -25.76% | 331% | 20127% |
| Overall | -14.52% | -11.14% | -24.34% | -24.98% | 347% | 16806% |
| Class D | -13.23% | -10.37% | -21.27% | -22.41% | 307% | 12140% |
| Class F | -14.65% | -12.67% | -20.57% | -20.65% | 253% | 2930% |

Conclusions

With constrained storage and low complexity, a neural network based in-loop filter is presented in this contribution. Without any further refinements, the proposed filter explored on VTM can still achieve a good trade-off on ECM.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] L.Wang, X. Xu, S. Liu. EE1-1.1: neural network based in-loop filter with constrained storage and low complexity. JVET-Y0078, 25th Meeting, by teleconference, 12–21 January 2022.

[] H. Zhu, X. Xu, and S. Liu, “Deep learning based in-loop filter with QP and residual distribution information,” Doc. AVS-M5654, Aug. 2020.

[] W. Lin, X. He, X. Han, D. Liu, J. See, J. Zou, H. Xiong, and F. Wu, “Partition-Aware Adaptive Switching Neural Networks for Post-Processing in HEVC,” IEEE Transactions on Multimedia, vol. 22, no. 11, pp. 2749–2763, Nov. 2020.

[] https://vcgit.hhi.fraunhofer.de/ecm/jvet-x-ee2/ECM/-/tags/ECM-3.0.

[] M. Karczewicz and Y. Ye. Common test conditions and evaluation procedures for enhanced compression tool testing. JVET-X2017, 24th Meeting, by teleconference, 6–15 October 2021.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-X2016, 24th Meeting, by teleconference, 6–15 October 2021.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-X2016, 24th Meeting, by teleconference, 6–15 October 2021.