| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0084-v1 |
| --- | --- |

| Title: | EE1-1.3: A Deep In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xue Zhang, Dong Jiang, Jucai Lin, Cheng Fang, Shuang Peng, | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network-based in-loop filtering method with QP based models. In this contribution, results of EE tests are proposed.

- EE 1.3.1, only Luma filter (without Chroma input).

AI {-5.66%, 0.84%, 0.92%}, RA{-2.26%, -0.20%, -0.25%}

- EE 1.3.2, only Luma filter (with Chroma input).

AI {-5.79%, 0.55%, 0.49%}, RA{-2.58%, -0.20%, -0.27%}

- EE 1.3.3, Luma filter and Chroma filter (with cross-component input).

AI {-5.80%, -16.54%, -18.19%}, RA{-2.68%, -11.82%, -11.20%}

Introduction

The proposed method is developed from the previous contribution JVET-W0059 and JVET-X0094.

In this contribution, several convolutional neural networks are trained for reducing the distortion incurred during compression. The luma and chroma components both have 5 different CNN models, corresponding to 5 QPs. The CNN model is selected according to colour components and QP.

Proposed method

Architecture

The architecture of the proposed CNN filtering method is shown in Fig.1. The network contains N residual blocks, several convolutional layers and a shortcut connection from input to output.

The input size for luma NN is 160x160, including the current CTU and 16 neighboring samples to each side of the current CTU. The chroma samples are up-sampled to 160x160, as an additional channel of the input tensor.

The input size for chroma NN is 80x80, including the current CTU and 8 neighboring samples to each side of the current CTU. The luma samples are down-sampled to 80x80, as an additional channel of the input tensor.

Other details related to the network architecture are listed as below:

- In Fig.1, N (N=64) represents the number of feature maps, kernel size is 3x3 for all convolutional layers, and only ReLU is used as the activation function.

- The number of residual blocks is M (M = 16) for luma and M_C (M_C = 10) for chroma.

- Different groups of models are trained for luma and chroma components, the two chroma components share the same network parameters.

- The CNN in-loop filter tool is applied after SAO and before ALF. An enable flag in SPS is set to control CNN in-loop filter tool, and a frame level enable flag for each component is also set to decide whether  utilize NN filtering method to it or not.

(a)

(b)

(c)

Fig. 1. (a) Architecture of the proposed luma CNN filtering method. M denotes the number of residual blocks. N stands for the number of feature maps. (b) Architecture of the proposed chroma CNN filtering method. M_C denotes the number of residual blocks. N stands for the number of feature maps. (c) Architecture of the residual block.

All the EE test are listed below, as shown in Fig.2:

- EE 1.3.1, only Luma filter (without Chroma input).

- EE 1.3.2, only Luma filter (with Chroma input).

- EE 1.3.3, Luma filter and Chroma filter (with cross-component input), proposed above.

EE1.3.1

EE1.3.2

EE1.3.3

Fig. 2. Architecture of EE tests

Training and Inference Information

In this contribution, individual models are trained with QPs in {22, 27, 32, 37, 42}. The corresponding CNN model is selected for the test sequence according to the given QP.

The NN is implemented by PyTorch for performing training process, and Libtorch for inference process in VTM11.0_NNVC.

The network information in the training stage is provided in Table 1 as suggested by [1].

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GeForce GTX 1080 Ti |
|  | Framework: | PyTorch v1.3.0 |
|  | Number of GPUs per Task | 1 GPU was used for training |
|  | Epoch: | EE 1.3.1, 10000 (intra), 3000(inter) EE 1.3.2/ EE 1.3.3, 10000 (intra), 3000(inter) |
|  | Batch size: | 32 |
|  | Training time: | EE 1.3.1, 170h/model (intra), 180h/model (inter) EE 1.3.2/EE 1.3.3, 170h/model (intra), 180h/model (inter) |
|  | Training data information: | DIV2K, BVI-4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional | Learning rate | 1e-4 |
|  | Number of iterations |  |
|  | Patch size | 160 x 160 (Luma) 80 x 80 (Chroma) |
|  | Learning rate: | 1.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | MSE |
|  | Preprocessing: | randomly cropped |

The network information in the inference stage is provided in Table 2 as suggested by [1].

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch 1.3.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | EE 1.3.1: intra/inter 1219841 /model (luma) x 10 = 12.20M 12.20M (total) EE 1.3.2: intra/inter 1220993 /model (luma) x 10 = 12.20M 12.20M (total) EE 1.3.3: intra/inter 1220993 /model (luma) x 10 = 12.20M 777281 /model (chroma) x 10 = 7.77M 19.98M (total) |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | EE 1.3.1: intra/inter 4.88 /model (luma) EE 1.3.2: intra/inter 4.88 /model (luma) EE 1.3.3: intra/inter 4.88 /model (luma) 3.11 /model (chroma) |
|  | MAC | EE 1.3.1: intra/inter 1902.6 kMAC/pixel (Luma) 0 kMAC/pixel (Chroma) 1268.4 kMAC/pixel (total) EE 1.3.2: intra/inter 1904.4 kMAC/pixel (Luma) 0 kMAC/pixel (Chroma) 1269.6 kMAC/pixel (total) EE 1.3.3: intra/inter 1904.4 kMAC/pixel (Luma) 1212.3 kMAC/pixel (Chroma) 1673.7 kMAC/pixel (total) |
| Optional | Total Conv. Layers | 35 (Luma) 23 (Chroma) |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is tested on top of VTM-11.0-NNVC[2] according to the common test conditions defined in [1]. The proposed CNN in-loop filtering method is applied after SAO and before ALF.

Test results are shown in Table 3 to Table 5.

- EE 1.3.1, only Luma filter (without Chroma input).

AI {-5.66%, 0.84%, 0.92%}, RA{-2.26%, -0.20%, -0.25%}

- EE 1.3.2, only Luma filter (with Chroma input).

AI {-5.79%, 0.55%, 0.49%}, RA{-2.58%, -0.20%, -0.27%}

- EE 1.3.3, Luma filter and Chroma filter (with cross-component input).

AI {-5.80%, -16.54%, -18.19%}, RA{-2.68%, -11.82%, -11.20%}

Compare EE1.3.1 and EE1.3.2, luma filter with chroma input will bring 0.13% gain for luma and about 0.4% in average for chroma in AI configuration, and 0.32% gain for luma in RA configuration.

Compare EE1.3.2 and EE1.3.3, without chroma filter, BD-rate drops 0.01% for luma in AI configuration, and drops 0.10% for luma in RA configuration.

Table 3. Performance of EE1.3.1 on top of VTM11.0-NNVC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.42% | 0.18% | 1.66% | 239% | 80567% |
| Class A2 | -4.86% | 0.35% | 0.13% | 183% | 63365% |
| Class B | -4.92% | 1.39% | 0.83% | 181% | 65947% |
| Class C | -6.47% | 1.92% | 2.04% | 138% | 49387% |
| Class E | -7.85% | -0.38% | -0.37% | 191% | 77358% |
| Overall | -5.66% | 0.84% | 0.92% | 180% | 65229% |
| Class D | -6.77% | 2.65% | 2.66% | 135% | 49266% |
| Class F | -3.05% | -0.14% | -0.14% | 156% | 68086% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.92% | -0.18% | -0.34% | 180% | 122739% |
| Class A2 | -2.82% | -0.25% | -0.37% | 181% | 116122% |
| Class B | -2.32% | -0.21% | -0.16% | 240% | 169421% |
| Class C | -2.04% | -0.16% | -0.19% | 203% | 151189% |
| Class E |  |  |  |  |  |
| Overall | -2.26% | -0.20% | -0.25% | 205% | 142881% |
| Class D | -2.20% | -0.01% | 0.11% | 201% | 158312% |
| Class F | -1.12% | -0.07% | -0.09% | 474% | 319447% |

Table 4. Performance of EE1.3.2 on top of VTM11.0-NNVC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.75% | -0.17% | -0.30% | 243% | 80362% |
| Class A2 | -4.94% | 0.35% | 0.14% | 186% | 63423% |
| Class B | -4.99% | 0.49% | 0.50% | 181% | 66047% |
| Class C | -6.51% | 2.03% | 1.98% | 140% | 48706% |
| Class E | -8.03% | -0.38% | -0.35% | 195% | 76843% |
| Overall | -5.79% | 0.55% | 0.49% | 183% | 64966% |
| Class D | -6.83% | 2.84% | 2.78% | 136% | 49658% |
| Class F | -3.23% | -0.15% | -0.14% | 157% | 66594% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.55% | -0.21% | -0.47% | 180% | 121859% |
| Class A2 | -3.22% | -0.31% | -0.37% | 175% | 110917% |
| Class B | -2.58% | -0.20% | -0.19% | 259% | 187176% |
| Class C | -2.12% | -0.10% | -0.14% | 230% | 178708% |
| Class E |  |  |  |  |  |
| Overall | -2.58% | -0.20% | -0.27% | 216% | 152813% |
| Class D | -2.23% | -0.03% | 0.03% | 223% | 183425% |
| Class F | -1.20% | -0.04% | -0.04% | 372% | 291051% |

Table 5. Performance of EE1.3.3 on top of VTM11.0-NNVC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.77% | -14.18% | -18.94% | 284% | 97953% |
| Class A2 | -4.93% | -16.01% | -17.99% | 212% | 75363% |
| Class B | -4.99% | -18.22% | -18.83% | 193% | 79156% |
| Class C | -6.53% | -14.69% | -17.24% | 146% | 58547% |
| Class E | -8.06% | -19.08% | -17.82% | 226% | 106932% |
| Overall | -5.80% | -16.54% | -18.19% | 202% | 79986% |
| Class D | -6.84% | -14.03% | -17.63% | 146% | 60532% |
| Class F | -3.20% | -10.38% | -5.21% | 138% | 77940% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.64% | -10.47% | -9.74% | 193% | 159056% |
| Class A2 | -3.45% | -11.29% | -12.85% | 194% | 150939% |
| Class B | -2.65% | -14.95% | -12.86% | 281% | 235256% |
| Class C | -2.19% | -9.31% | -8.98% | 278% | 321209% |
| Class E |  |  |  |  |  |
| Overall | -2.68% | -11.82% | -11.20% | 241% | 216302% |
| Class D | -2.34% | -10.29% | -11.34% | 267% | 318413% |
| Class F | -1.22% | -5.63% | -3.90% | 544% | 430170% |

Conclusions

This contribution presents a CNN-based in-loop filtering method. Compared with luma filter without chroma input, luma filter with chroma input will bring 0.13% gain for luma and about 0.4% in average for chroma in AI configuration, and 0.32% gain for luma in RA configuration.

References

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0-NNVC.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).