| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0092-v3 |
| --- | --- |

| Title: | Non-EE2: On chroma intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, two aspects of chroma intra prediction modification are proposed. For the first aspect, a new chroma intra prediction mode that uses the DIMD derivation method to derive the chroma intra prediction mode based on the collocated reconstructed luma samples is proposed. For the second aspect, it is proposed that a non-LM mode can be fused with a LM mode. The non-LM mode can be one of the proposed DIMD chroma mode, DM and four default chroma intra modes. It is reported that on top of ECM-3.1, when both two aspects are enabled, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.11%, -1.49%, -1.39%, 105%, 100%} for AI and {0.00%, -0.71%, -0.62%, 102%, 99%} for RA. When only the proposed DIMD chroma mode can be fused with the LM mode in non-I slices, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.03%%, -0.57%, -0.47%, 101%, 99%} for RA.

Introduction

The intra prediction modes enabled for the chroma components in ECM-3.1 [1] are six cross-component linear model (LM) modes including CCLM_LT, CCLM_L, CCLM_T, MMLM_LT, MMLM_L and MMLM_T modes, the direct mode (DM), and four default chroma intra prediction modes. The four default modes are given by the list {0, 50, 18, 1} and if the DM mode already belongs to that list, the mode in the list will be replaced with mode 66.

In the signaling of the chroma intra mode, a flag indicating whether LM modes are applied is signaled first. If the flag is signaled as true, it is signaled which of the six LM modes is applied. In the non-LM case, an intra_chroma_pred_mode is signaled to indicate which of the DM mode and the four default modes is applied as shown in Table 1.

Table 1 The binarization process for intra_chroma_pred_mode in ECM-3.1

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 100 | list[0] |
| 1 | 101 | list[1] |
| 2 | 110 | list[2] |
| 3 | 111 | list[3] |
| 4 | 0 | DM |

A decoder-side intra mode derivation (DIMD) method for luma intra prediction is included in ECM-3.1. First, a horizontal gradient and a vertical gradient are calculated for each reconstructed luma sample of the L-shaped template of the second neighboring row and column of the current block to build a Histogram of Gradients (HoG). Then, the two intra prediction modes with the largest and the second largest histogram amplitude values are blended with the Planar mode to generate the final predictor of the current luma block.

Proposed method

This contribution proposes methods to improve the coding efficiency of chroma intra prediction, including a decoder-side derived chroma intra prediction mode (DIMD chroma) and a fusion of a non-LM mode and a LM mode.

DIMD chroma mode

The proposed DIMD chroma mode uses the DIMD derivation method to derive the chroma intra prediction mode of the current block based on the collocated reconstructed luma samples. Specifically, a horizontal gradient and a vertical gradient are calculated for each collocated reconstructed luma sample of the current chroma block to build a HoG, as shown in Figure 1. Then the intra prediction mode with the largest histogram amplitude values is used for performing chroma intra prediction of the current chroma block.

Figure 1 the used collocated reconstructed luma samples

When the intra prediction mode derived from the DIMD chroma mode is the same as the intra prediction mode derived from the DM mode, the intra prediction mode with the second largest histogram amplitude value is used as the DIMD chroma mode.

A CU level flag is signaled to indicate whether the proposed DIMD chroma mode is applied as shown in Table 2.

Table 2 The binarization process for intra_chroma_pred_mode in the proposed method

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 1100 | list[0] |
| 1 | 1101 | list[1] |
| 2 | 1110 | list[2] |
| 3 | 1111 | list[3] |
| 4 | 10 | DIMD chroma |
| 5 | 0 | DM |

The fusion of chroma intra prediction modes

It is proposed that the DM mode, the four default modes and the proposed DIMD chroma mode can be fused with the MMLM_LT mode as follows:

pred=w0*pred0+w1*pred1≫shift

where pred0 is the predictor obtained by applying the non-LM mode, pred1 is the predictor obtained by applying the MMLM_LT mode and pred is the final predictor of the current chroma block. For I slices, the two weights, w0 and w1 are determined by the intra prediction mode of adjacent chroma blocks and shift is set equal to 2. Specifically, when the above and left adjacent blocks are both coded with LM modes, {w0, w1}={1, 3}; when the above and left adjacent blocks are both coded with non-LM modes, {w0, w1}={3, 1}; otherwise, {w0, w1}={2, 2}. For non-I slices, w0 and w1 are both set equal to 2.

For the syntax design, if a non-LM mode is selected, one flag is signaled to indicate whether the fusion is applied.

Simulation results

The proposed methods are implemented on top of ECM-3.1. The simulation results are tested under ECM test condition [2] with all intra and random access configurations as shown in Table 3.

Table 3. Simulation results of the proposed methods

|  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-3.1 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.11% |  | -1.74% |  |  | -1.27% |  | 105% |  |  | 99% |  |  |
| Class A2 | -0.07% |  | -1.43% |  |  | -1.32% |  | 106% |  |  | 99% |  |  |
| Class B | -0.10% |  | -1.29% |  |  | -1.40% |  | 104% |  |  | 101% |  |  |
| Class C | -0.11% |  | -1.35% |  |  | -1.28% |  | 106% |  |  | 100% |  |  |
| Class E | -0.14% |  | -1.82% |  |  | -1.70% |  | 104% |  |  | 101% |  |  |
| Overall | -0.11% |  | -1.49% |  |  | -1.39% |  | 105% |  |  | 100% |  |  |
| Class D | -0.10% |  | -0.87% |  |  | -1.05% |  | 107% |  |  | 99% |  |  |
| Class F | -0.11% |  | -0.89% |  |  | -0.81% |  | 104% |  |  | 100% |  |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over ECM-3.1 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.02% |  | -0.61% |  |  | -0.45% |  | 102% |  |  | 99% |  |  |
| Class A2 | -0.04% |  | -0.61% |  |  | -0.49% |  | 103% |  |  | 99% |  |  |
| Class B | 0.02% |  | -0.79% |  |  | -0.88% |  | 102% |  |  | 100% |  |  |
| Class C | 0.03% |  | -0.76% |  |  | -0.51% |  | 102% |  |  | 99% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | -0.71% |  |  | -0.62% |  | 102% |  |  | 99% |  |  |
| Class D | 0.07% |  | -0.61% |  |  | -0.98% |  | 102% |  |  | 99% |  |  |
| Class F | -0.04% |  | -0.35% |  |  | -0.43% |  | 102% |  |  | 99% |  |  |

In order to reduce the slight loss of the proposed methods for luma component in random access configuration, it is proposed that for non-I slices only the proposed DIMD chroma mode can be fused with the MMLM_LT mode. The simulation results are shown in Table 4.

Table 4. Simulation results of the proposed methods in the variant for RA

|  |  |  |  |  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Over ECM-3.1 |  |  |  |  |  |  |
|  | Y |  | U |  |  | V | EncT |  |  | DecT |  |
| Class A1 | -0.05% |  | -0.43% |  |  | -0.51% | 101% |  |  | 99% |  |
| Class A2 | -0.05% |  | -0.58% |  |  | -0.40% | 101% |  |  | 99% |  |
| Class B | -0.02% |  | -0.65% |  |  | -0.72% | 100% |  |  | 99% |  |
| Class C | -0.03% |  | -0.56% |  |  | -0.17% | 101% |  |  | 99% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.03% |  | -0.57% |  |  | -0.47% | 101% |  |  | 99% |  |
| Class D | -0.02% |  | -0.23% |  |  | -0.74% | 100% |  |  | 99% |  |
| Class F | -0.06% |  | -0.24% |  |  | -0.26% | 100% |  |  | 98% |  |

Conclusion

This contribution proposes two methods to improve the coding efficiency of chroma intra prediction which achieve over 1% chroma gain in AI configuration. It is suggested to further study in EE.

References

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).