| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0093 |
| --- | --- |

| Title: | EE2-3.3: On TMVP improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Jie Chen, Yan Ye, Xinwei Li | Tel: Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the results of TMVP merge candidate modifications, which are studied in EE2 Test 3.3. In Test 3.3a, when constructing the merge candidate list of non-subblock merge modes, the TMVP is conditionally added to the list according to the motion information of neighboring spatial blocks. And, the reference picture of the TMVP merge candidate is selected from any one of reference pictures in the reference picture list instead of being fixed to the picture with index 0. The selected reference picture is the one whose scaling factor is the closet to 1. In Test 3.3b, on top of Test 3.3a, after the reference picture of TMVP is selected, the template matching method is used to decide the scaling factor and inter prediction direction of the TMVP. And, the template matching method is only applied to non-low delay pictures. It is reported that on top of ECM-3.1, the overall coding performance impact for {Y, U, V, EncT, DecT} is:

Test 3.3a: RA {-0.02%, -0.03%, -0.02%, 99%, 100%}, LDB{-0.34%, -0.43%, -0.45%, 99%, 102%}

Test 3.3b: RA {-0.13%, -0.17%, -0.15%, 99%, 105%}, LDB{-0.34%, -0.43%, -0.45%, 99%, 102%}

Introduction

This contribution presents the simulation results of Test 3.3 [1] which studies the modifications to TMVP merge candidate.

Test 3.3a

In the merge candidate list construction of a current CU coded using MMVD, CIIP, GPM, template matching, AMVP-merge and regular merge modes, the TMVP candidate is conditionally added to the list according to the motion information of spatial neighboring blocks, which are {A1, B1, B2} depicted in Figure 1. The conditions are illustrated in Figure 2, where cond_1, cond_2 and cond_3 are listed as follows:

- cond_1: position C0 is available, is inter-coded, and is in the same CTU row of the current CU.

- cond_2: none of the spatial neighboring blocks has similar temporal motion with temporal motion C0 (or C1); Or, at least one of the spatial neighboring blocks has similar temporal motion with temporal motion C0 (or C1) and the spatial neighboring blocks use TMVP.

- cond_3: position C1 is inter-coded.

In checking the similarity of the temporal motion, if the inter prediction direction of the two temporal motion are the same and the motion vector differences in horizontal and vertical direction are both smaller than 5, the two temporal motion are treated as similar to each other.

Figure 1. Position of spatial and temporal candidates

Figure 2. Proposed conditions of adding TMVP candidate

In addition, when deriving TMVP candidate for MMVD, CIIP, GPM, template matching, AMVP-merge, regular merge, SbTMVP and affine modes, the target reference picture of the TMVP candidate can be selected from any one of reference picture in the list according to scaling factor. The scaling factor of each reference picture in the reference picture list is calculated, and the one whose scaling factor is the closest to 1 is used as the target reference picture of the TMVP.

Test 3.3b

Test 3.3b is implemented on top of Test 3.3a. When a CU is coded using template matching, AMVP-merge and regular merge modes and the current picture is a non-low delay picture, the TMVP candidate is refined using the template matching method. The template is used to decide whether the TMVP is L0-predicted, L1-predicted or bi-predicted, and is used to decide which one of the three scaling factors {tbtd, tbtd×18, tbtd×98} is used to perform temporal motion vector scaling. The tb is defined to be the POC difference between the reference picture of the current picture and the current picture and td is defined to be the POC difference between the reference picture of the co-located picture and the co-located picture, as shown in Figure 3.

Figure 3. Scaling of TMVP

Simulation results

The described methods are implemented on top of ECM-3.1 software and are tested under CTC for enhanced compression tool testing [2]. Table 1 and Table 2 show the simulation results of Test 3.3a and Test 3.3b, respectively.

Table 1. Simulation results of Test 3.3a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | -0.13% | 100% | 100% |
| Class A2 | -0.01% | 0.06% | -0.02% | 100% | 100% |
| Class B | -0.02% | -0.10% | -0.02% | 99% | 101% |
| Class C | -0.05% | 0.09% | 0.05% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.02% | 99% | 100% |
| Class D | -0.05% | 0.08% | 0.04% | 99% | 100% |
| Class F | 0.00% | -0.01% | -0.05% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.48% | -0.38% | 99% | 102% |
| Class C | -0.72% | -0.79% | -0.78% | 98% | 102% |
| Class E | 0.05% | 0.15% | -0.11% | 98% | 101% |
| Overall | -0.34% | -0.43% | -0.45% | 99% | 102% |
| Class D | -0.80% | -1.16% | -1.19% | 99% | 103% |
| Class F | -0.46% | -0.10% | -0.29% | 98% | 100% |

Table 2. Simulation results of Test 3.3b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.28% | -0.31% | 99% | 103% |
| Class A2 | -0.11% | -0.12% | -0.10% | 100% | 105% |
| Class B | -0.12% | -0.15% | -0.08% | 99% | 104% |
| Class C | -0.18% | -0.15% | -0.16% | 99% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.17% | -0.15% | 99% | 105% |
| Class D | -0.21% | -0.19% | -0.08% | 99% | 108% |
| Class F | -0.09% | -0.12% | -0.11% | 98% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.48% | -0.38% | 99% | 102% |
| Class C | -0.72% | -0.79% | -0.78% | 98% | 102% |
| Class E | 0.05% | 0.15% | -0.11% | 98% | 101% |
| Overall | -0.34% | -0.43% | -0.45% | 99% | 102% |
| Class D | -0.80% | -1.16% | -1.19% | 99% | 103% |
| Class F | -0.46% | -0.10% | -0.29% | 98% | 100% |

Conclusion

This contribution reports the results of EE2 Test 3.3 which improve the coding efficiency of TMVP merge candidate. With the proposed methods, 0.13% gain and 0.34% gain can be achieved in RA and LDB configurations, respectively. It is recommended to adopted Test 3.3b into the next version of ECM software.

References

- V. Seregin and et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-X2024, October 2021.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-X2017, October 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).