| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0097-r2 |
| --- | --- |

| Title: | AhG12: Removed DIMD from MPM list of TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Thierry Dumas Ya Chen Franck Galpin | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In ECM software, DIMD is required to performed prior to TIMD in the decoding process in order to fill in the TIMD-MPM list. It is argued that this process is associated with high complexity at the decoder side. This contribution proposes removing DIMD modes from the TIMD MPM list while keeping them in the default MPM list.

On top of ECM-3.1, the following results are obtained:

AI: 0,01%	-0,03%	0,03% EncT 100% DecT 97%

RA:  0,01%	0,00%	0,07%	EncT 100% DecT 100%

LDB: 0,01%	-0,03%	-0,08%	EncT 100%	DecT 100%

# Introduction

Compared to intra tools, DIMD and TIMD are associated with highest decoder complexity in ECM software. This is because the decoder is required to analyze the template pixels (in DIMD mode) or test several modes and deduce the best one (in TIMD mode). This leads to about 10% increase in decoding time in AI for each mode as reported in JVET-V0024 and JVET-W0123.

In the current ECM implementation, the decoder is required to perform DIMD when TIMD is used. This is because TIMD tests all MPM modes that include the DIMD derived modes. This is true for both intra coded CU with TIMD, and inter coded CU with CIIP, where the latter employs TIMD for its intra prediction part. In the worst-case scenario, both DIMD and TIMD are performed for each CU in the decoding process, which leads to high decoding complexity.

# Proposal

It is proposed to remove the DIMD modes from MPM list for TIMD mode, while keeping them in the default MPM. DIMD is no longer required to be performed prior to TIMD in the decoding process.

# Results

The following results are obtained on top of ECM-3.1 following the common testing conditions:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,05% | 0,15% | 98% | 96% |
| Class A2 | -0,01% | 0,05% | 0,08% | 97% | 90% |
| Class B | 0,00% | -0,07% | -0,04% | 97% | 93% |
| Class C | 0,01% | -0,05% | 0,00% | 103% | 102% |
| Class E | 0,01% | -0,01% | -0,01% | 106% | 109% |
| Overall | 0,01% | -0,03% | 0,03% | 100% | 97% |
| Class D | 0,03% | -0,12% | -0,11% | 99% | 97% |
| Class F | 0,10% | 0,00% | 0,04% | 106% | 103% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM 3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,10% | -0,14% | 98% | 98% |
| Class A2 | -0,01% | -0,02% | 0,05% | 101% | 100% |
| Class B | 0,02% | 0,04% | 0,05% | 101% | 101% |
| Class C | 0,03% | 0,03% | 0,26% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | 0,00% | 0,07% | 100% | 100% |
| Class D | -0,02% | 0,11% | -0,13% | 101% | 99% |
| Class F | 0,05% | -0,06% | -0,11% | 100% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM 3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,05% | -0,30% | -0,12% | 100% | 99% |
| Class C | -0,03% | 0,00% | 0,25% | 99% | 98% |
| Class E | 0,00% | 0,38% | -0,47% | 102% | 102% |
| Overall | 0,01% | -0,03% | -0,08% | 100% | 100% |
| Class D | 0,16% | 0,21% | -0,34% | 99% | 98% |
| Class F | -0,17% | -0,99% | -1,07% | 98% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution proposes to remove DIMD modes from MPM list for deriving TIMD mode. It is asserted that this reduces the decoder complexity, where DIMD and TIMD need not to processed successively for a given block. It is recommended to adopt this method to ECM software.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).