| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0104-v1 |
| --- | --- |

| Title: | AHG9: Transparency information SEI for transparent screens |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Pierre Andrivon Fabrice Le Leannec Mary-Luc Champel | Email: | thomase@xiaomi.com pandrivon@xiaomi.com fabricel@xiaomi.com champelmaryluc@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

It is asserted that a conventional video content displayed on a transparent display may result in a degraded experience for the viewer where important parts of the image may be lost such as logos, computer windows or characters, etc. Additionally, it is further asserted that the transparency effect perceived by the viewer of those transparent displays is controlled by the sample values of the displayed images and depends on the panel technology and the ambient lightning conditions.

Based on those observations, it is proposed to define metadata payload to signal the content creator intent with respect to how the decoded pictures should look like on a transparent display, i.e., which area should be transparent and to what degree.

To this end, a SEI signaling is presented which is asserted to address the above requirement and enable the transformation of the decoded pictures after decoding for both legacy and new content video content.

# Introduction

Over the past 20 years, transparent displays significantly progressed thanks to more efficient display technologies, first based on LCD and then OLED. This allowed to explore different markets such as head-mounted display, for military and automotive sectors, or digital signage to promote products in shop to customers.

Lately, the technology, especially based on OLED-type of screen, became mature enough to be produced at large scale and for both professional as well as private end users. Examples of such models are the Xiaomi Mi TV Lux, the Panasonic or the LG presented in this contribution.

The key principle of transparent display is that the light behind the display is able to traverse the display and reach the viewer. This way, the user can see the objects behind the panel and experience the transparency effect where both those objects and the displayed images on the panel blend together. In short, the perceived transparent effect predominantly depends on the sample values in the displayed image but is also influenced by the ambient lightning where the display seats. This has three consequences.

First of all, a conventional content displayed on a transparent display may result in a degraded experience for the user where important parts of the image are being lost because being coded with transparent sample values. For instance, a logo, a computer window or a person in a conventional video content can possibly be lost when displayed on a transparent display because they would be represented with the transparent color or close from it.

Secondly, the function controlling the transparency effect based on the sample values depend on the technology of the panel which means that a pre-adjustment for all the devices is not possible.

Thirdly, the ambient lightning is by definition unknown when encoding the content and cannot be factored in the content preparation stage.

Based on those three observations, it appears desirable to design a solution for preserving the content creator intent with respect to transparency display in a video sequence, for enabling legacy content to be properly displayed on transparent displays and for allowing a per-device adjustment of the transparency effect when the device is equipped with an ambient lighting sensor.

In light of those guiding principles, this contribution proposes to define metadata in the video bitstream which will allow the transformation of the decoded pictures to meet the content creator intent when displayed on transparent displays. It is proposed to define a new SEI message that will carry the necessary information for a post-decoding process to adjust the samples values of the decoded pictures in order to meet the transparency effect in the displayed image as desired by the content creator and possibly based on the current ambient lightning conditions.

# Background

## Some commercial panels

Even though transparent displays reached a high quality only recently, there already exist several models on the market. This list is not meant to be exhaustive.

Table 1 - Example models of commercialized transparent displays

| Models | Picture |
| --- | --- |
| Xiaomi Mi TV LUX [3] |  |
| Panasonic TP-55ZT100 [4] |  |
| LG 55EW5G-V [5] |  |

## Overview of transparency control function for LCD and OLED displays

For both LCD and OLED-based transparent screens, the fact that the viewer is able to perceive the scene or parts of the scene behind the TV is determined by the value of the pixels. There are two extreme cases. The first one is when the user doesn’t see at all what is behind the screen, i.e., total opacity. The second one is when the user fully sees what is behind the TV and no other image displayed on the TV, total transparency. In between those cases, there is a continuous spectrum of transparency level wherein the user sees the displayed image on the screen overlayed on top of the scene behind the TV. All those cases, full opacity, full transparency and intermediate transparency can be for the entire screen or localized up to a granularity of a pixel for the best see-through display technology such as current OLED displays.

Depending on LCD or OLED technologies, each of the effects described above, i.e., full opacity, full transparency and intermediate transparency, is realized in a different manner. For LCDs, a pixel is black when as much light as possible coming from the light source is stopped. As a result, a dark pixel will not let the light from behind the TV traverse and will appear opaque for the viewer. Conversely, a pixel is white when all the light comes through all the sub color pixels to form a white light beam. As a result, the light from behind may also traverse the pixel through the holes and thus the objects behind may be visible to the viewer, hence creating the transparency effect.

Table 2 - Full transparency and opacity values for LCD and OLED displays

| Effect | Transparent LCD | Transparent OLED |
| --- | --- | --- |
| Opaque pixel | Black pixel | White pixel |
| Transparent pixel | White pixel | Black pixel |

This effect is illustrated below in Figure 2 for transparent OLED screens.

Figure 2 - Comparison between a transparent OLED and a traditional LCD display (source: [1])

On the right side is a transparent OLED screen and on the right is a conventional LCD screen. Both screens display a grayscale gradient from left (white) to right (black). On a transparent OLED, the black pixels (light turned off) let the light coming from behind passing through the screen. This effectively means that in this simple version of a transparent OLED screen, it is not possible to display black on the screen.

Conversely, it is not possible to display white pixels on transparent LCD screens provided the scene on the back of the TV is not white. As shown on Figure 3, the objects behind the TV are barely visible in the black area of the display but appear clearly visible in the white area of the screen.

Figure 3 - Demo of transparent LCD screen (source: [2])

# Definition of the transparency information

## Signaling

The amount of data may be large, especially for high resolution videos. However, we believe that requiring an advanced coding scheme (entropy-based or even up to a full video encoding) would outweigh the benefits of a lightweight design, that is, using the CPU for decoding the transparency mask and the dedicated hardware for the video sequence. In addition, a requiring legacy content to be transcoded to a new format would also hinder the possibility of offering quickly and widely such experience to the users.

Therefore, we propose three measures to reduce the burden on the bitrate:

- Allowing the persistence of the signaled mask for several consecutive coded pictures

- Allowing the signaled mask to have a smaller resolution than the coded video sequence

- Allowing a run-length encoding in the SEI for encoding the mask values

Table 4 - Signalling of the transparency mask as SEI with optional RLE compression

| transparency_mask( payloadSize ) { | Descriptor |
| --- | --- |
| mask_size_present_flag | u(1) |
| if(mask_size_present_flag) { |  |
| mask_width | ue(v) |
| mask_height | ue(v) |
| } |  |
| rle_scheme_used_flag | u(1) |
| if(rle_scheme_used_flag) { |  |
| number_of_runs | ue(v) |
| for(i=0; i < number_of_runs; i++) { |  |
| run_value[ i ] | u(8) |
| run_length[ i ] | ue(v) |
| } |  |
| } else { |  |
| for(i=0; i < MaskHeight; i++) |  |
| for(j=0; j < MaskWidth; j++) |  |
| mask_value[ i ][ j ] | u(8) |
| } |  |
| } |  |

mask_size_present_flag equal to 1 specifies that the payload contains explicit width and height of the mask. When equal to 0, the width and the height of the mask is inferred.

mask_width specifies the width of the transparency mask.

mask_height specifies the height of the transparency mask.

The variables MaskHeight and MaskWidth are determined as follows:

if(mask_size_present_flag) {
		MaskWidth = mask_width
		MaskHeight = mask_height
	} else {
		MaskWidth = pps_pic_width_in_luma_samples
		MaskHeight = pps_pic_height_in_luma_samples
	}

rle_scheme_used_flag equal to 1 specifies that the mask values are run-length encoded. When equal to 0, the mask values are signaled explicitly.

number_of_runs specifies the number of runs.

run_value[ i ] specifies the value of the i-th run.

run_length[ i ] specifies the length of the i-th run.

mask_value[ i ][ j ] specifies the transparency of the collocated luma sample denoted by the variable LumaTransparency[ i ][ j ]. When mask_value[ i ][ j ] is equal to 0, the collocated luma sample is transparent. When mask_value[ i ][ j ] is equal to 255, the collocated luma sample is opaque. When MaskWidth or MaskHeight are not equal to respectively pps_pic_width_in_luma_samples or pps_pic_height_in_luma_samples, the collocated luma sample is determined after resampling of the transparency mask. The transparency attribute of a luma sample is transferred to its associated samples (e.g., chroma samples) and then to the collocated converted samples (e.g., RGB samples) for display. When mask_value[ i ][ j ] is not present (i.e., RLE scheme used), the value of LumaTransparency[ i ][ j ] is inferred as follows:

if(rle_scheme_used_flag) {
	int i = 0
	int j = 0
	for(k=0; k < number_of_runs; k++) {
		for(l=0; l < run_length[ k ]; l++) {
			LumaTransparency [ i ][ j ] = run_value[ k ]
			if(i < MaskWidth) {
				i++
			} else {
				i = 0
				j++
			}
		}
	}
} else {
	for(i=0; i < MaskHeight; i++)
		for(j=0; j < MaskWidth; j++)
			LumaTransparency [ i ][ j ] = mask_value[ i ][ j ]
}

Given a mask of the same resolution as the decoded picture in luma samples, a post-decoding process can adjust these decoded pictures by implementing the following pixel-based operation.

For c = 0,

Iadjusted[ c ][ x ][ y ] = Clip3(
						0,
						( 1 << BitDepth ) − 1,
						Idecoded[ c ][ x ][ y ] * LumaTransparency[ x ][ y ] ÷ ( (1 << 8) − 1  ) +
						TransparentValue[ c ] * ( 1 − ( LumaTransparency[ x ][ y ] ÷ ( (1 << 8 ) − 1 ) ) )

And for c=1..2,

Iadjusted[ c ][ x ][ y ] = Clip3(
						0,
						( 1 << BitDepth ) − 1,
						Idecoded[ c ][ x ][ y ] * LumaTransparency[ x * SubWidthC ][ y * SubHeightC ] ÷ ( ( 1 << 8 ) − 1 ) ) ) +
						TransparentValue[ c ] * ( 1 − ( LumaTransparency[ x * SubWidthC ][ y * SubHeightC ] ÷ ( ( 1 << 8) − 1 ) ) )

where Idecoded[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded image Idecoded, Iadjusted [ c ][ x ][ y ] is the transparency-adjusted image at the same position and colour component, where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples – 1 for c = 0 and x = 0.. PicWidthInSamplesC − 1, and y = 0.. PicHeightInSamplesC – 1 for c = 1..2.

# Simulation of transparency mask signaling

## Context

To estimate the compression efficiency of the proposed scheme, a Python script was developped to calculate the size in bits of the two syntax elements:

| run_value[ i ] | u(8) |
| --- | --- |
| run_length[ i ] | ue(v) |

The first step was to manually create a mask based on the first frame of the sequence. Then, this mask was fed to the script and losslessly compressed.

The simulation was performed for several sequences as presented below. The summarising table of the results is given at the end of the clause 4.

## Johnny

The sequence “Johnny” is representative of a video sequence with a talking person. In real-world applications, this could correspond to a video call or a TV news program.

A relevant mask for those types of videos is a mask covering the person.

The mask is 8-bit monochrome picture of the same resolution as the video sequence, i.e., 1280 by 720 luma samples.

## Desktop

The “Desktop” video sequence corresponds to use cases where the image of a desktop can be transmitted and displayed on a transparent screen.

In this case, it is important that the buttons and the interface the user interacts with are always visible. Conversely, some parts of the desktop like the background may be partially or fully transparent. In addition, the active window may be fully opaque, but the other windows may be partially transparent. This scenario was chosen for the simulation and the corresponding mask is given.

When applying this mask, the resulting pictures emphasizes on the active window while the other windows are still partially visible.

## BasketballDrive

The “BasketballDrive” is representative of any sport event that would be displayed on a transparent screen.

In this scenario, one can imagine that the players could be made standing out of the stadium for dramatic effect by making the background disappear. Such a mask was created for the purpose of the simulation.

Given the mask above, the resulting image is given below.

## Simulation results

The results the simulated masks are given below.

| Sequence | Resolution | Frame rate | Mask resolution | Mask uncompressed size (bits) | Mask compressed size (bits) | Ratio | Simulated mask bitrate (kbits/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Johnny | 1280x720 | 60 | 1280x720 | 7 372 800 | 287 177 | 96,1% | 17 231 |
| Johnny | 1280x720 | 60 | 640x360 | 1 843 200 | 92 310 | 95,0% | 5 599 |
| Desktop | 1920x1080 | 60 | 1920x1080 | 16 588 800 | 55 597 | 99,7% | 3 336 |
| Desktop | 1920x1080 | 60 | 960x540 | 4 147 200 | 47 743 | 98,8% | 2 865 |
| BasketballDrive | 1920x1080 | 50 | 1920x1080 | 16 588 800 | 301 124 | 98% | 15 056 |
| BasketballDrive | 1920x1080 | 50 | 960x540 | 4 147 200 | 106 807 | 97,4% | 5 340 |

The simulation shows that encoding the mask at native resolution and frame rate can be a bit expensive for some sequences like Johny and BasketballDrive. However, spatially downsampling the mask by just a factor two gives reasonable bitrates compared to the expected video bitrate for such sequences and resolutions. In addition, the mask may also be signaled every two frames which would effectively half the simulated bitrate.

It appears then reasonable to envision such transport of the transparency information.

# Conclusion

Transparent displays are becoming mature technologies. However, conventional video content displayed on a transparent display can result in a degraded experience for the viewer where important parts of the image may be lost such as logos, computer windows or characters, etc. Additionally, the transparency effect perceived by the viewer of those transparent displays is controlled by the sample values of the displayed images and depends on the panel technology and the ambient lightning condition.

We thus recommend the adoption of the proposed SEI message to enable the transformation of the decoded pictures after decoding for both legacy and new content video content. This design is a codec-agnostic, requires one video decoder, doesn’t require legacy content to be transcoded and allows on-device adjustment per technology and also when the ambient lightning measurement is available.

# References

- Planar® LookThru™ Transparent OLED Display Content Developer’s Guide, https://www.planar.com/media/438990/020-1316-00-rev-3_planar-lookthru-content-developers-guide-03-2020.pdf

- HUGELY Impressive 48 inch Transparent Showcase in Demo, Crystal Display Systems, https://www.youtube.com/watch?v=rNwk_rxg7Gw

- Turning moment for TV industry: Xiaomi Unveils Mi TV LUX Transparent Edition  – Mi Blog

- Panasonic Commercializes Transparent OLED Display Module with Superb Image Visibility | Headquarters News | Panasonic Newsroom Global

- LG 55EW5G-V: Transparent OLED Signage | LG Global Business

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).