| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0111-v2 |
| --- | --- |

| Title: | AhG11: Hybrid Conventional/Deep-learning-based image coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Franck Galpin Thierry Dumas Philippe Bordes Fabien Racapé Edouard François (InterDigital) Yue Li, Kai Zhang, Li Zhang (Bytedance) Hongtao Wang Kevin Reuze Anand Meher Kotra Marta Karczewicz (Qualcomm) | Email: | franck.galpin@interdigital.com thierry.dumas@interdigital.com philippe.bordes@interdigital.com fabien.racape@interdigital.com edouard.francois@interdigital.com yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com hongtaow@qti.qualcomm.com |
| Source: | InterDigital, Bytedance, Qualcomm |  |  |

_____________________________

# Abstract

This contribution demonstrates the compression performance of a hybrid conventional/deep learning-based image coding approach, compared to fully conventional and end-to-end trained methods. Neural Network based Intra Prediction (as described in JVET-X0118) and Neural Network based Post-filtering (as described in JVET-X0066) are implemented on top of ECM-3.1. The compression performance is evaluated on the Kodak image dataset and compared to other state-of-the-art approaches. Average PSNR BD-rate variations of -11.7%, -27.5 and -26.6% on Y, U and V components, respectively, are reported on the Kodak dataset compared to the VTM-15.0 anchor.

# Introduction

Neural Network (NN)-based end-to-end image compression showed great improvement in recent years. The CLIC image compression challenge also shows that the best methods are now on par with traditional approaches. In JVET AhG11, hybrid methods, where Neural Networks and traditional tools are used together, are also studied.

This contribution reports the results of an implementation of two NN-based tools on top of ECM-3.1:

- Intra prediction (NNIP) described in JVET-X0118 and implemented in EE1 3.1.2

- Post-filtering described in JVET-X0066 and implemented in 1.2.2

# Evaluation

The Kodak dataset [1] is commonly used for evaluating end-to-end NN-based compression methods. Currently, end-to-end approaches and traditional codec do not use the same metrics, datasets, test conditions and platform for evaluation, which makes it difficult to directly compare the two approaches.

## State of the art end-to-end approaches

Figure 1: RGB PSNR vs. bit-rate (bpp) on the Kodak dataset

Figure 1 shows the RD-curves from [2] on the Kodak dataset. It should be noted that:

- The models have been trained using RGB MSE as criterion and evaluated on RGB PSNR,

- VTM-11.2 and HM-16.22 (RExt) are configured to encode a 444 YUV input. RGB PSNR is computed after converting back to RGB. Moreover, both codecs are optimized for YUV metrics.

As a reference, using the 4 bitrates in the range [0.2-1.6] bpp, and using HM as an anchor, a BD-rate gain of 17.8% for Cheng2020 [3] and 18.8% for VTM are reported.

## Results using a hybrid conventional/NN-based image codec

Figure 2: YUV(4:2:0) PSNR vs. bitrate on the Kodak dataset

Using the same dataset, Figure 2 compares the performances of HM, VTM and the hybrid codec described in Section 1. In this case, a YUV420 PSNR is used (weighted as 411) and images were encoded and compared to a YUV420 version of the dataset.

Because of the metric and format changes, PSNRs are not directly comparable. However, using HM as an anchor, similar gains are reported, compared to VTM’s gains in RGB PSNR in Figure 1. Considering the 4 bitrates in the range [0.2-1.3], VTM shows -20.2% of BD-rate variation over HM (compared to 18.8% in RGB PSNR using YUV 444 inputs).

The hybrid codec gives a -31.3% BD-rate variation compared to HM and -13.9% compared to VTM.

Playing with the flexible nature of traditional codecs, other simulations were run, changing the configuration of ECM, typically using more partitioning choices. While slowing down the encodings, it improved the results, providing up to -32.5% BD-rate variation, compared to HM and -15.3% compared to VTM.

# Conclusion

While the improvement of end-to-end codecs for image compression has been very fast recently, this contribution demonstrates that hybrid approaches combined with NN-based modules also significantly improve the raw gains of image compression, while keeping the advantage of the flexible nature of traditional codecs.

The table below sums up the BD-rate variations obtained on the Kodak dataset, using HM as an anchor.

| codec | RGB PSNR | YUV PSNR |
| --- | --- | --- |
| VTM | -18.8% | -20.2% |
| Cheng 2020 | -17.8% |  |
| ECM 3.1 |  | -25.5% |
| Hybrid |  | -31.3% |
| Hybrid slow |  | -32.5% |

# References

Kodak image dataset: http://r0k.us/graphics/kodak/

J. Bégaint, F. Racapé, S. Feltman, A. Pushparaja, “CompressAI: A PyTorch library and evaluation platform for end-to-end compression research,” m54467, July 2020.

Z. Cheng, H. Sun, M. Takeuchi and J. Katto, "Learned Image Compression With Discretized Gaussian Mixture Likelihoods and Attention Modules," 2020 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), 2020, pp. 7936-7945, doi: 10.1109/CVPR42600.2020.00796.

# Appendix

- RGB to YUV420 conversion has been done using the following command-line:

ffmpeg -i kodimxx.png -pix_fmt yuv420p kodimxx.yuv

- Source code of the merged version is attached. Models and SADL are not attached and can be found in each branch of the EE1 repository.

- Configuration files for the normal and slow versions are also attached.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).